CEPT Report <No>

Report from CEPT to the European Commission in response to the Mandate on shared use of 3800-4200 MHz by terrestrial wireless broadband systems providing local-area network connectivity

Report approved on DD Month YYYY by the ECC(Arial 9pt bold)

This is a working document. The modifications inserted are based on inputs for ECC PT1 #72-74 and there was no time to review them. All texts will be further reviewed and discussed in upcoming correspondence group meetings until the ECC PT1 #75.

# Executive summary

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	6

2	Existing and planned use of 3.8-4.2 GHz	7

2.1	Fixed Satellite Service	7

2.2	Fixed Service	8

3	Terrestrial wireless broadband systems providing local area (i.e. low/medium power) network connectivity in 3.8-4.2 GHz	9

3.1	Background and description	9

3.2	Use cases	9

3.3	Parameters to be used for studies	9

3.3.1	Coexistence scenarios	10

3.3.2	Propagation parameters	10

4	Sharing Studies with in-band services	11

4.1	WBB LMP	11

4.2	Fixed Service	11

4.3	Fixed Satellite Service (Space-to-Earth)	11

5	Compatibility studies with Adjacent band Services	12

5.1	Mobile Service below 3.8 GHz	12

5.1.1	5G commercial	12

5.2	Fixed Satellite service (space-to-Earth) below 3.8 GHz	12

5.3	Aeronautical Mobile (R) service above 4.2 GHz (WAIC)	12

5.4	Aeronautical radionavigation service in 4.2-4.4 GHz in 4.2-4.4 GHz (Radio altimeters)	12

6	technical Feasibility	13

7	Harmonised technical conditions	14

8	Standardisation needs	15

9	Conclusions	16

ANNEX 1: cept mandate	17

1.	Purpose	18

2.	Policy context and inputs	19

3.	Justification	20

4.	Task order and schedule	20

ANNEX 2: Proposal for EC Decision	22

ANNEX 3: WBB LMP parameters for studies on 3800-4200 MHz	23

ANNEX 4: Other parameters and assumptions for studies on 3800-4200 MHz	24

ANNEX 5: Potential deployment scenarios for local area networks in the 3.8-4.2 GHz band (ECC PT1(22)100_Nokia)	25

ANNEX 6: Sharing and Compatibility studies	32

ANNEX 7: Coexistence study between WBB LMPapplications (low/medium power) and FSS receiving earth stations (ECC PT1(23)047_France)	33

ANNEX 8: Coexistence study between local 5G-vertical applications (for the scenarios of ECC PT1(22)100) and FSS receiving earth stations (ECC PT1(22)212_France)	41

ANNEX 9: Adjacent band co-existence study between MFCN in 3400-3800 MHz and LAN in 3800-4200 MHz (ECC PT1(22)102_Orange)	46

ANNEX 10: Sharing study between FSS ES and LANWBB LMP in the band 3800-4200 MHz (CG4G(22)017_Intelsat)	65

ANNEX 10: Sharing study between FSS SATPAQ and WBB LMP in the band 3800-4200 MHz (CG4G(22)017_Intelsat)	94

ANNEX 11: Coexistence studies between WBS in 3.8-4.2 GHz and 5G Commercial systems in 3.4-3.8 GHz (ECC PT1(22)206_Nokia)	109

ANNEX 12: Sharing study between WBS base stations and FSS earth stations in the band 3.8-4.2 GHz (ECC PT1(23)049_Ericsson)	116

ANNEX 13: Coexistence study of WBB LMP with MFCN and FSS ES ECC PT1(23)039_Nokia)	125

List of References	134

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| ECC | Electronic Communications Committee |

# Introduction

drafting managed by FM60

# Existing and planned use of 3.8-4.2 GHz

drafting managed by FM60

## Fixed Satellite Service

The 3400-4200 MHz band has been used for decades by the FSS for space-to-Earth links (downlink), together with the 5850-6725 MHz frequency band for Earth-to-space links (uplink). In CEPT countries, the 3600-3800 MHz and 3800-4200 MHz have been usually used more extensively by FSS earth stations than the lower part 3400-3600 MHz.

Due to the introduction of 5G in 3.4-3.8 GHz, as primary band in Europe, CEPT recommended in areas intended for 5G, such as urban, suburban areas, or along transport routes such as roads and railways that administrations consider relocation of earth stations operating in 3400-3800 MHz to a different geographical location or to a different band above 3800 MHz (See ECC report 287). In addition, in the areas where 5G is intended to be used, CEPT recommended administrations not to issue authorisations to new sites in this band for FSS and to consider the higher bands above 3800 MHz for future FSS usage.

In consequence, administrations maintained a limited number of FSS earths station in 3600-3800 MHz and many stations have migrated from the band below 3800 MHz to the 3800-4200 MHz frequency band.

C-band is an essential frequency band for FSS because of its unique characteristics such as a wide geographic coverage over continents and a resistance to rain fade. The band is therefore critical for services provided to inter-tropical regions, and as part of their global reach, many earth stations are located in European geographic area for inter-continental communications. Applications of C-band services include connectivity for enterprises and public institutions (remote locations or large continental networks), mobile backhauling, video contribution and distribution.

For example:

Various international broadcasters use C band FSS for international distribution of content from Europe to the rest of the world. Although this content is uplinked in a different band, the downlink is in the 3.8-4.2 GHz band. Successful operation of this system relies on interference-free reception of the downlinked signal for continuous power control (at an uplink site) and monitoring use. Large dish earth stations also need to receive beacon signals transmitted from the satellite for tracking purposes.

Many video contribution links are also received in Europe in the C-band from other regions before being distributed from Europe.

Existing FSS earth stations in 3800-4200 MHz are limited in number and locations are well identified. New C-band earth stations sites could be established in European geographic area in the future in locations that would be well identified. In addition, CEPT is currently studying the possibility to exempt from individual licensing small C-band IoT terminals in other frequency bands.  The development of such application could lead to the need for more gateway earth stations in the band 3.8-4.2GHz.

Due to introduction of 5G below 3800 MHz, some administrations have implemented in addition to 5G harmonised technical conditions in 3400-3800Mhz, national measures in order to protect the earths stations above 3800 MHz. Those national frameworks are giving visibility and legal certainty for the future development of earth stations in 3800-4200 MHz while also ensuring development of 5G in 3400-3800 MHz.

Downlink C-band satellite communications are and will remain concentrated in the band 3800 – 4200 MHz in European geographic area. As this is the only remaining part of C band, CEPT assessed and proposed conditions in order to preserve this band for long term development of FSS in accordance with the objectives of the EC mandate.

## Fixed Service

# Terrestrial wireless broadband systems providing local area (i.e. low/medium power) network connectivity in 3.8-4.2 GHz

drafting managed by PT1

## Background and description

CEPT assessed in this CEPT report the technical feasibility of the shared use of the 3.8-4.2 GHz frequency band by terrestrial wireless broadband systems providing local-area network connectivity (WBB LMP) according to the objective of the EC mandate.

CEPT noted early initiatives in few CEPT countries in the 3.8-4.2 GHz band either with framework or trials (experimental authorisations) in order to support a different demand compared to others frequency bands already harmonised for 5G.  Other national initiatives have been also launched in part of 2.6 GHz and 3.4-3.8 GHz in order to respond to some vertical markets with different adjacent and usages conditions.

Within 3.4-3.8 GHz, current equipment in CEPT/Europe typically rely on standardised equipment taking into consideration the protection of radars below 3.4 GHz and 5G harmonisation in 3.4-3.8 GHz (i.e. hardware band filters for 3.4-3.8 GHz). It should be noted that the “in-band blocking” parameters of those 5G commercial networks stops at 3860 MHz. Therefore from a blocking perspective, 5G networks operating below 3.8 GHz may be protected from unsynchronised WBB LMP as long as they operate above 3860 MHz. CEPT developed a detailed analysis in this CEPT report in order to address and analyse the issue.

Moreover, within 3.8-4.2 GHz, the current initial/early launch of WBB LMP in CEPT/Europe benefit from 5G equipment designed for the US market (3.45-3.98 GHz). Even when operating above 3860 MHz, this equipment will not be protected from blocking effect originating from un-synchronisation with 5G networks below 3.8 GHz. CEPT developed a detailed analysis in this CEPT report in order to address and analyse the issue. Additional studies have been done to assess WBB LMP OOB emissions in order to properly protect 5G networks operating below 3.8 GHz.

## Use cases

The 400 MHz available in the 3.8-4.2 GHz frequency band could enable terrestrial wireless broadband systems to provide a variety of services for various local users, such as local communities as well as industrial connectivity and automation. The wide range of local use-cases for different industrial and non-industrial environments will benefit from harmonised   technical conditions to respond to various vertical/industrial needs for both indoor and outdoor environments. Some industrial use-case examples in 3.8-4.2 GHz band are listed below:

Indoors: Connectivity for remote asset monitoring and control, IoT based automation, quality and control management, predictive maintenance, energy optimisation etc.

Outdoors: Connectivity for logistics in ports, IoT services in agriculture, location tracking of moving assets,  etc.

The WBB LMP could also respond to Broadcasting use case as systems deployed within TV production environments (indoor and outdoor) to support multiple camera feeds and control signals.

Some of them  are time-critical in nature and have strict requirements in e.g., latency and reliability.

## Parameters to be used for studies

See ANNEX 3:.

### Coexistence scenarios

See ANNEX 4: section 4.

### Propagation parameters

See ANNEX 4: section 3.

# Sharing Studies with in-band services

’ drafting managed by PT1

’From PT1 WI Scope, point i): protection and the future evolution and development of incumbent users sharing this band, in particular receiving satellite earth stations and terrestrial fixed links

## WBB LMP

## Fixed Service

See ANNEX 4: section 1.1.

## Fixed Satellite Service (Space-to-Earth)

See ANNEX 4: section 1.2.

In the 3.8-4.2 GHz band, licensed earth stations communicate only with geostationary satellites (GSO).

Due to its unique characteristics which cannot be substituted by Ku or Ka bands, a large number of FSS downlink earths stations migrated from the band below 3.8 GHz to 3.8-4.2 GHz (others remain below 3.8 GHz). This migration took place during the last years prior the introduction of 5G in 3.4-3.8 GHz. Sharing solutions shall ensure protection and the future evolution and development of incumbent users sharing this band, in particular receiving satellite earth stations and terrestrial fixed links.

# Compatibility studies with Adjacent band Services

’ drafting managed by PT1

’From PT1 WI Scope, point ii): coexistence of terrestrial wireless broadband systems providing local-area network connectivity and uses operating in adjacent bands such as terrestrial systems providing wireless broadband electronic communications services in the 3.4-3.8 GHz frequency band and radio altimeters on board aircraft in the 4.2-4.4 GHz frequency band.

## Mobile Service below 3.8 GHz

### 5G commercial

See ANNEX 4: section 2.1

## Fixed Satellite service (space-to-Earth) below 3.8 GHz

See ANNEX 4: section 2.2.

Due to its unique characteristics which cannot be substituted by Ku or Ka bands, FSS downlink earths stations are still in operation below 3.8 GHz.

## Aeronautical Mobile (R) service above 4.2 GHz (WAIC)

## Aeronautical radionavigation service in 4.2-4.4 GHz in 4.2-4.4 GHz (Radio altimeters)

# technical Feasibility

drafting managed by FM60

# Harmonised technical conditions

drafting managed by FM60

’From PT1 WI Comments section: PT1 will assist WG FM in defining the harmonised technical conditions, possibly a BEM.

# Standardisation needs

drafting managed by FM60

’There is no specification for 3.8-4.2 GHz 5G local access connectivity - CEPT work in response to this EC Mandate should trigger follow up action from ETSI side: Harmonised standard to be developed taken into due consideration results of CEPT studies. This section will highlight parameters to be included in future harmonised standard 3.8-4.2 GHz 5G LAN

# Conclusions

drafting managed by FM60

cept mandate

|  | EUROPEAN COMMISSION Communications Networks Content & Technology Directorate-General Connectivity Radio Spectrum Policy |
| --- | --- |

Brussels, 13 October 2021

DG CONNECT/B4

RSCOM21-40rev2

INTERNAL

RADIO SPECTRUM COMMITTEE

Working Document

Subject:	Draft Mandate to CEPT on technical conditions regarding the shared use of the 3.8-4.2 GHz frequency band for terrestrial wireless broadband systems providing local-area network connectivity in the Union

This is a Committee working document which does not necessarily reflect the official position of the Commission. No inferences should be drawn from this document as to the precise form or content of future measures to be submitted by the Commission. The Commission accepts no responsibility or liability whatsoever with regard to any information or data referred to in this document.

Draft Mandate to CEPT 
on technical conditions regarding the shared use of the 3.8-4.2 GHz frequency band for terrestrial wireless broadband systems providing local-area network connectivity in the Union

- Purpose

The Commission Communication on Connectivity for a competitive digital single market, towards a European gigabit society updated with the Commission Communication “2030 Digital Compass: the European way for the Digital Decade”, set out ambitious connectivity objectives for the Union to be achieved through the widespread deployment and take-up of very high capacity networks, including 5G. The Commission Communication ‘5G for Europe: an Action Plan’ highlighted 5G as a key enabler of the digitalisation of “vertical industries” (such as transport, logistics, automotive, health, energy, smart factories, media and entertainment). It also identified a need for coordinated action at Union level, including the identification and harmonisation of spectrum for 5G to serve innovative business models and solutions for locally licensed access to spectrum. The RSPG recognised that there is a specific demand for mid-band spectrum and recommended that Member States investigate the possible use of the band 3.8-4.2 GHz for local vertical applications (i.e. low/medium power) while protecting receiving satellite earth stations and other existing applications and services.

In addition, the Commission Communication on ‘A New Industrial Strategy for Europe’, which lays out the vision for the industrial transformation in the Union for the next 10 years stresses the importance of strengthening the digital single market to underpin the Union’s digital transition. It calls on the Union to speed up investments in 5G as a major enabler for future digital services, thus setting it at the heart of the industrial data wave.

This mandate invites CEPT to assess the technical feasibility of the shared use of the 3.8-4.2 GHz frequency band by terrestrial wireless broadband systems providing local-area network connectivity with focus on vertical users and other terrestrial wireless use cases and, on that basis, deliver harmonised technical conditions for the shared use of the band. Those harmonised technical conditions should in particular ensure the protection and the possibility of future evolution and development of incumbent spectrum users in this band (notably receiving satellite earth stations in the fixed satellite service and terrestrial fixed links) and the coexistence with spectrum users in adjacent bands (such as radio altimeters on board aircraft operating in the 4.2-4.4 GHz frequency band).

- Policy context and inputs

The RSPG has developed three Opinions (November 2016, January 2018 and January 2019) on a strategic spectrum roadmap towards 5G for Europe, in which it had identified 5G pioneer bands and addressed implementation challenges for 5G. In particular in its third opinion, the RSPG concludes that connectivity for vertical industries (‘verticals’) could be provided by mobile operator’s solutions, third-party providers and directly by verticals themselves in EU-harmonised bands for electronic communications services or in dedicated spectrum for verticals. The RSPG recommends that Member States also consider other spectrum solutions including dedicated or shared spectrum for the business/sectoral needs (‘verticals needs’) that may not be met by mobile operators. This is also confirmed by the RSPG’s Opinion of 16 June 2021 ‘on a radio spectrum policy programme’.

In its recent Opinion of 16 June 2021 ‘the RSPG recommends to study the possible use of the 3.8-4.2 GHz frequency band for local vertical applications (i.e. low /medium power), while protecting receiving satellite earth stations, as well as other existing radio applications and services.

Furthermore, in its Opinion of 16 June 2021 ‘on spectrum sharing – pioneer initiatives and bands’, the RSPG inter alia urges Member States to promote studies on sharing approaches and technologies that would lead to increased possibilities of sharing or co-existence solutions and to encourage CEPT and ETSI to cooperate in support of this policy.

At present, a number of industrial sectors are looking at 5G as an enabler of the fourth industrial revolution (Industry 4.0). The deployment of reliable and resilient wireless local- area connectivity is increasingly becoming a necessity for business-critical industrial processes, such as related to automated manufacturing in smart factories, which has also been highlighted by ICT companies.  Due to different national circumstances e.g. priorities for efficient spectrum use, Member States have addressed demand for locally licensed access to spectrum in mid-bands in a dissimilar way.

The potential deployment of terrestrial wireless broadband systems providing local-area network connectivity (with base stations operating at low/medium power) for vertical and possibly other terrestrial wireless use cases within the 3.8-4.2 GHz frequency band in the Union, subject to an authorisation decision at Member State level, requires harmonised technical conditions. This promotes ecosystem development and efficient spectrum use. It would also foster the development of innovative sharing conditions in the 3.8-4.2 GHz frequency band between terrestrial wireless broadband systems providing local-area network connectivity and the incumbent users in need of protection and the possibility of future evolution and development.

In addition, any possible usage of the frequency band 3.8-4.2 GHz in combination with spectrum resources in other bands may be further assessed in a second stage taken into account the results of this mandate.

- Justification

Pursuant to Article 4(2) of the Radio Spectrum Decision, the Commission may issue mandates to the CEPT for the development of technical implementing measures with a view to ensuring harmonised technical conditions for the availability and efficient use of radio spectrum necessary for the functioning of the internal market. Such mandates shall set the task to be performed and their timetable.

The results of this Mandate should facilitate the deployment of terrestrial wireless broadband systems providing local-area network connectivity. These should support innovation and digital industrial transformation. In recognition of existing services within the 3.8-4.2 GHz frequency band other than terrestrial wireless broadband, particular attention should be paid to ensuring the protection and the possibility of future evolution and development of receiving earth satellite stations and terrestrial fixed links. Furthermore, the coexistence with terrestrial systems providing wireless broadband electronic communications services and radio altimeters operating in adjacent bands should also be duly addressed.

- Task order and schedule

The CEPT is herewith mandated to study the feasibility of using the 3.8-4.2 GHz frequency band by terrestrial wireless broadband systems providing local-area network connectivity in a shared manner and to develop, if feasible, relevant harmonised technical conditions therefor, which are suitable for 5G technology and protect as well as ensure the possibility of future evolution and development of incumbent spectrum users within the band and in adjacent bands.

The CEPT shall, where relevant, take full account of EU law applicable and support the principles of service and technological neutrality, non-discrimination and proportionality insofar as technically possible.

The CEPT is requested to collaborate actively with all concerned stakeholders and ETSI, which develops harmonised standards for the presumption of conformity under the Radio Equipment Directive 2014/53/EU. In particular, the CEPT should take into consideration ETSI standards, which define 5G systems and facilitate shared spectrum use.

Specifically, CEPT is mandated to perform the following tasks:

1.	Study and assess the technical feasibility of the shared use of the 3.8-4.2 GHz frequency band by terrestrial wireless broadband systems providing local-area (i.e. low/medium power) network connectivity. In this regard, consider sharing solutions, including innovative features, which ensure:

i.	protection and the future evolution and development of incumbent users sharing this band, in particular receiving satellite earth stations and terrestrial fixed links;

ii.	co-existence of terrestrial wireless broadband systems providing local-area network connectivity and uses operating in adjacent bands such as terrestrial systems providing wireless broadband electronic communications services in the 3.8-4.2 GHz frequency band  and radio altimeters on board aircraft in the 4.2-4.4 GHz frequency band.

2.	Subject to the sharing solutions and the results of Task 1, as appropriate, develop a harmonised frequency arrangement as well as the least restrictive harmonised technical conditions for the shared use of the 3.8-4.2 GHz frequency band by terrestrial wireless broadband systems providing local-area connectivity. These harmonised technical conditions shall avoid interference, protect relevant incumbent uses within the band and in adjacent bands, and facilitate cross-border coordination.

Based on the results of sharing studies within the 3.8-4.2 GHz frequency band and co-existence studies with uses in adjacent bands, the CEPT  may include, where necessary, guidance on appropriate receiver characteristics for radio equipment as part of the harmonised technical conditions or/and recommend to ETSI to consider the results of those studies when developing relevant harmonised standards.

In performing the aforementioned tasks, the CEPT shall allow to the greatest extent possible channelling arrangements and effective coordination with other existing systems and services to accommodate national circumstances and market demand, and the guidance provided by the Commission in consultation with the Radio Spectrum Committee.

CEPT should provide deliverables according to the following schedule:

| Delivery date | Deliverable | Subject |
| --- | --- | --- |
| November 2022 | Interim Report from CEPT to the Commission | Description of work undertaken and interim results under this Mandate |
| July 2023 | Final Draft Report from CEPT to the Commission | Description of work undertaken and final results under this Mandate |
| March 2024 | Final Report from CEPT to the Commission, taking into account the outcome of the public consultation | Description of work undertaken and final results under this Mandate, taking into account the results of the public consultation |

In addition, the CEPT is requested to report on the progress of its work pursuant to this Mandate to all the meetings of the Radio Spectrum Committee that will be taking place during the course of the Mandate.

The Commission, with the assistance of the Radio Spectrum Committee may consider applying the results of this mandate in the Union, pursuant to Article 4 of the Radio Spectrum Decision.

Proposal for EC Decision

WBB LMP parameters for studies on 3800-4200 MHz

ECC PT1(23)135 ANNEX VIII-09

Other parameters and assumptions for studies on 3800-4200 MHz

ECC PT1(23)135 ANNEX VIII-10

Potential deployment scenarios for local area networks in the 3.8-4.2 GHz band (ECC PT1(22)100_Nokia)

Introduction

In addressing the EC Mandate tasks on the shared use of the 3.8-4.2 GHz band for local area networks, it is important to capture and address the wide range of use-cases and requirements of potential users, such as enterprises and local communities. In this contribution, we present an example use-case using a commercially available system in the 3.8-4.2 GHz frequency band, demonstrating how different Base Station (BS) deployment configurations can affect the coverage and the deployment complexity of local area networks in this frequency band for the coverage of a given industrial site.

Use cases and deployment environments

The 400 MHz available in the 3.8-4.2 GHz frequency band could enable terrestrial wireless broadband systems for local area networks to provide a variety of services for various users, such as local communities as well as industrial connectivity and automation. The wide range of use-cases for different industrial and non-industrial environments require different technical conditions to maximise capacity and cost-efficient connectivity for both indoor and outdoor environments.

Some industrial use-case examples in 3.8-4.2 GHz band are listed below, for both indoor and outdoor environments:

Indoors: Connectivity for remote asset monitoring and control, IoT based automation, quality and control management, predictive maintenance, energy optimisation etc.

Outdoors: Connectivity for logistics in ports, IoT services in agriculture, location tracking of moving assets, offshore operations etc.

Coverage scenarios

In this section, we consider an outdoor industrial use-case – coverage of an industrial site near the sea (of an area of approximately 4 km2). We evaluate the impact of different BS transmit power levels and antenna heights to the received signal strength at various locations around the industrial site. We assumed four different deployment scenarios to provide coverage to the 4 km2 industrial area using a commercially available system.

Table 7: Network deployment scenarios

| Deployment Scenario | EIRP | BS height | Number of BS location | Number of Remote Radio Head (RRH) |
| --- | --- | --- | --- | --- |
| 1 | 53.5 dBm | 15-40 m | 8 | 13 |
| 2 | 24 dBm | 15 m | 8 | 13 |
| 3 | 24 dBm | 25 m | 8 | 13 |
| 4 | 24 dBm | 5 m | 43 | 110 |

The received signal strength (Reference Signal Received Power, RSRP) is simulated at a receiver height of 1.5 m above ground.

Deployment Scenario 1

Table 8 summarizes the parameters of a simulated local area network for deployment scenario 1. Figure 1 presents the received signal strength, in terms of RSRP. To achieve optimal coverage under this scenario, we have considered in our network planning the deployment of the same type of antennas at different heights between 15 and 40 metres. The results indicate that by deploying 13 RRHs in 8 BS locations it is possible to provide adequate coverage (RSRP ≥ -105 dBm) for 90% of the whole industrial site considered, with 73.7% of it having RSRP ≥ -90 dBm.

Table 8: Network parameters for deployment Scenario 1

| Parameter | Value |
| --- | --- |
| EIRP | 53.5 dBm |
| Bandwidth | 20 MHz |
| Base Station heights | 15-40 m |
| Number of BS locations | 8 |
| Number of RRHs | 13 |
| Receiver height | 1.5 m |

Figure 1: Simulated received signal strength (RSRP) of a local area network deployment as per the parameters of deployment Scenario 1

Deployment Scenario 2

To illustrate the importance and the impact of the transmit power levels in planning the coverage of the same area, we provide a comparison of the RSRP levels when deploying a local area network with lower BS power levels than those of deployment Scenario 1 and with fixed antenna heights at 15 m.

Table 9 summarizes the parameters of a simulated local area network for deployment Scenario 2.

Table 9: Network parameters for deployment Scenario 2

| Parameter | Value |
| --- | --- |
| EIRP | 24 dBm |
| Bandwidth | 20 MHz |
| Base Station heights | 15 m |
| Number of BS locations | 8 |
| Number of RRHs | 13 |
| Receiver height | 1.5 m |

Figure 2 shows that by deploying 13 RRHs in 8 BS locations, adequate coverage (RSRP ≥ -105 dBm) is achieved only for 11.7% of the whole area considered, with 0% of it having RSRP ≥ -90 dBm.

Figure 2: Simulated received signal strength (RSRP) of a local area network deployment as per the parameters of deployment Scenario 2

Deployment Scenario 3

The same BS power level as Scenario 2 (i.e. 24 dBm) with a height of 25 m was considered to assess the impact on coverage.

Even if higher antenna heights are considered, as shown in Table 10, the coverage provided to the area of the industrial site, as seen in Figure 3, didn’t improve.

Table 10: Network parameters for deployment Scenario 3

| Parameter | Value |
| --- | --- |
| EIRP | 24 dBm |
| Bandwidth | 20 MHz |
| Base Station heights | 25 m |
| Number of BS locations | 8 |
| Number of RRHs | 13 |
| Receiver height | 1.5 m |

By deploying 13 RRHs in 8 BS location at 25m height, adequate coverage (RSRP ≥ -105dBm) is achieved for only 11.3% of the area, which is less than the coverage achieved in deployment Scenarios 1 and 2. This indicates that higher antenna heights, if considered independently, do not always present a solution for greater coverage. Therefore, having the possibility for deploying a range of antenna heights together with a range of BS transmit powers is a key aspect for finding an appropriate balance to overcome coverage challenges in industrial environments.

Figure 3: Simulated received signal strength (RSRP) of a local area network deployment as per the parameters of deployment Scenario 3

Deployment Scenario 4

Deployment Scenario 4 evaluates the impact to received signal strength levels for a local area network with BSs at 5 m height transmitting at 24 dBm (parameters in Table 11). To improve the coverage of the industrial site area, the number of RRHs and BS locations has been increased, compared to the previous scenarios. Therefore, as seen in Figure 4, by deploying 110 RRHs in 43 locations, adequate coverage is provided for 49.8% of the area, with just 4.3% of it having RSRP levels greater than or equal to -90 dBm.

Table 11: Network parameters for deployment Scenario 4

| Parameter | Value |
| --- | --- |
| EIRP | 24 dBm |
| Bandwidth | 20 MHz |
| Base Station heights | 5 m |
| Number of BS locations | 43 |
| Number of RRHs | 110 |
| Receiver height | 1.5 m |

Figure 4: Simulated received signal strength (RSRP) of a local area network deployment as per the parameters of deployment Scenario 4

Additional considerations

The results of the coverage studies of a specific industrial site under the four different deployment scenarios provided in the previous section, highlight that power limits and heights for BS deployments will impact the ability of potential users on how to utilise the 3.8-4.2 GHz band for the wide variety of industrial applications. Different environments and use-cases will require different deployment characteristics for local area networks in order to accommodate the coverage and capacity demands.

Furthermore, the need for identifying numerous suitable locations to deploy an extended number of BSs in industrial environments, as well as the requirement of extensive network planning to provide adequate coverage, will impose additional challenges and cost implications to enterprises. This would increase the risk of reduced adaptation and ecosystem development in the band. These aspects are contradictory to the “low cost – easy deployment” concept of the use of the 3.8-4.2 GHz band for local area networks.

Moreover, the current utilisation of the 3.4-3.8 GHz and the 3.8-4.2 GHz band from existing services vary significantly among CEPT countries. As a result, in accordance with the EC Mandate, the EC regulation should facilitate the freedom for national administrations to define the detailed technical licence conditions of the use of the 3.8-4.2 GHz band for local area networks, in line with their national needs, taking into account possible restrictions raised from existing utilisation of the 3.8-4.2 GHz and the adjacent bands.

Conclusions

Table 12 summarises the coverage percentages of the area of the industrial site with RSRP levels greater or equal to -105 dBm at the receiver height of 1.5 m, for the four different deployment scenarios.

Table 12: Summary of percentage of area with adequate signal strength

| Scenario No. | EIRP | BS antenna height | Number of BS location | Number of Remote Radio Head (RRH) | Percentage of area with RSRP ≥ -105 dBm |
| --- | --- | --- | --- | --- | --- |
| 1 | 53.5 dBm | 15-40 m | 8 | 13 | 90.0% |
| 2 | 24 dBm | 15 m | 8 | 13 | 11.7% |
| 3 | 24 dBm | 25 m | 8 | 13 | 11.3% |
| 4 | 24 dBm | 5 m | 43 | 110 | 49.8% |

Considering the wide range of industrial and local community type of use cases for the 3.8-4.2 GHz band, different applications will have different needs for coverage and capacity. We have presented a specific use-case of demonstrating the coverage requirements of an industrial site area (~4km2) for industrial operations. The results indicate that it is possible to provide adequate coverage for 90% of the area, using BSs with 53.5 dBm EIRP (13 RRHs in 8 locations) at heights between 15 and 40 metres. For the same area, when simulating BSs with lower power and fixed antenna heights (deployment Scenario 2 and 3), the coverage range was significantly reduced. Even when we simulated the coverage of the same area with a local area network of ~10x more RRHs in ~5x more BS locations (deployment Scenario 4), the range of adequate coverage was ~50% less compared to that achieved with the BS power levels of deployment Scenario 1.

The transmit level and the technical deployment parameters of the BSs in the 3.8-4.2 GHz band should be able to accommodate the variety of use-cases for verticals in a cost-efficient and easy-implementable manner in the different deployment environments. As an example, the use of local area networks for mining applications would require greater coverage in isolated locations, where incumbent use of the band is highly likely to be absent.

Furthermore, current use of incumbents in the 3.8-4.2 GHz as well as in the adjacent bands presents significant variations and differences among the CEPT countries.

As a result, each Member State should have the option to define the detailed technical licence conditions of the use of 3.8-4.2 GHz for terrestrial wireless broadband systems for local area networks, based on their geography, the extent of the band utilisation by incumbents, as well as the needs and the requirements for a plethora of use cases and applications. It is essential to highlight that when defining the shared use of 3.8-4.2 GHz for local area networks, one size does not fit all and that the differences of national administrations should be taken into consideration when defining the national technical licence conditions for the use of this band. By doing so, the potential users will be encouraged to utilise the band for a wide range of use-cases and will accelerate the development and the adoption of an end-to-end ecosystem in the near future.

Sharing and Compatibility studies

Coexistence study between WBB LMPapplications (low/medium power) and FSS receiving earth stations (ECC PT1(23)047_France)

The present study determines the coexistence conditions between WBB LMP stationsand FSS receiving stations in the 3.8-4.2 GHz frequency band while identifying the key parameters on the co-channel protection distances.

The study considered the example of protection of two real FSS earth stations in France. The first one is the Rambouillet Teleport site (Eutelsat) located at coordinates longitude 1.7825° and latitude 48.54944°. The second site is the CECS Aussaguel earth station located at coordinates longitude 1.49909° and latitude 43.42958.

Recommendation ITU-R P.452-16 was used to model propagation losses. This propagation model provides a prediction method for the evaluation of propagation losses from about 0.1 GHz to 50 GHz, accounting for both clear-air and hydrometeor scattering interference mechanisms.

The models within Recommendation ITU-R P.452 are designed to calculate propagation losses not exceeded for time percentages over the range 0.001 ≤ p ≤ 50%. This assumption does not imply the maximum loss will be at p = 50%. In this section, the used percentage of time is set to 20%.

Predicting with more accurately the path loss, requires knowing the exact position of all obstacles including building in the path between the transmitter and the receiver. This can be achieved by using a national terrain and buildings database: in France they are both provided by the ”Institut National de l’Information Géographique et Forestière” (IGN), with a precision of 5 m (1m is also possible but it increases significantly the computation time and provides a level of accuracy that is not necessarily needed for the purposes of this report). However, the terrain profile is taken from SRTM data of 1s.

Once the path profile between the transmitter and the receiver is determined, the P.452 is applied for a time percentage of 20%.

Characteristics of the Fixed Satellite system

In the 3.8-4.2 GHz band, licensed earth stations communicate only with geostationary satellites (GSO).

FSS protection criteria

The ITU-R Recommendation S.1432 has defined the long-term protection criterion by an I/N=-10 dB not to be exceeded for more than 20% of the time. However, the ITU-R Recommendation SF.1006 has defined the short-term criterion by an I/N=-1.3 dB not to exceed more than 0.0016% of the time. Earth station noise temperatures for FSS in the 3.6-3.8 GHz band are between 34 and 180°K.

FSS antenna

The antennas of the FSS earth stations are modelled by the diagram of Annex 3 of Appendix 8 of the Radio Regulations. Their maximum gain is between 40 and 50 dBi.

Characteristics of WBB LMP

The regulatory conditions set up by OFCOM (see hereafter) could be used as a starting point. The relevance of these conditions for WBB LMPdeployment could be confirmed and ECC PT1 could assess the protection area around earth stations, for such conditions, and, based on real earth stations deployment information, the availability of spectrum for verticals.

These OFCOM parameters (https://www.ofcom.org.uk/__data/assets/pdf_file/0035/157886/shared-access-licence-guidance.pdf) are as follows. This includes also some assumptions for the antenna heights.

Table 13:

|  |  | Low Power | Medium Power |
| --- | --- | --- | --- |
| Environment |  |  | Only rural |
| Tx power/ E.I.R.P | BS (E.I.R.P) | 18 dBm / 5 MHz for carriers >20 MHz (24 dBm for < 20 MHz) | 36 dBm / 5 MHz for carriers >20 MHz (42 dBm for < 20 MHz) |
|  | Terminal (Ptx) | 28 dBm TRP (mobile/nomadic) | 28 dBm TRP (mobile/nomadic) |
| Tx bandwidth |  | BW < 100 MHz |  |
| Additional assumptions on antenna |  |  |  |
| Antenna diagram |  | Omni-directional | Omni-directional |
| Indoor/outdoor ratio | BS | 100% outdoor | 100% outdoor |
|  | Terminal (fixed/mobile) | Indoor/outdoor | Indoor/outdoor |
| Antenna height | BS | 5 m and 10 m | 5 m, 10 m and 20 m |
|  | Terminal (fixed/mobile) | 1.5 m | 1.5 m |

Simulation results

In the present study, only the interference coming from the BS (WBB LMPdownlink) is evaluated which emit with an EIRP of 18 dBm/5 MHz for the low power BS and 36 dBm/5 MHz for the medium power BS. The simulation results are presented in terms of I/N which is calculated as follows:

I/N (dB) = e.i.r.p(dBm/5MHz)  – PL (dB) + Gr(dBi) – N (dBm/5MHz)

The propagation model used in the studies is ITU-R P.452-16, with time percentage p=20%, using a national terrain and building database. The relief and the building are respectively taken from the SRTM(1s) and IGN(5m) data.

The protection criterion considered is that of the long term (I/N = -10 dB) which leads to a threshold interference level of:

Ilong term(dBm/5MHz)=-10+10log10(1.38 10-23 TFSS°K×B5MHz)= -123 dBm/5MHz

For a receiver noise temperature of 70°K.

𝐺𝑟: Gain towards the horizon for a given azimuth of the FSS receiving from a geostationary satellite constellation.

When the FSS station sweeps geostationary orbit 𝐺𝑟 = max{Gain(θ)} where θ=(FSS→Geo^FSS→5G site).

Teleport site (Eutelsat) Earth Station in Rambouillet

Interference from low power WBB LMP BS (e.i.r.p 18 dBm/5MHz)

Figure 5: I/N from BS of antenna height of 5m (left) and of 10m (right)

Figure 6: The areas in red where the I/N exceeds -10 dB for a BS of 5m (left) and of 10m (right)

Figure 7: The areas in red where the I/N exceeds -10 dB for a BS of 20m (left) and of 30m (right)

Interference from medium power WBB LMP BS (e.i.r.p 36 dBm/5MHz)

Figure 8: I/N from BS of antenna height of 5m (left) and of 10m (right)

Figure 9: The areas in red where the I/N exceeds -10 dB for a BS of 5m (left) and of 10m (right)

Figure 10: The areas in red where the I/N exceeds -10 dB for a BS of 20m (left) and of 30m (right)

CECS Aussaguel Earth Station site in Issus

Interference from low power WBB LMP BS (e.i.r.p 18 dBm/5MHz)

Figure 11: I/N from BS of antenna height of 5m (left) and of 10m (right)

Figure 12: The areas in red where the I/N exceeds -10 dB for a BS of 5m (left) and of 10m (right)

Interference from medium power WBB LMP BS (e.i.r.p 36 dBm/5MHz)

Figure 13: I/N from BS of antenna height of 5m (left) and of 20m (right)

Figure 14: The areas in red where the I/N exceeds -10 dB for a BS of 5m (left) and of 20m (right)

Analysis of study results

From the simulation results, it can be noted that the maximum e.i.r.p and the maximum antenna height (or alternatively maximum EIRP for various range of antenna heights) of the base station have an important impact on the co-channel protection distances.

Thus, it may be necessary to define some limitation to antenna height of medium power station, since it impacts the interfering distance. Other metrics for technical conditions could be used instead, such as defining a maximum pfd at a certain distance of the WBB LMP base stations.

Additionally, limiting the bandwidth for a given WBB LMP base local coverage would ensure the possibility of

a later deployment of an earth stations, in close vicinity and receiving in other part of the 3.8-4.2 GHz band

further WBB LMP base local coverage in the same area and in another block

Of course, a spectrum mask to be met by WBB LMP base stations is also essential to ensure the protection of earth stations in adjacent blocks.

In addition, the restriction to “only rural” for medium power could be questioned since, in some countries, earth stations are also encouraged to be installed in rural areas, including to facilitate their protection from MNO networks operating in the frequency band 3.4-3.8 GHz. On the other hand, possible additional restrictions should be investigated for the antenna height given the relevance of this parameter in terms of interfering distances.

Table 14: Required separation distance (km) to protect the site of Rambouillet when considering the long-term protection criteria

| WBB LMP network | Antenna height (m) | Required separation distance (km) |
| --- | --- | --- |
| LP (BS e.i.r.p 18 dBm/5MHz) | 5 | 12 |
|  | 10 | 26 |
|  | 20 | 33 |
|  | 30 | 45 |
| MP (BS e.i.r.p 36 dBm/5MHz) | 5 | 35 |
|  | 10 | 53 |
|  | 20 | 60 |
|  | 30 | 72 |

Coexistence study between local 5G-vertical applications (for the scenarios of ECC PT1(22)100) and FSS receiving earth stations (ECC PT1(22)212_France)

The present study assess the interferences corresponding to the deployment scenarios of local 5G-vertical applications proposed in ECC PT1(22)100) and a real FSS earth stations in France which is the Rambouillet Teleport site (Eutelsat) located at coordinates longitude 1.7825° and latitude 48.54944°.

Recommendation ITU-R P.452-16 was used to model propagation losses. This propagation model provides a prediction method for the evaluation of propagation losses from about 0.1 GHz to 50 GHz, accounting for both clear-air and hydrometeor scattering interference mechanisms.

The models within Recommendation ITU-R P.452 are designed to calculate propagation losses not exceeded for time percentages over the range 0.001 ≤ p ≤ 50%. This assumption does not imply the maximum loss will be at p = 50%. In this section, the used percentage of time is set to 20%.

Predicting with more accurately the path loss, requires knowing the exact position of all obstacles including building in the path between the transmitter and the receiver. This can be achieved by using a national terrain and buildings database: in France they are both provided by the ”Institut National de l’Information Géographique et Forestière” (IGN), with a precision of 5 m (1m is also possible but it increases significantly the computation time and provides a level of accuracy that is not necessarily needed for the purposes of this report). However, the terrain profile is taken from SRTM data of 1s.

Once the path profile between the transmitter and the receiver is determined, the P.452 is applied for a time percentage of 20%.

Characteristics of the Fixed Satellite system

In the 3.8-4.2 GHz band, licensed earth stations communicate only with geostationary satellites (GSO).

FSS Protection Criteria

The ITU-R Recommendation S.1432 has defined the long-term protection criterion by an I/N=-10 dB not to be exceeded for more than 20% of the time. However, the ITU-R Recommendation SF.1006 has defined the short-term criterion by an I/N=-1.3 dB not to exceed more than 0.0016% of the time. Earth station noise temperatures for FSS in the 3.6-3.8 GHz band are between 34 and 180 K.

FSS antenna

The antennas of the FSS earth stations are modelled by the diagram of Annex 3 of Appendix 8 of the Radio Regulations. Their maximum gain is between 40 and 50 dBi.

Characteristics of 5G-vertical applications

The table below summarises the four different outdoor local area network deployment configurations proposed in ECC PT1(22)100.

Table 15:

| Parameter | Scenario 1 | Scenario 2 | Scenario 3 | Scenario 4 |
| --- | --- | --- | --- | --- |
| EIRP (dBm) | 53.5 | 24 | 24 | 24 |
| Bandwidth (MHz) | 20 | 20 | 20 | 20 |
| Base Station heights (m) | 15-40 (25m is taken in the study) | 15 | 25 | 5 |
| Number of BS locations | 8 | 8 | 8 | 43 |
| Number of RRHs | 13 | 13 | 13 | 110 |
| Receiver height (m) | 1.5 | 1.5 | 1.5 | 1.5 |

Simulation results

In the present study, only the interference coming from the BS (5G Vertical downlink) is evaluated which emit with an EIRP of 18 dBm/5 MHz for the low power BS and 47.5 dBm/5 MHz for the high power BS (scenario 1). The simulation results are presented in terms of I/N which is calculated as follows:

I/N (dB) = e.i.r.p(dBm/5MHz)  – PL (dB) + Gr(dBi) – N (dBm/5MHz)

The propagation model used in the studies is ITU-R P.452-16, with time percentage p=20%, using a national terrain and building database. The relief and the building are respectively taken from the SRTM(1s) and IGN(5m) data.

The protection criterion considered is that of the long term (I/N = -10 dB) which leads to a threshold interference level of:

Ilong term (dBm/5MHz) = -10 + 10log10(1.38 10-23 TFSSK×B5MHz)= -123 dBm/5MHz

For a receiver noise temperature of 70 K.

𝐺𝑟: Gain towards the horizon for a given azimuth of the FSS receiving from a geostationary satellite constellation. When the FSS station sweeps geostationary orbit 𝐺𝑟 = max{Gain(θ)} where θ=(FSS→Geo ^ FSS→5G site).

Teleport site (Eutelsat) Earth station in Rambouillet

Figure 15: I/N from BS deployed according to scenario 1 (left). The areas in red where the I/N exceeds -10 dB (right)

Figure 15 above shows that the interference generated on the FSS station by the 5G-vertical deployed according to scenario 1 proposed by the ECC PT1(22)100 is very high and leads to a separation distance around 60 km to protect the FSS station. Thus, a deployment of the 5G-vertical BS with an emission of 53.5 dBm/20 MHz with an antenna height of 25m will strongly interfere with the FSS stations

Figure 16: I/N from BS deployed according to scenario 2 (left). The areas in red where the I/N exceeds -10 dB (right)

Figure 16 above shows that the interference generated on the FSS station by the 5G-vertical deployed according to scenario 2 proposed by the ECC PT1(22)100 is high and leads to a separation distance around 40 km to protect the FSS station. Thus, a deployment of the 5G-vertical BS with an emission of 24 dBm/20 MHz with an antenna height of 15m will interfere with the FSS stations

Figure 17: I/N from BS deployed according to scenario 3 (left). The areas in red where the I/N exceeds -10 dB (right)

Figure 17 shows that the interference generated on the FSS station by the 5G-vertical deployed according to scenario 3 proposed by the ECC PT1(22)100 is very high and leads to a separation distance around 50 km to protect the FSS station. Thus, a deployment of the 5G-vertical BS with an emission of 24 dBm/20 MHz with an antenna height of 25m will strongly interfere with the FSS stations

Figure 18: I/N from BS deployed according to scenario 4 (left). The areas in red where the I/N exceeds -10 dB (right)

Figure 18 shows that the interference generated on the FSS station by the 5G-vertical deployed according to the scenario 4 proposed by ECC PT1(22)100 is important. However, a deployment of the 5G-vertical BS with an emission of 24 dBm and an antenna height of 5m constitutes a more favourable coexistence condition than the other scenarios. A high number of BS may increase the level of interference at the receiver and thus increase the separation distance needed.

Analysis of study results

From the simulation results, it can be noted that the maximum e.i.r.p and the maximum antenna height (or alternatively maximum EIRP for various range of antenna heights) of the base station have an important impact on the co-channel protection distances.

Thus, it may be necessary to define some limitation to antenna height of medium power station since it impacts the interfering distance.

Despite the non-consideration of the aggregation effect and by not taking into account the short-term criterion, the result of this preliminary study shows that the first three scenarios, proposed by the ECC PT1(22)100, lead to enormous interference where an important protection distance is needed. However, the fourth scenario, for a BS of 24 dBm E.I.R.P. and an antenna height of 5m, constitutes a more favourable coexistence condition than the other scenarios.

Adjacent band co-existence study between MFCN in 3400-3800 MHz and LAN in 3800-4200 MHz (ECC PT1(22)102_Orange)

Introduction

The MFCN frequency band 3.4-3.8 GHz was defined by ETSI in the European Harmonised Standard EN 301 908-24 through two notes (Note 4 and Note 5) below the Table 1-2 referring to the frequency band n77 (3300-4200 MHz) and n78 (3300-3800 MHz).

Note 4: In Europe, according to [i.24] and [i.8], radio equipment in band n77 operates between 3400 MHz and 3800 MHz (FDL_low = 3400 MHz and FDL_high = 3800 MHz).

Note 5: In Europe, according to [i.24] and [ i.8], radio equipment in band n78 operates between 3400 MHz and 3800 MHz (FDL_low = 3400 MHz and FDL_high = 3800 MHz).

The frequency band 3800-4200 MHz is not covered as an independent frequency band in the European Harmonised standard EN 301 908-24.

In this document, it is assumed that 3800-4200 MHz is an independent frequency band when defining the out of band emission mask and the receiver blocking mask, the spurious emissions is defined as 40 MHz away from the band edge, the in-band blocking and out of band blocking are also defined relative to the band edge 3800 MHz and 4200 MHz.

Analysis of Interference from LAN to MFCN

Interference from outdoor LAN Small cell to MFCN Macro cell

Simulation assumptions and scenario

As shown in Figure 19 and Figure 20, two cases are considered:

unsynchronised operation between MFCN and LAN without guard band;

unsynchronised operation with a guard band of 60 MHz.

Figure 19: Unsynchronised operation without guard band

Figure 20: Unsynchronised operation with a guard band of 60 MHz

Two co-existence scenarios are studied, as shown in Figure 21 and Figure 22 :

- Scenario: LAN outdoor Small cell is in the middle of MFCN 5G NR macro cell;

- Scenario 2: MFCN Macro cell 5G NR BS is at the LAN outdoor small cell coverage edge.

Figure 21: Scenario_1 – LAN outdoor Small cell is in the middle of MFCN 5G NR Macro cell

Figure 22: Scenario_2 – MFCN Macro cell 5G NR BS is at the LAN outdoor Small cell coverage edge

The LAN BS (Medium Range BS) emission mask is plotted in Figure 23.

Figure 23: LAN BS Tx mask

The MFCN 5G NR BS (wide area BS) is illustrated in Figure 24.

Figure 24: MFCN 5G NR BS Rx mask

The system parameters and deployment assumptions used in the simulations are summarized in Table 16.

Table 16: System parameters and deployment assumptions

| Parameters | MFCN 5G NR BS | LAN1 BS | LAN2 BS |
| --- | --- | --- | --- |
| Centre Frequency (MHz) | 3755 | 3850 | 3950 |
| Channel bandwidth (MHz) | 90 | 100 | 100 |
| BS Tx Power (EIRP dBm) | 75 (120 W) | 50, 45, 40, 35, 30 | 49, 45, 40, 35, 30 |
| 5G BS AAS antenna | 8x12 |  |  |
| Element gain (dBi) | 7.1 |  |  |
| 5G BS antenna height (m) | 25 |  |  |
| 5G BS antenna downtilt (°) | 6 |  |  |
| H/V element spacing | 0.5 for H 0.7 for V |  |  |
| Emission Mask | 3800-3840 MHz: SEM Above 3840 MHz: -30 dBm/MHz |  |  |
| BS Rx mask | 3800-3820 MHz: ACS 3820-3880 MHz: in-band blocking 3880-4000 MHz: out of band blocking |  |  |
| LAN BS Tx mask |  |  | SEM |
| LAN BS Rx mask |  |  | ACS, in-band blocking, out of band blocking |
| BS antenna pattens | ITU-R M.2101 for AAS Macro cell BS ITU-R F.1336 for non-AAS outdoor Small cell BS Omni for indoor | ITU-R F.1336 for non-AAS outdoor Small cell BS Omni for indoor |  |
| LAN BS antenna height (m) |  | 6 for outdoor 3 for indoor |  |
| LAN BS antenna downtilt (°) | -6 for Macro cell 0 for Small cell | 0 |  |
| BS noise figure (dB) | 3 | 8 for MR BS 11 for LA BS |  |
| Cell range (m) | 400 | 100 m for outdoor Small cell 50 m for indoor Small cell |  |

Table 17: 5G NR BS Tx mask from 0 to 40 MHz (120 W Tx Power)

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm | 1 MHz |

Table 18: 5G NR (LAN) Medium Range BS Tx Mask

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | Prated,x-53dB-75f_offsetMHz-0.05dB | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Prated,x - 60dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Prated,x - 60dB, -25dBm) (Note 3) | 100 kHz |

Table 19: Propagation models used in the simulations

| Scenario | Propagation model |
| --- | --- |
| MFCN Macro cell BS to UE | 3GPP TR38.901 Path Loss Model: UMa, LOS Probabilities |
| LAN Small cell BS to UE | 3GPP TR38.901 Path Loss Model: UMi, LOS Probabilities |
| LAN BS to MFCN BS | 3GPP TR38.901 Path Loss Model: UMa, LOS Probabilities |
| LAN BS to MFCN Micro cell BS | 3GPP TR38.901 Path Loss Model: UMi, LOS Probabilities |

Simulation results

        - Simulation results for the scenario_1

The simulation scenario_1 (as shown in Figure 12) results are given in Table 20.

Figure 25: Simulation secnario_1

Table 20: Simulation results (LAN cell radius 100 m in middle 5G Macro cell with BS antenna at 6 m height )

| LAN BS Tx Power (EIRP dBm) | LAN 3800-3900 MHz |  |  | LAN 3900-4000 MHz LAN 3860-3960 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | I_unwanted | I_blocking | TP loss | I_unwanted | I_blocking | TP loss |
| 50 | -83.6 | -71.9 | 30.674% | -105.2 | -122.3 | 2.268% |
| 45 | -89.3 | -77.6 | 23.201% | -110.2 | -127.3 | 1.277% |
| 40 | -93.7 | -82.1 | 17.095% |  |  |  |
| 35 | -98.9 | -87.2 | 12.331% |  |  |  |
| 30 | -103.8 | -92.1 | 8.379% |  |  |  |
| 25 | -109.0 | -97.4 | 5.381% |  |  |  |

From the simulation results in the Table 20, it can be seen that the unsynchronised LAN operation in 3800-3900 MHz create an MFCN 5G NR UL throughput loss more than 5% even with a transmitting power of 25 dBm EIRP, the limiting factor is the blocking effect.

The MFCN 5G NR UL throughput loss from the unsynchronised operation of LAN above 3860 MHz at EIRP <=50 dBm is below 3%.

Figure 26 below shows the MFCN 5G NR UL throughput loss as function of LAN BS antenna height for LAB BS EIRP=45 dBm. It can be seen at LAN BS antenna height of 20 m, MFCN 5G NR BS suffers the more interference from LAN BS.

Figure 26: MFCN 5G BS UL TP loss vs LAN BS antenna height

        - Simulation results for the scenario_2

The simulation scenario_2 (as shown in Figure 27) results are given in Table 21.

Figure 27: Simulation secnario_2

Table 21: Simulation results (LAN cell radius 100 m in middle 5G Macro cell with BS antenna at 6 m height)

| LAN BS Tx Power (EIRP dBm) | LAN 3800-3900 MHz |  |  | LAN 3900-4000 MHz LAN 3860-3960 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | I_unwanted | I_blocking | TP loss | I_unwanted | I_blocking | TP loss |
| 50 | -77.3 | -65.7 | 45.34% | -97.8 | -114.9 | 4.822% |
| 45 | -82.2 | -70.6 | 35.902% | -103.1 | -120.2 | 2.597% |
| 40 | -87.1 | -75.5 | 27.715% | -107.7 | -124.9 | 1.724% |
| 35 | -91.9 | -80.2 | 20.523% |  |  |  |
| 30 | -97.2 | -85.5 | 14.094% |  |  |  |
| 25 | -101.8 | -90.1 | 9.803% |  |  |  |
| 20 | -107.1 | -95.4 | 6.166% |  |  |  |
| 15 | -111.9 | -100.2 | 3.901% |  |  |  |

From the simulation results in the Table 21 for the scenario_2, it can be seen that the unsynchronised LAN operation in 3800-3900 MHz create an MFCN 5G NR UL throughput loss more than 5% even with a transmitting power of 20 dBm EIRP, the limiting factor is the blocking effect.

The MFCN 5G NR UL throughput loss from the unsynchronised operation of LAN above 3860 MHz at EIRP <=40 dBm is below 2%.

Figure 28 below shows the MFCN 5G NR UL throughput loss as function of LAN BS antenna height for LAB BS EIRP=40, 35, 30 dBm. It can be seen at LAN BS antenna height of 20 m, MFCN 5G NR BS suffers the more interference from LAN BS.

Figure 28: MFCN 5G BS UL TP loss vs LAN BS antenna height

Summary

The simulation results for the unsynchronised operation between LAN BS and MFCN 5G NR BS show that:

Co-existence is difficult for LAN operation in the frequency range 3800-3900 MHz, the limiting factor is the MFCN 5G NR BS receiver blocking

Co-existence is possible for LAN operation above 3860 MHz under the condition of LAN BS emission power limit EIRP <=35 dBm.

Interference from indoor LAN Smallcell to MFCN Macrocell

Simulation scenario and assumptions

The simulation scenario is illustrated in Figure 29 below.

Figure 29: Simulation scenario between LAN indoor Small cell and outdoor MFCN 5G NR Macro cell

For LAN indoor Small cell, it is assumed that LAN BS antenna height is at 3 m, an omni antenna with a gain of 3 dBi. The cell radius is 50 m, the separation distance between LAN indoor Small cell BS and the MFCN 5G NR BS is 100 m. Wall penetration loss is 15 dB with a standard deviation of 5 dB. LAN indoor Small cell follows a distribution as below:

40% ground floor;

30% 1st floor;

20% 2nd floor;

10% 3rd floor.

Simulation results

The simulation results for LAN indoor Small cell unsynchronised operation in 3800-3900 MHz are given in Table 22.

Table 22: Simulation results for the co-existence between MFCN Macro cell and LAN indoor Small cell

| LAN BS Tx Power (EIRP dBm) | Indoor LAN 3800-3900 MHz |  |  |
| --- | --- | --- | --- |
|  | I_unwanted | I_blocking | TP loss |
| 50 | -90.4 | -78.7 | 22.257% |
| 45 | -95.6 | -84.0 | 16.078% |
| 40 | -100.3 | -88.7 | 11.551% |
| 35 | -105.8 | -94.2 | 7.737% |
| 30 | -110.5 | -98.8 | 4.805% |
| 25 | -115.4 | -103.7 | 3.07% |
| 20 | -120.5 | -108.8 | 1.691% |

The simulation results in the Table 22 show that when a LAN indoor Small cell is at EIRP<=20 dBm, the MFCN 5G NR BS located at 100 m away has an UL throughput loss less than 2%.

Summary

The simulation results show for the case of unsynchronised operation between LAN indoor Small cell and MFCN 5G NR Macro cell, MFCN UL throughput loss is less than 2% with LAN indoor Smallcell BS EIRP<=20 dBm.

Outdoor LAN Small cell to outdoor MFCN Small cell

Simulation scenarios and assumptions

In the simulations, two scenarios are considered:

- Scenario_1: face-to-Face at 100 m separation, as shown in Figure 30.

Figure 30: Scenario_1, face-to-face

- Scenario_2: 50 m/50 m separation, as shown in Figure 31.

Figure 31: Scenario_2, 50 m/50 m separations

It is assumed 5G BS Medium Range BS has a noise figure NF=8 dB, panel antenna with Gain=9 dBi

Simulation results

        - Simulation results for the scenario_1

The simulation results for the scenario_1 are given in Table 23.

Table 23: Simulation results for the scenario_1

| LAN BS Tx Power (dBm) | Scenario_1 |  |  |
| --- | --- | --- | --- |
|  | I_unwanted (dBm) | I_blocking (dBm) | UL TP loss (%) |
| 40 | -91.7 | -103.9 | 14.544 |
| 35 | -96.8 | -109 | 6.009 |
| 30 | -101.7 | -113.9 | 2.326 |

From the Table 23, it can be seen that MFCN Small cell UL throughput loss is below 3% when the LAN outdoor Small cell BS EIRP is less than 30 dBm.

        - Simulation results for the scenario_2

The simulation results for the scenario_2 are given in Table 24.

Table 24: Simulation results for the scenario_2

| LAN BS Tx Power (dBm) | Scenario_1 |  |  |
| --- | --- | --- | --- |
|  | I_unwanted (dBm) | I_blocking (dBm) | UL TP loss (%) |
| 40 | -94.2 | -106.3 | 9.345 |
| 35 | -99.1 | -111.3 | 3.669 |
| 30 | -104.1 | -116.3 | 1.487 |

From the Table 24, it can be seen that MFCN Small cell UL throughput loss is below 2% when the LAN outdoor Small cell BS EIRP is less than 30 dBm.

Summary

The simulation results show that in case LAN outdoor Small cell BS EIRP is limited to 30 dBm, the MFCN outdoor Small cell UL throughput loss is less than 3%.

Analysis of interference from MFCN to LAN

Interference from MFCN Macro cell to outdoor LAN Small cell

Simulation scenarios & assumptions

5G NR Macrocell BS emission mask is plotted in Figure 32.

Figure 32: 5G NR BS Tx mask (Wide area BS)

LAN Medium range BS receiver mask is illustrated in Figure 33.

Figure 33: LAN Small cell BS Rx mask (Medium Range BS)

It is assumed LAN outdoor Small cell BS has a noise figure NF=8 dB, with a directional antenna with gain of 6 dBi.

Two scenarios are simulated:

- Scenario_1: 200m separation between MFCN 5G NR BS and LAN outdoor Small cell BS;

- Scenario_2: 100m separation between MFCN 5G NR BS and LAN outdoor Small cell BS.

Simulation results

        - Simulation results for the scenario_1

Simulation scenario_1 is illustrated in Figure 34. The simulation results are plotted in Figure 35 for different LAN outdoor Small cell BS antenna heights.

Figure 34: Scenario_1, 200 m separation distance

Figure 35: LAN outdoor Small cell UL TP loss

        - Simulation results for the scenario_2

Simulation scenario_2 is illustrated in Figure 36. The simulation results are plotted in Figure 37 for different LAN outdoor Small cell BS antenna heights.

Figure 36: Scenario_2, 100 m separation distance

Figure 37: LAN outdoor Small cell UL TP loss

Summary

The simulation results show that LAN outdoor Small cell uplink throughput loss can vary from 30% to 90% depending on the LAN Small cell BS antenna height and the separation distance. The limiting factor is the LAN BS receiver blocking. A guard band does not help much and is not sufficient.

Interference from MFCN Macrocell to indoor LAN Smallcell

Simulation scenarios & assumptions

Local area BS is used for Indoor LAN Small cell. It is assumed that the local area BS for LAN indoor Small cell has a noise figure of 11 dB. An omni-directional antenna of 3 dBi gain is used for indoor LAN Small cell.

Local area BS receiver mask is plotted in Figure 38.

Figure 38: Local area BS receiver blocking mask

5G NR UE emission mask is plotted in Figure 39.

Figure 39: 5G NR UE Tx mask

The simulation scenario is illustrated in Figure 40. In the simulation with SEAMCAT, the 5G NR UE is generated within the same area of the LAN indoor Small cell coverage area. 15 dB wall loss is used in the simulation.

Figure 40: Simulation scenario for interference from MFCN Macro cell to LAN indoor Small cell

Simulation results

LAN indoor Small cell UL throughput loss caused by interference from outdoor Macro cell BS and by interference from 5G NR UE connecting to the outdoor Macro cell BS are simulated. The simulation results are given in Table 25.

Table 25: LAN indoor Small cell UL TP loss

| Interferer | I_unwanted | I_blocking | UL TP loss (%) |
| --- | --- | --- | --- |
| 5G NR UE (UL) | -107.6 | -114.4 | 3.861% |
| 5G NR BS (DL) | -113.2 | -91.8 | 24.197% |

The simulation results show that:

Interference from indoor 5G UE connecting to outdoor Macro cell 5G BS to indoor LAN BS does not appear as a big issue;

Interference from outdoor Macro cell 5G BS to indoor LAN BS is still a problem.

Summary

Synchronisation and Semi-synchronisation

Synchronisation

The simulation results presented in section A7.2 and A7.3 show that the co-existence between MFCN 5G NR in 3710-3800 MHz and Local area network in 3800-4200 MHz is not easy when they are not synchronised. In particular the interference from MFCN 5G NR Macro cell to LAN outdoor Small cells. A guard band does not help much.

The synchronisation can be a solution to ensure the co-existence between MFCN 5G NR and Local Area Network. The synchronisation needs

- Common phase clock reference;

- Compatible frame structure

Semi-synchronisation

Usually MFCN 5G NR network has more downlink data capacity than UL, the DL/UL ratio is usually 75% / 25%. Some applications of local area network may require more UL data capacity. As described in ECC Report 296, the semi-synchronisation could be a solution for Local Area Network where it needs more UL data capacity under condition of tolerating some UL interference:

"DL to UL modifications": the default DL transmission direction in the flexible part is modified into UL

From BS-BS interference perspective, the network that modifies the default DL into UL will not interfere with the other network(s) but it will receive additional interference from the other network(s);

In case Local area network use this "DL to UL modifications" semi-synchronisation, the LAN UL throughput loss (%) will be

TP_loss = TP_loss1 * P2

Where:

TP_loss1 is the simulated LAN UL TP Loss for unsynchronised case;

P2 is the collision probability

For example, at 6 m LAN BS antenna height, the simulated LAN UL throughput loss for the second channel 3900-4000 MHz is TP_loss1 = 30%, if the "DL to UL modifications" semi-synchronisation is used, the resultant LAN UL throughput loss for different UL/DL ratios are given in the Table 26 below.

Table 26: LAN UL TP loss when using semi-synchronisation

| 5G NR DL/UL ratio | 75% / 25% | 75% / 25% | 75% / 25% |
| --- | --- | --- | --- |
| LAN DL/UL ratio | 75% / 25% | 50% / 50% | 25% / 75% |
| DL to UL collision probability | 0% | 25% | 50% |
| LAN UL TP Loss | 0% | 7.5% | 15% |

Conclusions

Based on the simulation results and the analysis, the following conclusions can be made:

- When the Local Area Network operating in 3800-4200 MHz is not synchronised with MFCN 5G NR in 3400-3800 MHz, the technical conditions for protecting MFCN UL are:

a guard band of 60 MHz (3800-3860 MHz);

LAN BS transmit power EIRP<=30 dBm/100 MHz for both outdoor and indoor Small cell deployment.

But the interference from MFCN 5G NR to LAN uplink is a problem due to LAN BS receiver blocking characteristics. The possible solution is to define a more robust LAN BS receiver blocking performance in the future harmonised standard.

Synchronisation between MFCN 5G NR in 3400-3800 MHz and Local Area Network in 3800-4200 MHz is a good solution to ensure the co-existence:

Common phase clock reference

Compatible frame structure

The possible constraint may be the DL / UL ratio for some LAN applications, in case LAN needs more UL data capacity, the “DL to UL modification” semi-synchronisation as described in the ECC Report 296 can be considered as a possible solution.

The European harmonised standard for the MFCN 5G NR BS EN 301 908-24 is not yet officially published. This standard includes the European MFCN frequency band 3400-3800 MHz but does not cover the frequency band 3800-4200 MHz band. The frequency band 3800-4200 MHz and some technical conditions, e.g., BS receiver blocking performance, based on ECC PT1 study, need to be included in the future European harmonised standard.

Sharing study between FSS ES and LANWBB LMP in the band 3800-4200 MHz (CG4G(22)017_Intelsat)

This annex concentrates on studying the required separation distance to protect FSS ES taking into account a single unsynchronized localized BS deployed for vertical use. The unsynchronized assumption is key as this entails that the band will indeed be used by local-area networks and will not be used for wide scale IMT deployment.

This annex will consider various power options for the IMT BS to determine the impact of a local-area BS into an FSS ES.

Technical characteristics

Interferer characteristics: Terrestrial wireless broadband systems providing local-area network

Case 1: directional AAS antenna modelling

The following table is based on some of the characteristics for AAS IMT BS for the frequency range 3-6 GHz presented in Annex 4.4 to 5D/716.

Table 27: Base station characteristics

| Base station characteristics |  |
| --- | --- |
| Cell radius (AAS) | 0.4 km |
| Sectorization | 1 sector |
| Frequency reuse | 1 |
| Maximum AAS BS output power | See below considerations on maximum e.i.r.p. density limits |
| TDD / FDD | TDD |
| BS TDD activity factor | 75% |

The antenna pattern chosen for the AAS is based on the parameters from the small cell from Table 9 of Annex 4.4 to 5D/716. The small cell parameters were chosen as the Work Item considers low/medium power local area terrestrial networks.

Table 28: Base station antenna characteristics

| 1 | Base station antenna characteristics |  |
| --- | --- | --- |
| 1.1 | Antenna pattern | Refer to section 5 of Recommendation ITU-R M.2101ITU-R M.2101 |
| 1.2 | Element gain (dBi) (Note 1) | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 2) | 8 × 8 elements |
| 1.7 | Horizontal/Vertical radiating element/sub-array spacing, dh /dv | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.7a | Number of element rows in sub-array, Msub | N/A |
| 1.7b | Vertical radiating element spacing in sub-array, dv,sub | N/A |
| 1.7c | Pre-set sub-array down-tilt, θsubtilt (degrees) | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 1) | 2 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element/sub-array (dBm) (Note 5, 6) | 16 |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 |
| 1.11 | Base station vertical coverage range (degrees) (Notes 3, 4, 7) | 90-120 |
| 1.12 | Mechanical downtilt (degrees) (Note 4) | 6 |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization. This means that this parameter in row 1.8 is not needed for the calculation of the BS composite antenna gain and e.i.r.p. Note 2: For the small/micro cell case, 8 × 8 means there are 8 vertical and 8 horizontal radiating elements. For the extended AAS model case, 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. Note 3: The vertical coverage range is given in global coordinate system, i.e. 90° being at the horizon. Note 4: The vertical coverage range in row 1.11 includes the mechanical downtilt given in row 1.12. Note 5: The conducted power per element assumes 8 × 8 × 2 elements for the micro/small cell case, and 4 x 8 x 2 sub-arrays for the macro case (i.e. power per H/V polarized element). Note 6: In sharing studies, the transmit power calculated using row 1.9 is applied to the typical channel bandwidth given in Table 5-1 and 6-1 respectively for the corresponding frequency bands. Note 7: In sharing studies, the UEs that are below the base station vertical coverage range can be considered to be served by the “lower” bound of the electrical beam, i.e. beam steered towards the max. coverage angle. A minimum BS-UE distance along the ground of 35m should be used for urban/suburban and rural macro environments, 5 m for micro/outdoor small cell, and 2 m for indoor small cell/urban scenarios. |  |  |

Case 2: directional non-AAS antenna modelling

The following table is based on some of the characteristics for non-AAS IMT BS for the frequency range 3-6 GHz presented in Annex 4.4 to 5D/716.

Table 29: Base station characteristics

| Base station characteristics/Cell structure |  |
| --- | --- |
| Cell radius / Deployment density (non-AAS) | Cell radius 0.4 km |
| Sectorization | 1 sector |
| Non-AAS BS downtilt | 6 degrees |
| Frequency reuse | 1 |
| Non-AAS BS antenna pattern | Recommendation ITU-R F.1336 (recommends 3.1) ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336. Vertical beamwidths of actual antennas may also be used when available. |
| Maximum Non-AAS BS output power | See below consideration on power limits |
| Maximum Non-AAS BS antenna gain | 18 dBi |
| TDD / FDD | TDD |
| BS TDD activity factor | 75% |

Case 3: Omnidirectional BS

For this case, the antenna is assumed to have a 0 dBi gain in all directions.

Power limits

For this study 3 different maximum e.i.r.p. limitations are considered:

“Low-power”: maximum e.i.r.p. = 18 dBm/5MHz

“Medium power”: maximum e.i.r.p. = 36 dBm/5MHz

Limit based on 5A/322: maximum e.i.r.p. = 24 dBm/5MHz

BS height

For all the cases considered in this study, the terrestrial local-area network base station is assumed to be at 10m height. This assumption can have an important impact on the results of the sharing studies. A higher antenna height could lead to higher separation distances.

Receiver characteristics: FSS ES

FSS parameters are based on characteristics provided by ITU-R WP 4A as well as on characteristics of existing FSS ES where indicated, as shown in the table below.

Table 30: FSS earth station parameters

| Parameter | Typical value |
| --- | --- |
| Antenna size (m) | 2.4-12 m |
| ES Carrier Bandwidth (MHz) | 40 MHz |
| Antenna reference pattern | ITU-R S.465 |
| Receiving system noise temperature | 120 K for small antennas (1.2-3 m) 70 K for large antennas (4.5 metres and above) |
| ES antenna elevation pointing | 10 degrees |
| ES Antenna Centre Height above ground | 10 m |

FSS protection criterion for in-band studies is defined by ITU-R WP 4A as shown in Table 31.

Table 31: Protection Criteria for FSS (in-band)

| Frequency Ranges | Percentage of time for which the I/N value could be exceeded (%) | I/N Criteria (dB) |
| --- | --- | --- |
| 3 600-3 800 MHz | 20% 0.005% | −10.5 −1.3 |

Simulations

Case 1: local area AAS BS impact into FSS ES receiver

Given the time varying characteristic of the AAS antenna assumed a statistical analysis is conducted. A number of iterations are performed:

The distance between the AAS antenna and the FSS ES is set at the beginning of the analysis

For each iteration:

The AAS antenna is pointing in a given direction within its coverage area (combination of its mechanical and electrical pointing)

The interference towards the  FSS ES is calculated and stored

The resulting CDF is plotted and compared with the FSS ES protection criteria

If the protection criteria are not met with the set distance, the statistical analysis is conducted once again assuming a larger separation distance.

For each of the cases presented below, the required separation along with the corresponding CDF of interference is presented.

Some intermediate results are presented below in order to provide more insight behind the results of each of the sub-cases below:

BS elevation and azimuth distribution

At each iteration, a user equipment is assumed to be randomly deployed within the BS service area. The following figure illustrates the distribution of such UEs step after step.

Figure 41:

At every step, the BS is pointing to the randomly distributed UE. The following figures present the distribution in elevation and azimuth of the BS.

Figure 42:

It’s important to note that the BS coverage is limited to a range of 30 degrees which is common for AAS antenna characteristics.

BS gain distribution towards the FSS ES

The resulting gain CDF can be established from the above pointing distribution of the BS.

Figure 43:

The gain of the AAS antenna is considered to determine the correct e.i.r.p. in the direction of the FSS ES victim:

e.i.r.p.BS→FSS=e.i.r.p.max-(GBSmax-GBS→FSS)

Clutter modelling at each iteration

This case 1 assumed two scenarios:

The first assuming no clutter attenuation;

The second assuming the clutter modelling following section 3.2 of ITU-R P.2108. The clutter was assumed systematically at the BS end, assuming that the local-area network would be deployed below the clutter height in order to mitigate interference into existing services. The following figure presents the CDF of the clutter losses. The height of the local area network is also key in ensuring that the local area network is within the surrounding clutter environment.

Figure 44:

It is important to note that the P.2108 model only applies to suburban and urban environments. As shown in the above figure this model provides an important amount of attenuation (15-50dB with an average of 32 dB).

Case 1.1: Medium power – max. e.i.r.p. = 36 dBm/5MHz

The following table summarizes the required separation distances to meet the long term and short term FSS ES protection criteria in turn. The CDF of interference versus the percentage of time for the required separation distance are presented.

Without clutter attenuation

Figure 45: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 44 km

Figure 46: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 300 km

With clutter attenuation (P.2108)

Figure 47: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 14 km

Figure 48: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 25 km

Case 1.2: Low power – max. e.i.r.p. = 18 dBm/5MHz

Without clutter attenuation

Figure 49: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 23 km

Figure 50: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 100 km

With clutter attenuation (P.2108)

Figure 51: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 2 km

Figure 52: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 8 km

Case 1.3: Power level from 5A/322 document – max. e.i.r.p. = 24 dBm/5MHz

Without clutter attenuation

Figure 53: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 29 km

Figure 54: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 170 km

With clutter attenuation (P.2108)

Figure 55: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 4.2 km

Figure 56: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 14 km

Case 2: local area non-AAS BS impact into FSS ES receiver

The non-AAS antenna has a fixed mechanical down-tilt. The antenna pointing is therefore non-time variant. The analysis will therefore assume that the non-AAS BS is pointing towards the FSS ES in azimuth. The following figure presents the non-AAS antenna pattern following F.1336:

Figure 57:

Assuming the 6 degrees mechanical downtilt, the non-AAS gain towards the FSS ES is 10.4 dBi (7.6 dB lower than the peak antenna gain).

The results are presented in the form of an interference versus distance graph for each of the cases considered below. To complement the results, several FSS ES elevations were assumed: 10, 15, 30 and 48 degrees. Once again both long-term and short-term analysis results are provided. For this case, the only time varying parameter is the propagation model. For each of the cases the percentage of time associated with the protection criteria is used when implementing the ITU-R Recommendation P.452 propagation model. The following figure presents the propagation model P.452 using different percentage of time:

Figure 58:

For this case, the clutter model P.2108 was not considered since this is not a statistical analysis.

Case 2.1: Medium power – max. e.i.r.p. = 36 dBm/5MHz

Figure 59: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 35.8 km

Figure 60: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 209.5 km

Case 2.2: Low power – max. e.i.r.p. = 18 dBm/5MHz

Figure 61: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 21.8 km

Figure 62: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 41 km

Case 2.3: Power level from 5A/322 document – max. e.i.r.p. = 24 dBm/5MHz

Figure 63: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 26.1 km

Figure 64: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 93.6 km

Additional results for cases 2.1/2.2/2.3 considering different deployment scenarios and clutter attenuation following Recommendation ITU-R P.452

Various deployment scenarios are assumed for the WBB LMP BS and FSS ES deployment: industrial site, urban, suburban and rural.

Recommendation ITU-R P.452 (Section 4.5.2), provides several clutter categories including a nominal clutter height and distance from the antenna for each. The various categories considered in this study have been reflected in the table below.

Table 32: Nominal clutter heights and distances (Rec. ITU-R P.452)

| Clutter (ground-cover) category | Nominal height, ha (m) | Nominal distance, dk (km) |
| --- | --- | --- |
| High crop fields Park land Irregularly spaced sparse trees Orchard (regularly spaced) Sparse houses | 4 | 0.1 |
| Suburban | 9 | 0.025 |
| Urban | 20 | 0.02 |
| Industrial zone | 20 | 0.05 |

The FSS ES is assumed to be in a rural or suburban area with an antenna height of 10m. This is taken to represent one of the typical FSS ES deployment. For this section the FSS ES is assumed to be pointing at 10 degrees elevation.

Two different WBB LMP BS antenna heights are assumed in turn: 10m and 20m.

The resulting I/N versus separation distances are contained in the tables below:

Results – Long term protection criteria

| BS height = 10m |  |
| --- | --- |
| Rural/Suburban BS -> Rural/suburban FSS ES | Urban site BS -> Rural/suburban FSS ES |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 35.8 km - e.i.r.p. = 24 dBm/5MHz ==> 26.1 km - e.i.r.p. = 18 dBm/5MHz ==> 21.8 km | Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 29.2 km - e.i.r.p. = 24 dBm/5MHz ==> 20.2 km - e.i.r.p. = 18 dBm/5MHz ==> 11.8 km |
| Industrial site BS -> Rural/suburban FSS ES Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 29.6 km - e.i.r.p. = 24 dBm/5MHz ==> 20.6 km - e.i.r.p. = 18 dBm/5MHz ==> 12.5 km |  |
| BS height = 20m |  |
| Same result for all deployment cases as BS is above nominal clutter height: Rural/Suburban/Urban/Industrial BS -> Rural/suburban FSS ES |  |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 42 km - e.i.r.p. = 24 dBm/5MHz ==> 31.9 km - e.i.r.p. = 18 dBm/5MHz ==> 27.2 km |  |

Results – Short term protection criteria

| BS height = 10m |  |
| --- | --- |
| Rural/Suburban BS -> Rural FSS ES | Urban site BS -> Rural FSS ES |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 210.5 km - e.i.r.p. = 24 dBm/5MHz ==> 94.3 km - e.i.r.p. = 18 dBm/5MHz ==> 41.5 km | Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 54.8 km - e.i.r.p. = 24 dBm/5MHz ==> 20.5 km - e.i.r.p. = 18 dBm/5MHz ==> 6.5 km |
| Industrial site BS -> Rural FSS ES Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 59.4 km - e.i.r.p. = 24 dBm/5MHz ==> 22.1 km - e.i.r.p. = 18 dBm/5MHz ==> 7.1 km |  |
| BS height = 20m |  |
| Same result for all deployment cases as BS is above nominal clutter height: Rural/Suburban/Urban/Industrial BS -> Rural SatPaq |  |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 208.1 km - e.i.r.p. = 24 dBm/5MHz ==> 90.5 km - e.i.r.p. = 18 dBm/5MHz ==> 38.2 km |  |

Separation distances vary depending on the deployment scenario and associated clutter:

- Long term protection criteria:

  - e.i.r.p. = 36 dBm/5MHz: 29.2 – 42 km

  - e.i.r.p. = 24 dBm/5MHz: 20.2 – 31.9 km

  - e.i.r.p. = 18 dBm/5MHz: 11.8 – 27.2 km

- Short term protection criteria:

  - e.i.r.p. = 36 dBm/5MHz: 54.8 – 210.5 km

  - e.i.r.p. = 24 dBm/5MHz: 20.5 – 94.3 km

  - e.i.r.p. = 18 dBm/5MHz: 6.5 – 41.5 km

Case 3: local area omnidirectional BS into FSS ES receiver

Similarly to case 2, the antenna pointing is non-time variant. The BS is considered to have a gain of 0 dBi in all directions. The BS e.i.r.p. is therefore the same level in all directions. The results are presented in the form of an interference versus distance graph for each of the cases considered below. It is important to note that the distance required to meet the FSS protection criteria is in this case applicable in all direction around the BS.

Case 3.1: Medium power – max. e.i.r.p. = 36 dBm/5MHz

Figure 65: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 41.5 km

Figure 66: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 276.8 km

Case 3.2: Low power – max. e.i.r.p. = 18 dBm/5MHz

Figure 67: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 27.4 km

Figure 68: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 109.2 km

Case 3.3: Power level from 5A/322 document – max. e.i.r.p. = 24 dBm/5MHz

Figure 69: Long term protection criteria I/N = -10.5 dB not to be exceeded for more than 20%
Required separation distance: 32.4 km

Figure 70: Short term protection criteria I/N = -1.3 dB not to be exceeded for more than 0.005%
Required separation distance: 167.2 km

Additional results for cases 3.1/3.2/3.3 considering different deployment scenarios and clutter attenuation following Recommendation ITU-R P.452

Various deployment scenarios are assumed for the WBB LMP BS and FSS ES deployment: industrial site, urban, suburban and rural.

Recommendation ITU-R P.452 (Section 4.5.2), provides several clutter categories including a nominal clutter height and distance from the antenna for each. The various categories considered in this study have been reflected in the table below.

Table 33: Nominal clutter heights and distances (Rec. ITU-R P.452)

| Clutter (ground-cover) category | Nominal height, ha (m) | Nominal distance, dk (km) |
| --- | --- | --- |
| High crop fields Park land Irregularly spaced sparse trees Orchard (regularly spaced) Sparse houses | 4 | 0.1 |
| Suburban | 9 | 0.025 |
| Urban | 20 | 0.02 |
| Industrial zone | 20 | 0.05 |

The FSS ES is assumed to be in a rural or suburban area with an antenna height of 10m. This is taken to represent one of the typical FSS ES deployment. For this section the FSS ES is assumed to be pointing at 10 degrees elevation.

Two different WBB LMP BS antenna heights are assumed in turn: 10m and 20m.

The resulting I/N versus separation distances are contained in the tables below:

Results – Long term protection criteria

| BS height = 10m |  |
| --- | --- |
| Rural/Suburban BS -> Rural/suburban FSS ES | Urban site BS -> Rural/suburban FSS ES |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 41.5 km - e.i.r.p. = 24 dBm/5MHz ==> 32.4 km - e.i.r.p. = 18 dBm/5MHz ==> 27.4 km | Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 35.8 km - e.i.r.p. = 24 dBm/5MHz ==> 25.9 km - e.i.r.p. = 18 dBm/5MHz ==> 21.5 km |
| Industrial site BS -> Rural/suburban FSS ES Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 36.2 km - e.i.r.p. = 24 dBm/5MHz ==> 26.2 km - e.i.r.p. = 18 dBm/5MHz ==> 21.8 km |  |
| BS height = 20m |  |
| Same result for all deployment cases as BS is above nominal clutter height: Rural/Suburban/Urban/Industrial BS -> Rural/suburban FSS ES |  |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 47.9 km - e.i.r.p. = 24 dBm/5MHz ==> 38.5 km - e.i.r.p. = 18 dBm/5MHz ==> 33.3 km |  |

Results – Short term protection criteria

| BS height = 10m |  |
| --- | --- |
| Rural/Suburban BS -> Rural FSS ES | Urban site BS -> Rural FSS ES |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 276.8 km - e.i.r.p. = 24 dBm/5MHz ==> 168.2 km - e.i.r.p. = 18 dBm/5MHz ==> 109.2 km | Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 126.2 km - e.i.r.p. = 24 dBm/5MHz ==> 29.1 km - e.i.r.p. = 18 dBm/5MHz ==> 23.1 km |
| Industrial site BS -> Rural FSS ES Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 129.3 km - e.i.r.p. = 24 dBm/5MHz ==> 29.7 km - e.i.r.p. = 18 dBm/5MHz ==> 23.5 km |  |
| BS height = 20m |  |
| Same result for all deployment cases as BS is above nominal clutter height: Rural/Suburban/Urban/Industrial BS -> Rural SatPaq |  |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 275.5 km - e.i.r.p. = 24 dBm/5MHz ==> 164.7 km - e.i.r.p. = 18 dBm/5MHz ==> 104.9 km |  |

Separation distances vary depending on the deployment scenario and associated clutter:

- Long term protection criteria:

  - e.i.r.p. = 36 dBm/5MHz: 35.8 – 47.9 km

  - e.i.r.p. = 24 dBm/5MHz: 25.9 – 38.5 km

  - e.i.r.p. = 18 dBm/5MHz: 21.5 – 33.3 km

- Short term protection criteria:

  - e.i.r.p. = 36 dBm/5MHz: 126.2 – 276.8 km

  - e.i.r.p. = 24 dBm/5MHz: 29.1 – 168.2 km

  - e.i.r.p. = 18 dBm/5MHz: 23.1 – 109.2 km

Overview of results and additional considerations

The following table provides a summary of the above results:

Table 34:

| Case # | Antenna type | Max. e.i.r.p. | Distance to meet the FSS long term protection criteria (km) | Distance to meet the FSS long short term protection criteria (km) |
| --- | --- | --- | --- | --- |
| 1.1 | AAS antenna without clutter | 36 dBm/5MHz | 44 | 300 |
| 1.2 |  | 18 dBm/5MHz | 23 | 100 |
| 1.3 |  | 24 dBm/5MHz | 29 | 170 |
| 1.1 | AAS antenna with clutter | 36 dBm/5MHz | 14 | 25 |
| 1.2 |  | 18 dBm/5MHz | 2 | 8 |
| 1.3 |  | 24 dBm/5MHz | 4.2 | 14 |
| 2.1 | Non-AAS antenna | 36 dBm/5MHz | 35.8 | 209 |
| 2.2 |  | 18 dBm/5MHz | 21.8 | 41 |
| 2.3 |  | 24 dBm/5MHz | 26.1 | 93.6 |
| 3.1 | Omnidirectional antenna | 36 dBm/5MHz | 41.5 | 276.8 |
| 3.2 |  | 18 dBm/5MHz | 27.4 | 109.2 |
| 3.3 |  | 24 dBm/5MHz | 32.4 | 167.2 |

From the results it can be seen that the required separation distance is longer to protect the short-term protection criteria. One can also note that the separation distance varies greatly depending on the assumptions taken. It is therefore paramount that certain conditions reflecting these technical assumptions are translated and reflected in regulatory text. Through this contribution, it is proposed to consider the following assumptions to be translated into regulation for the potential shared use of the 3.8-4.2 GHz frequency band for terrestrial low powered wireless broadband systems providing local-area network connectivity in CEPT:

Unsynchronized BS: it is important to highlight that these applications would only be for local area networks and should not be used for wide-spread synchronized mobile deployment. This is key in ensuring that current and future deployments of FSS ES are not impeded in the band.

Power limitation: this could be in the form of a maximum e.i.r.p. limit. Looking at the results in this annex, it is proposed to use the low e.i.r.p. level of 18dBm/5MHz

Antenna height: The study presented in this annex analysed presented results using two antenna height of 10m and 20m. As seen from the analysis, the 20m height would lead to higher separation distances given the lack of clutter attenuation on the path. It is therefore proposed to limit the height of the outdoor WBB LMP antennas to 10monly a specific antenna height of 10m. However, the higher the antenna height the more potential impact into other existing services. In addition, a lower antenna height would also help with granting clutter attenuation on the interfering path.

Antenna downtilt: The study considered different antenna types. The antenna downtilt can ensure lower levels of interference towards the horizon. A minimum downtilt is therefore proposed to be considered as a technical condition. A preliminary proposal of 6 degrees could be considered for both AAS and non AAS antennas.

Exclusion zones: Given the separation distances obtained in the study above, exclusion zones could be considered as a trigger for coordination around existing FSS ES.

Licensing framework: This was not addressed in this report. However a clear process needs to be defined to verify the protection of existing services for each local area network demand.

Protection of future FSS ES: this point is crucial and depends on the above conditions. The deployment of these local area networks for verticals should not constrain future FSS ES deployments in the band.

All the regulatory issues listed above would require the attention of a dedicated meeting. This would directly fall under task 2 of the EC mandate. As recorded in the minutes of the 58th ECC meeting: “the ECC Chairman concluded that PT1 shall continue its work on Task 1 and that WG FM shall carry out the regulatory framework”.  The points raised above shall therefore be addressed during the development of the regulatory framework under the purview of WG FM.

- Sharing study between FSS SATPAQ and WBB LMP in the band 3800-4200 MHz (CG4G(22)017_Intelsat)

This annex concentrates on studying the required separation distance to protect the SatPaq application operating under FSS taking into account a single unsynchronized localized WBB LMP BS deployed for vertical use. The unsynchronized assumption is key as this entails that the band will indeed be used by local-area networks and will not be used for wide scale IMT deployment.

This annex will consider various power options for the IMT BS to determine the impact of a WBB LMP BS into a SatPaq receiver operating in 3.8-4.2 GHz.

Technical characteristics

Interferer characteristics: Terrestrial wireless broadband systems providing local-area network

Case 1: directional AAS antenna modelling

The following table is based on some of the characteristics for AAS IMT BS for the frequency range 3-6 GHz presented in Annex 4.4 to 5D/716.

Table 35: Base station characteristics

| Base station characteristics |  |
| --- | --- |
| Cell radius (AAS) | 0.4 km |
| Sectorization | 1 sector |
| Frequency reuse | 1 |
| Maximum AAS BS output power | See below considerations on maximum e.i.r.p. density limits |
| TDD / FDD | TDD |
| BS TDD activity factor | 75% |

The antenna pattern chosen for the AAS is based on the parameters from the small cell from Table 9 of Annex 4.4 to 5D/716. The small cell parameters were chosen as the Work Item considers low/medium power local area terrestrial networks.

Table 36: Base station antenna characteristics

| 1 | Base station antenna characteristics |  |
| --- | --- | --- |
| 1.1 | Antenna pattern | Refer to section 5 of Recommendation ITU-R M.2101 |
| 1.2 | Element gain (dBi) (Note 1) | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 2) | 8 × 8 elements |
| 1.7 | Horizontal/Vertical radiating element/sub-array spacing, dh /dv | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 |
| 1.11 | Base station vertical coverage range (degrees) (Notes 3, 4, 7) | 90-120 |
| 1.12 | Mechanical downtilt (degrees) (Note 4) | 6 |
| Note 1: The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization. This means that this parameter in row 1.8 is not needed for the calculation of the BS composite antenna gain and e.i.r.p. Note 2: For the small/micro cell case, 8 × 8 means there are 8 vertical and 8 horizontal radiating elements. For the extended AAS model case, 4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays. Note 3: The vertical coverage range is given in global coordinate system, i.e. 90° being at the horizon. Note 4: The vertical coverage range in row 1.11 includes the mechanical downtilt given in row 1.12. Note 7: In sharing studies, the UEs that are below the base station vertical coverage range can be considered to be served by the “lower” bound of the electrical beam, i.e. beam steered towards the max. coverage angle. A minimum BS-UE distance along the ground of 35m should be used for urban/suburban and rural macro environments, 5 m for micro/outdoor small cell, and 2 m for indoor small cell/urban scenarios. |  |  |

Case 2: directional non-AAS antenna modelling

The following table is based on some of the characteristics for non-AAS IMT BS for the frequency range 3-6 GHz presented in Annex 4.4 to 5D/716.

Table 37: Base station characteristics

| Base station characteristics/Cell structure |  |
| --- | --- |
| Cell radius / Deployment density (non-AAS) | Cell radius 0.4 km |
| Sectorization | 1 sector |
| Non-AAS BS downtilt | 6 degrees |
| Frequency reuse | 1 |
| Non-AAS BS antenna pattern | Recommendation ITU-R F.1336 (recommends 3.1) ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336. Vertical beamwidths of actual antennas may also be used when available. |
| Maximum Non-AAS BS output power | See below consideration on power limits |
| Maximum Non-AAS BS antenna gain | 18 dBi |
| TDD / FDD | TDD |
| BS TDD activity factor | 75% |

Case 3: Omnidirectional BS

For this case, the antenna is assumed to have a 0 dBi gain in all directions.

Power limits

For this study, 2 different maximum e.i.r.p. limitations are considered:

“Low-power”: maximum e.i.r.p. = 18 dBm/5MHz

“Medium power”: maximum e.i.r.p. = 36 dBm/5MHz

BS height

For all the cases considered in this study, two different antenna heights are assumed for the WBB LMP base station: 10 and 20m. This assumption can have an important impact on the results of the sharing studies, both on the propagation loss and on the clutter assumptions.

Receiver characteristics: SatPaq

SatPaq is a smart phone sleeve that is capable of communicating with FSS satellites using the C band frequency spectrum. SatPaq parameters are provided in the table below.

Table 38: SatPaq parameters

| Parameter | Typical value |
| --- | --- |
| Carrier Bandwidth (MHz) | 0.25 MHz |
| Antenna reference pattern | See figure below (max gain 15 dBi) |
| Receiving system noise temperature (K) | G/T = -15dB/K |
| Antenna elevation pointing | 20 degrees to 50 degrees is typical |
| Antenna height | 1.5m |

The protection criterion used in the study was assumed to be I/N = -6dB.

Propagation assumptions

Propagation model

the Recommendation ITU-R P.452 model.

For case 1 (interference analysis with WBB LM using an AAS), a time dependent statistical analysis is performed since the AAS antenna pointing is time dependent. The percentage of time used in this simulation is therefore randomised.

For case 2 (interference analysis with WBB LM using a non-AAS), the analysis is non-time dependant as none of the parameters evolve with time. For this case a fixed 20% time is assumed.

For case 3 (interference analysis with WBB LM using an omnidirectional antenna), the analysis is non-time dependant as none of the parameters evolve with time. For this case a fixed 20% time is assumed.

Clutter model

The following clutter models were used in the studies:

- Case 1: Recommendation ITU-R P.2108 section 3.2. This model is applicable since the analysis is time variant and statistical in nature

- Case 2 and 3: Recommendation ITU-R P.452. The P.2108 is not applicable in this case as the analysis is not statistical since both the FSS and non-AAS antenna pointing are non-time variant.

Given its application, the SatPaq is assumed to be deployed in a rural area.

Various deployment scenarios are assumed for the WBB LMP BS deployment: industrial site, urban, suburban and rural.

Recommendation ITU-R P.452 (Section 4.5.2), provides several clutter categories including a nominal clutter height and distance from the antenna for each. The various categories considered in this study have been reflected in the table below.

Table 39: Nominal clutter heights and distances (Rec. ITU-R P.452)

| Clutter (ground-cover) category | Nominal height, ha (m) | Nominal distance, dk (km) |
| --- | --- | --- |
| High crop fields Park land Irregularly spaced sparse trees Orchard (regularly spaced) Sparse houses | 4 | 0.1 |
| Suburban | 9 | 0.025 |
| Urban | 20 | 0.02 |
| Industrial zone | 20 | 0.05 |

For case 1, P.2108 clutter model is applied at the receive and transmitter end if the antenna is within the clutter. In addition, as pointed out in Recommendation P.2108, a minimum separation distance of 250m is required to apply clutter to one end of the path and 1km is required to consider clutter applied at both ends, if applicable. For case 1, the nominal distance parameter is not used. For cases 2 and 3 that implement Recommendation ITU-R P.452 for the clutter model, the above values are taken in the different deployment cases considered.

In the case of the SatPaq receiver, the antenna height is assumed 1.5m and the deployment is considered to be in rural area. Therefore, clutter loss will be applied consistently for the receiving end.

Simulations

Case 1: local area AAS BS impact into SatPaq receiver

Given the time varying characteristic of the AAS antenna assumed a statistical analysis is conducted. A number of iterations are performed:

The distance between the AAS antenna and the SatPaq is set at the beginning of the analysis

For each iteration:

The AAS antenna is pointing in a given direction within its coverage area (combination of its mechanical and electrical pointing)

The interference towards the SatPaq is calculated and stored

The resulting CDF is plotted and compared with the SatPaq protection criteria

If the protection criteria is not met with the set distance, the statistical analysis is conducted once again assuming a larger separation distance.

For each of the cases presented below, the separation distance along with the corresponding CDF of interference is presented.

Some intermediate results are presented below in order to provide more insight behind the results of each of the sub-cases below:

BS elevation and azimuth distribution

At each iteration, a user equipment is assumed to be randomly deployed within the BS service area. The following figure illustrates the distribution of such UEs step after step.

Figure 71: Uniform distribution of potential UE target within the WBB LMP BS coverage

At every step, the BS is pointing to the randomly distributed UE. The following figures present the distribution in elevation and azimuth of the BS.

Figure 72: Resulting elevation distribution of the WBB LMP BS (left figure – BS 10m height; right figure – BS 20m height)

Figure 73: Resulting azimuth distribution of the WBB LMP BS

It’s important to note that the BS coverage is limited to a range of 30 degrees which is common for AAS antenna characteristics.

BS gain distribution towards the SatPaq

The resulting gain CDF can be established from the above pointing distribution of the BS.

Figure 74: Gain distribution of the WBB LMP BS (left figure – BS 10m height; right figure – BS 20m height)

The gain of the AAS antenna is considered to determine the correct e.i.r.p. in the direction of the SatPaq victim:

e.i.r.p.BS→SatPaq=e.i.r.p.max-(GBSmax-GBS→SatPaq)

Case 1.1: Medium power – max. e.i.r.p. = 36 dBm/5MHz

The following figures provide the results for the medium power case for various WBB LMB BS deployment environment assuming that the BS at both a 10 and 20m height. See implementation of clutter losses in section A10.1.3.2. Separation distances presented below each figure are taking for the 20% case.

| BS height = 10m |  |
| --- | --- |
| Rural/Suburban BS -> Rural SatPaq | Urban/Industrial site BS -> Rural SatPaq |
| Separation distance: ≈ 3km | Separation distance: ≈ 1.5km |
| BS height = 20m |  |
| Same result for both deployment cases as BS is above nominal clutter height: Rural/Suburban BS -> Rural SatPaq Urban/Industrial site BS -> Rural SatPaq |  |
| Separation distance: ≈ 2km |  |

Case 1.2: Low power – max. e.i.r.p. = 18 dBm/5MHz

The following figures provide the results for the low power case for various WBB LMB BS deployment environment assuming that the BS at both a 10 and 20m height. See implementation of clutter losses in section A10.1.3.2. Separation distances presented below each figure are taking for the 20% case.

| BS height = 10m |  |
| --- | --- |
| Rural/Suburban BS -> Rural SatPaq | Urban/Industrial site BS -> Rural SatPaq |
| Separation distance: ≈ 0.5-1km | Separation distance: ≈ 0.5-1km |
| BS height = 20m |  |
| Same result for both deployment cases as BS is above nominal clutter height: Rural/Suburban BS -> Rural SatPaq Urban/Industrial site BS -> Rural SatPaq |  |
| Separation distance: ≈ 0.5-1km |  |

Case 2: WBBLMP non-AAS BS impact into SatPaq receiver

The non-AAS antenna has a fixed mechanical down-tilt. The antenna pointing is therefore non-time variant. The analysis will therefore assume that the non-AAS BS is pointing towards the SatPaq in azimuth. The following figure presents the non-AAS antenna pattern following F.1336:

Figure 75:

Assuming the 6 degrees mechanical downtilt, the non-AAS gain towards the SatPaq is 10.4 dBi (7.6 dB lower than the peak antenna gain).

The results are presented in the form of an interference versus distance graph for each of the cases considered below. The SatPaq elevation is set at 20 degrees. For this case, the only time varying parameter is the propagation model. The percentage of time used in Rec. P.452 is set to 20%.

For this case, see implementation of clutter losses in section A10.1.3.2.

Cases 2.1/2.2/2.3: results for the different power levels

| BS height = 10m |  |
| --- | --- |
| Rural/Suburban BS -> Rural SatPaq | Urban site BS -> Rural SatPaq |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 10 km - e.i.r.p. = 24 dBm/5MHz ==> 4 km - e.i.r.p. = 18 dBm/5MHz ==> 2.5 km | Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 3 km - e.i.r.p. = 24 dBm/5MHz ==> 1.5 km - e.i.r.p. = 18 dBm/5MHz ==> 1 km |
| Industrial site BS -> Rural SatPaq Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 3.5 km - e.i.r.p. = 24 dBm/5MHz ==> 1.8 km - e.i.r.p. = 18 dBm/5MHz ==> 1 km |  |
| BS height = 20m |  |
| Same result for all deployment cases as BS is above nominal clutter height: Rural/Suburban/Urban/Industrial BS -> Rural SatPaq |  |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 13.5 km - e.i.r.p. = 24 dBm/5MHz ==> 4.5 km - e.i.r.p. = 18 dBm/5MHz ==> 2.5 km |  |

Case 3: WBB LMP omnidirectional BS into FSS ES receiver

Similarly to case 2, the antenna gain is non-time variant. The BS is considered to have a gain of 0 dBi in all directions. The BS e.i.r.p. is therefore the same level in all directions. The results are presented in the form of an interference versus distance graph for each of the cases considered below. It is important to note that the distance required to meet the FSS protection criteria is in this case applicable in all direction around the BS.

Cases 3.1/3.2/3.3: results for the different power levels

| BS height = 10m |  |
| --- | --- |
| Rural/Suburban BS -> Rural SatPaq | Urban site BS -> Rural SatPaq |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 14.5 km - e.i.r.p. = 24 dBm/5MHz ==> 8 km - e.i.r.p. = 18 dBm/5MHz ==> 6 km | Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 5.5 km - e.i.r.p. = 24 dBm/5MHz ==> 1.5 km - e.i.r.p. = 18 dBm/5MHz ==> 1 km |
| Industrial site BS -> Rural SatPaq Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 6.5 km - e.i.r.p. = 24 dBm/5MHz ==> 2 km - e.i.r.p. = 18 dBm/5MHz ==> 1 km |  |
| BS height = 20m |  |
| Same result for all deployment cases as BS is above nominal clutter height: Rural/Suburban/Urban/Industrial BS -> Rural SatPaq |  |
| Separation distances: - e.i.r.p. = 36 dBm/5MHz ==> 19 km - e.i.r.p. = 24 dBm/5MHz ==> 11 km - e.i.r.p. = 18 dBm/5MHz ==> 5 km |  |

Overview of results and additional considerations

The following table provides a summary of the above results on the impact into a SatPaq victim:

Table 40:

| Case # | Antenna type / WBB deployment type and height | Max. e.i.r.p. | Distance to meet the SatPaq long term protection criteria (km) |
| --- | --- | --- | --- |
| 1.1 | AAS antenna @ 10 m Rural/Suburban deployment | 36 dBm/5MHz | 3 |
| 1.2 |  | 18 dBm/5MHz | 0.5 – 1 |
| 1.1 | AAS antenna @ 10 m Urban/Industrial deployment | 36 dBm/5MHz | 1.5 |
| 1.2 |  | 18 dBm/5MHz | 0.5 – 1 |
| 1.1 | AAS antenna @ 20 m Rural/Suburban/Urban/Industrial deployment | 36 dBm/5MHz | 2 |
| 1.2 |  | 18 dBm/5MHz | 0.5 – 1 |
| 2.1 | Non AAS antenna @ 10 m Rural/Suburban deployment | 36 dBm/5MHz | 10 |
| 2.2 |  | 18 dBm/5MHz | 2.5 |
| 2.3 |  | 24 dBm/5MHz | 4 |
| 2.1 | Non AAS antenna @ 10 m Urban/Industrial deployment | 36 dBm/5MHz | 3 – 3.5 |
| 2.2 |  | 18 dBm/5MHz | 1 |
| 2.3 |  | 24 dBm/5MHz | 1.5 – 1.8 |
| 2.1 | Non AAS antenna @ 20 m Rural/Suburban/Urban/Industrial deployment | 36 dBm/5MHz | 13.5 |
| 2.2 |  | 18 dBm/5MHz | 2.5 |
| 2.3 |  | 24 dBm/5MHz | 4.5 |
| 3.1 | Omnidirectional antenna @ 10 m Rural/Suburban deployment | 36 dBm/5MHz | 14.5 |
| 3.2 |  | 18 dBm/5MHz | 6 |
| 3.3 |  | 24 dBm/5MHz | 8 |
| 3.1 | Omnidirectional antenna @ 10 m Rural/Suburban deployment | 36 dBm/5MHz | 5.5 – 6.5 |
| 3.2 |  | 18 dBm/5MHz | 1 |
| 3.3 |  | 24 dBm/5MHz | 1.5 – 2 |
| 3.1 | Omnidirectional antenna @ 20 m Rural/Suburban/Urban/Industrial deployment | 36 dBm/5MHz | 19 |
| 3.2 |  | 18 dBm/5MHz | 5 |
| 3.3 |  | 24 dBm/5MHz | 11 |

Based on those results one can note that the separation distance varies greatly depending on the assumptions taken. It is therefore paramount that certain conditions reflecting these technical assumptions are translated and reflected in regulatory text. Noting that the SatPaq is a mobile terminal and the fact that separation distances can reach values of 19km in the case of the medium power, the following proposed conditions would be needed to ensure possible coexistence with the SatPaq application:

Unsynchronized WBB LMP BS: it is important to highlight that these applications would only be for local area networks and should not be used for wide-spread synchronized mobile deployment. The studies in this document only considered the interference of a single BS taking into account the local nature of the intended deployment. This is key in ensuring that current and future deployments of FSS ES are not impeded in the band.

Power limitation: this could be in the form of a maximum e.i.r.p. limit. Looking at the results in this annex, it is proposed to use the low e.i.r.p. level of 18dBm/5MHz

WBB LMP antenna height: The study presented in this annex analysed two different antenna heights of 10m and 20m. As seen from the analysis, the 20m height would lead to higher separation distances given the lack of clutter attenuation on the path. It is therefore proposed to limit the height of the outdoor WBB LMP antennas to 10m.

Antenna downtilt: The study considered different antenna types. The antenna downtilt can ensure lower levels of interference towards the horizon. A minimum downtilt is therefore proposed to be considered as a technical condition. A preliminary proposal of 6 degrees could be considered for both AAS and non AAS antennas.

Protection of future utilisation of the band for FSS application: this point is crucial and depends on the above conditions. The deployment of these local area networks for verticals should not constrain future satellite based applications in the band.

Coexistence studies between WBS in 3.8-4.2 GHz and 5G Commercial systems in 3.4-3.8 GHz (ECC PT1(22)206_Nokia)

Introduction

The scope of the EC Mandate regarding the use of the 3.8-4.2 GHz band for terrestrial wireless broadband systems (WBS) providing local-area (low/medium power) network connectivity considers the assessment of coexistence with incumbent systems sharing this band, as well as with incumbent systems operating in the adjacent bands.

This contribution assesses the coexistence of WBS services in the 3.8-4.2 GHz band with terrestrial systems providing wireless broadband electronic communications services (typically 5G networks) in the 3.4-3.8 GHz band.

Coexistence study assumptions

Definition of Low and Medium power

The EC Mandate characterises the WBS in 3.8-4.2 GHz as low and medium power systems that provide broadband local area connectivity. In order to perform the studies and assess coexistence, the terms low and medium power need to be specified. The technical specification ETSI TS 138 104 provides a definition of the Base Station (BS) output power for both AAS and non-AAS BS types. These definitions are provided in Table 41.

Table 41: Power levels of WBS in 3.8-4.2 GHz

| Parameter | Low Power BS | Medium Power BS |
| --- | --- | --- |
| Non-AAS power level | 24 dBm (Note 1) | 38 dBm (Note 2) |
| AAS Power level (TRP) | 33 dBm (Note 3) | 47 dBm (Note 4) |
| Note 1: Rated carrier output at the antenna connector Note 2: Rated carrier output at the antenna connector Note 3: ETSI TS 138 104, Table 9.3.1-1 Note 4: ETSI TS 138 104, Table 9.3.1-1 |  |  |

Unwanted emissions

In this coexistence study, it has been assumed that the WBS in 3.8-4.2 GHz and the 5G commercial systems in 3.4-3.8 GHz are operating in an unsynchronised manner. The unsynchronised operation of the two adjacent band systems requires the consideration of the unwanted emission levels to assess coexistence.

ETSI TS 138 104 specifies the conducted transmitter and receiver characteristics for BS operating in band n77. Draft ETSI EN 301 908-24 specifies that, in Europe, radio equipment in band n77 operate between 3400 and 3800 MHz. As a result, only the generic ACS and ACLR levels from ETSI TS 138 104 would apply for WBS, while the more detailed filter characteristics of ETSI TS 138 104 would apply for the operation of 5G commercial BS systems.

Table 42 shows the adjacent band transmitter and receiver characteristics for the WBS BS.

Table 42: Adjacent band transmitter and receiver characteristics for WBS BS

| Parameter | Low Power BS | Medium Power BS |
| --- | --- | --- |
| ACLR | 45 dB | 45 dB |
| ACS | 45.6 dB | 42.6 dB |

Table 43 and Table 44 show the adjacent band transmitter and receiver characteristics for the 5G commercial BS.

Table 43: Adjacent band transmitter characteristics for 5G commercial BS

| Tx characteristics (Note 1) | Frequency offset from upper edge of the channel BW | Level (conducted power) |
| --- | --- | --- |
| Unwanted emissions | 0 MHz ≤ Δf < 5 MHz | -7dBm /100kHz at 0.05 MHz above the upper edge of the channel BW |
|  |  | -14dBm /100kHz at 5.05 MHz above the upper edge of the channel BW |
|  | 5 MHz ≤ Δf < 10 MHz | -14dBm /100 kHz |
|  | 10 MHz ≤ Δf < 40 MHz | -15 dBm /MHz |
|  | 40 MHz ≤ Δf | -30 dBm /MHz |
| Note 1: ETSI TS 138 104, Table 6.6.4.2.2.1-2: Wide Area BS operating band unwanted emission limits (NR bands above 1GHz) for Category B |  |  |

Table 44: Adjacent band receiver characteristics for 5G commercial BS

| Rx characteristics (Note 1) | Frequency offset from upper edge of the channel BW | Level |
| --- | --- | --- |
| ACS | 0 MHz ≤ Δf < 20 MHz | 37.6 dB |
| In band blocking | 20 MHz ≤ Δf < 60 MHz | 46.6 dB |
| Out of band blocking | 60 MHz ≤ Δf | 74 dB |
| Note 1: ETSI TS 138 104, Tables 7.4.1.2.-1 and -2, Tables 7.4.2.2.-0 and -1 and Table 7.5.2-1 |  |  |

Additional study parameters

The remaining parameters considered for the coexistence studies are shown in Table 45.

Table 45:

| WBS Parameter | Value for Low Power BS | Value for Medium Power BS |
| --- | --- | --- |
| Bandwidth | 100 MHz | 100 MHz |
| Outdoor antenna height | 10 m | 20 m |
| Indoor antenna height | 3 m |  |
| Noise Figure | 10 dB (Note 1) | 13 dB (Note 2) |
| I/N threshold | -6 dB | -6 dB |
| AAS antenna pattern | ITU-R M.2101 |  |
| Non-AAS antenna pattern | ITU-R F.1336 |  |
| 5G commercial Parameter | Value |  |
| Element gain | 6.4 dBi |  |
| Antenna array configuration (row x col) | 4x8 elements |  |
| Antenna sub-array | 3 |  |
| Conducted power | 28 dBm |  |
| EIRP | 72.2827 dBm |  |
| Bandwidth | 100 MHz |  |
| Antenna height | 20 m |  |
| Noise Figure | 5 dB |  |
| I/N threshold | -6 dB |  |
| AAS antenna pattern | Extended AAS Model (Note 3) |  |
| Non-AAS antenna pattern | ITU-R F.1336 |  |
| Propagation parameter | Model |  |
| Propagation model | ITU-R P.452 (20% of time) |  |
| Clutter model | ITU-R P.2108 (random % of time) |  |
| Indoor wall loss | ITUI-R P.2109 |  |
| Note 1: Picocell Noise Figure as per ITU-R M.2292 Note 2: Microcell Noise Figure as per ITU-R M.2292 Note 3: 3GPP TR 38.803, Section 5.2.3.2.4: https://www.3gpp.org/ftp/Specs/archive/38_series/38.803/38803-e30.zip |  |  |

Adjacent band Coexistence between WBS (interferer) and 5g commercial (victim)

For the assessment of coexistence between the unsynchronised operation of WBS and 5G commercial, two different deployment scenarios are considered. An urban scenario where clutter is applied in both WBS and 5G commercial BS sides and a suburban scenario where clutter is applied only in one of the Tx/Rx sides.

Coexistence between WBS and 5G commercial in an urban scenario

The results of the coexistence analysis for the urban scenario, are shown in Figure 76 and Figure 77 below.

Figure 76: Urban environment, WBS into AAS 5G BS

Figure 77: Urban environment, WBS into non-AAS 5G BS

From the above figures, it can be observed that in the simulation of the urban scenario, the separation distances for WBS BS to satisfy the 5G commercial I/N criterion of -6dB is less than 1km.

Coexistence between WBS and 5G commercial in a suburban scenario

The results of the coexistence analysis for the urban scenario, are shown in Figure 78 and Figure 79 below.

Figure 78: Suburban, WBS into AAS 5G BS

Figure 79: Suburban, WBS into non-AAS 5G BS

From the above figures, it can be observed that in the simulation of the suburban scenario, the separation distances for WBS BS to satisfy the 5G commercial I/N criterion of -6dB is less than 1km.

Adjacent band Coexistence between 5G commercial (interferer) and WBS (victim)

Coexistence between 5G commercial and WBS in an urban scenario

The results of the coexistence analysis for the urban scenario, are shown in Figure 80 and Figure 81 below.

Figure 80: Urban, AAS 5G BS into WBS

Figure 81: Urban, non-AAS 5G BS into WBS

From the above figures, it can be observed that in the simulation of the urban scenario, the separation distances for 5G commercial BS to satisfy the WBS I/N criterion of -6dB is less than 1km.

Coexistence between 5G commercial and WBS in the suburban scenario

The results of the coexistence analysis for the suburban scenario, are shown in Figure 82 and Figure 83 below.

Figure 82: Suburban, AAS 5G BS into WBS

Figure 83: Suburban, non-AAS 5G into WBS

From the above figures, it can be observed that in the simulation of the suburban scenario, the separation distance for 5G commercial BS to satisfy the WBS outdoor medium power I/N criterion of -6dB is approximately 5km. For the rest of the WBS deployment types, the required separation distances are up to approximately 1km.

Conclusions from the coexistence studies

From the coexistence analysis that was presented in Section 3, it can be concluded that for the unsynchronised adjacent band operation of WBS and 5G commercial, the separation distances required to satisfy the I/N threshold of -6dB are less than 1km when the WBS is the interfering system and up to approximately 5km when the 5G commercial BS is the interfering system, depending on different WBS deployment types.

More specifically, in the urban scenario, where clutter is applied in both the Tx and the Rx side, the separation distances do not exceed 1km.

In the suburban scenario, where clutter is applied in only one side of the Tx or the Rx, the separation distances to satisfy the I/N criterion of -6dB are higher when the 5G commercial system is the interferer (approximately up to 5km) compared to when the WBS system is the interferer (less than 1km for all WBS deployment types).

In all scenarios, the separation distances as a result of the interference from and to indoor low power WBS BS appears to be minimal and in all cases less than 1km.

In all scenarios, the separation distances as a result of the interference from and to outdoor low power WBS BS follows closely the behaviour observed for the indoor low power WBS BS.

At this point, it is important to highlight that the deployment scenarios of WBS systems in the 3.8-4.2 GHz, are not expected to provide nationwide coverage similar to public 5G networks. They are expected to be deployed locally, providing focused and area specific connectivity either or both indoors and outdoors.

While the synchronised operation of WBS and 5G commercial systems is not expected to be problematic, in the unsynchronised operation, the coexistence studies indicate that the separation distances required to satisfy the WBS I/N coexistence criterion are higher than those required for the 5G commercial systems in the suburban scenario, while they are very similar in the urban scenario. As a result, it can be seen from the analysis that the required separation distances so that the deployments of an unsynchronised WBS operator can avoid interference from 5G commercial, already satisfy the separation distances required to avoid causing interference to 5G commercial services.

Sharing study between WBS base stations and FSS earth stations in the band 3.8-4.2 GHz (ECC PT1(23)049_Ericsson)

PARAMETERS AND DEPLOYMENT CONSIDERATIONS

The following tables present the wireless broadband systems (WBS) system and deployment related parameters. Unless otherwise stated, most of the parameters are based on the agreed technical and operational characteristics in the ITU-R Working Party 5D (WP 5D) Chairman’s Report Annex 4.4 (Document 5D/716).

Table 46: WBS system and deployment-related parameters

| Parameter | Value |
| --- | --- |
| Base Station |  |
| Carrier frequency | 3.85 GHz |
| Channel bandwidth | 100 MHz |
| BS Antenna height | 10 m |
| Cell radius | 400 m |
| Sectorization | 1 sector |
| Frequency reuse | 1 |
| BS TDD activity factor | 75% |
| Network loading factor | 50% |
| User Terminal |  |
| UE height | 1.5 m |
| Indoor user terminal usage | 70% |
| Indoor user terminal penetration loss | Rec. ITU-R P.2109-1 |
| UE density for terminals that are transmitting simultaneously | 3 UEs per sector |
| UE TDD activity factor | 25% |

Table 47: Antenna and power characteristics for WBS

| Parameter | Value |
| --- | --- |
| Base station (AAS) |  |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration* (Row × Column) | 8×8 and 4×4 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.7 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element for 8×8 AAS** (dBm) | Medium power: 28 (corresponding to a TRP = 47 dBm) Low power: 14 (corresponding to a TRP = 33 dBm25) |
| Conducted power (before Ohmic loss) per antenna element for 4×4 AAS** (dBm) | Medium power: 34 (corresponding to a TRP = 47 dBm25) Low power: 20 (corresponding to a TRP = 33 dBm25) Additional case: 15.5 (corresponding to a max. EIRP = 36 dBm/5 MHz (49 dBm/100 MHz), and a TRP = 28.55 dBm) |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range*** (degrees) | 90-120 |
| Mechanical downtilt (degrees) | 10 |
| Maximum base station EIRP for 8×8 AAS (dBm/5MHz) | Medium power: 60.52 (73.53 dBm/100MHz) Low power: 46.52 (59.53 dBm/100MHz) |
| Maximum base station EIRP for 4×4 AAS (dBm/5MHz) | Medium power: 54.48 (67.49 dBm/100MHz) Low power: 40.48 (53.49 dBm/100MHz) Additional case: 36 (49 dBm/100MHz) |
| Base station (non-AAS) |  |
| Antenna pattern | Refer to Recommendation ITU-R F.1336 |
| Non-AAS BS antenna pattern | Recommendation ITU-R F.1336 (recommends 3.1) ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336. Vertical beamwidths of actual antennas may also be used when available. |
| Antenna polarization | Linear ±45º |
| Feeder Loss (dB) | 3 |
| Maximum non-AAS BS output power (before feeder loss) (dBm) | Medium power: 38 Low power: 2427 |
| Maximum non-AAS BS antenna gain (dBi) | Medium power: 18 Low power: 0 |
| Mechanical downtilt (degrees) | 10 |
| Maximum base station EIRP for non-AAS (dBm/5MHz) | Medium power: 40 (53 dBm/100MHz) Low power: 8 (21 dBm/100MHz) |
| User Terminal |  |
| Typical antenna gain (dBi) | −4 |
| Body loss (dB) | 4 |
| Power control model | Refer to Recommendation ITU-R M.2101 |
| Maximum user terminal output power, PCMAX (dBm) | 23 |
| Power target value per RB, P0_PUSCH (dBm) | −87.2 |
| Path loss compensation factor, α | 0.8 |

* For the small/micro cell case, for example, 8×8 means there are 8 vertical and 8 horizontal radiating elements.

** For example, for an 8×8 AAS, the conducted power per element assumes 8×8×2 elements (i.e., power per H/V polarized element).

*** The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments, 5 m for micro/outdoor small cell, and 2 m for indoor small cell/urban scenarios.

The FSS ES parameters considered in this study are based on the agreed technical, operational characteristics, and protection criteria of FSS systems provided by WP 4A (Document 5D/734). Table 3 contains the FSS ES parameters used in this study and Table 4 contains the FSS protection criteria

Table 48: FSS ES Parameter

| Parameter | Value |
| --- | --- |
| Antenna diameter (m) | 3 |
| Peak antenna gain (dBi) | 39.5 |
| Antenna pattern | Refer to Recommendation ITU-R S.465 |
| Receiving system noise temperature | 120 K for small antennas (1.2-3 m) 70 K for large antennas (4.5 metres and above) |
| Antenna elevation angle (degrees) | 10 |
| Antenna height (m) | 10 |

Table 49: FSS ES Protection Criteria

| Frequency Ranges | Percentage of time for which the I/N value could be exceeded (%) | I/N Criteria (dB) |
| --- | --- | --- |
| 3 600-3 800 MHz (s-E) | 20% 0.005% | −10.5 −1.3 |

The protection criteria specified are related to the required availability of FSS links which is associated with time. However, Monte Carlo sharing studies conducted between FSS and WBS systems under WRC-23 assumptions may involve other considerations based on additional variables which are not varying in the time domain (e.g., geographical locations in the space or deployment domain associated with WBS positions). Thus, it may be appropriate to understand percentages as being in other domains, such as time, location, and probability.

DEPLOYMENT

A FSS earth station (ES) located in Rambouillet Teleport site (48.549° N, 1.783° E) is assumed to perform this study. The WBS local network consists of a single base station (BS). In each snapshot of the Monte Carlo simulation, 3 user equipments (UEs) with random azimuth angles are randomly located within each sector where both azimuth and location are uniformly distributed. In this study we consider two cases, one case where the FSS ES and the WBS BS are pointing towards each other (Figure 84), and another case where the FSS ES is pointing towards the WBS BS but the WBS BS is pointing towards the opposite direction of the FSS ES (Figure 85). Figure 84 and Figure 85 show an exemplary deployment snapshot with a separation distance between the FSS ES and the WBS BS of 3 km.

Figure 84. Deployment comprising a single FSS ES and a single WBS BS (case 1)

Figure 85. Deployment comprising a single FSS ES and a single WBS BS (case 2)

ADDITIONAL CONSIDERATIONS

Recommendation ITU-R P.2001 time percentage

The Rec. ITU-R P.2001 is used in this sharing study between stations on the surface of the Earth as specified by the ITU-R WPs 3K and 3M (document 5D/722). For Monte Carlo simulations, this recommendation specifies that the time percentage (Tpc) in each snapshot should be randomly generated in the range 0-100%, thus we use a random variable with a uniform distribution for Tpc.

Clutter loss

As elaborated in our contribution PT1_CG4G(22)007, due to the lack of exact information regarding vegetation/forest areas and other obstacles along the propagation path, the use of the clutter loss model in Rec. ITU-R P.2108 is a good compromise to account for the additional attenuation due to vegetation and/or other objects. In this study, the clutter losses for terrestrial paths are calculated in accordance with Rec. ITU-R P.2108-1 using a uniformly distributed random percentage of locations on at least one of the ends of the propagation path. It is noted that this Recommendation indicates that statistical models are to be used when precise knowledge of the radio path is not known such as the width of streets, heights of buildings, and depth of vegetation.

Monte carlo Study Results

Intermediate results for low power WBS with a 4×4 AAS (TRP = 33 dBm)

Assuming the deployment in Figure 84, the following intermediate results correspond to the case of a low power WBS BS with 4×4 AAS.

Figure 86 shows the CDF curves of the FSS ES antenna gain towards the BS (left) and vice versa (right). The BS antenna gain includes the Ohmic losses. The BS highest gain towards the ES is 18.11 dB which is in line with the gain of an antenna array with 4x4 elements and an element gain of 6.4 dBi (6.4 + 10log10(16) = 18.4412 dBi).

Figure 86: CDF curves of the antenna gains from the FSS ES towards the BS and vice versa

Figure 87 shows the CDF curves of the FSS ES antenna gain towards the UEs (left) and vice versa (right). The UEs antenna gain has a value of -4 dBi for all snapshots.

Figure 87: CDF curves of the antenna gains from the FSS ES towards the UEs and vice versa

To calculate the UE power control gain, Figure 88 shows the CDF curves of the UEs antenna gains towards the BS (left) and vice versa (right). The BS antenna gain includes the Ohmic losses. As expected, the BS highest gain value is almost 18.4412 dB and the UEs antenna gain has a value of -4 dBi always.

Figure 88: CDF curves of the antenna gains from the UEs towards the BSs and vice versa

Figure 89 shows the CDF curve of the coupling loss between the UEs and its BS (left) and the histogram of the UEs power control gain (right), namely, the UE power reduction assuming a power control with 3 dB steps.

Figure 89: CDF of the coupling loss between UEs-BS, and histogram of UL power control gain

The coupling loss is calculated as follows:

CouplingLoss=-GBS→UE-GUE→BS+PathLoss+ClutterLoss+AdditionalLosses

In Figure 89, the path losses are assumed to be free space path losses (FSPL), the clutter loss model is based on Rec. ITU-R P.2108 (random percentage of locations), and the additional losses include the building entry and body losses.

Figure 90 shows the CDF curves of the coupling loss between the BS and the FSS ES (left) and between the UEs and the FSS ES (right), where the path loss model is based on Rec. ITU-R P.2001 using a smooth terrain profile and the clutter loss model is based on Rec. ITU-R P.2108. No polarization losses are considered.

Figure 90: CDFs of the coupling loss between BS-FSS ES and UEs-FSS ES

For a separation distance of 14.4 km and 14.5 km between the FSS ES and the WBS BS, Figure 91 shows the CDF curves of the I/N from a low power WBS BS with 4x4 and 8x8 AASs, respectively, and the aggregated I/N from the UEs. As can be seen, the long-term protection criterion is not exceeded by any of the curves. As expected, the interference is dominated by the BS and not the UEs.

Figure 91: CDF curves of the aggregated I/N from WBS BS and UEs

Note that the size of the AAS antenna changes very little the long-term criterion results.

Summary of results (long-term criterion)

Assuming the deployment in Figure 84 (i.e., WBS BS and FSS ES pointing towards each other), in the next table are summarized the separation distances for the long-term protection criterion assuming clutter losses on one and on both ends of the propagation path.

Table 50: Separation distances to prevent harmful interference to a FSS ES for the scenario in Fig. 1 (long-term criterion)

| Case | Corresponding maximum EIRP [dBm/5MHz] | Clutter losses on one end [km] | Clutter losses on both ends [km] |
| --- | --- | --- | --- |
| Medium power with 8×8 AAS (TRP = 47 dBm) | 60.52 | 23 | 3.3 |
| Medium power with 4×4 AAS (TRP = 47 dBm) | 54.48 | 22.8 | 3.2 |
| Medium power with non-AAS (Conducted power = 38 dBm) | 40 | 11.1 | < 1 |
| Low power with 8×8 AAS (TRP = 33 dBm) | 46.52 | 14.5 | < 1 |
| Low power with 4×4 AAS (TRP = 33 dBm) | 40.48 | 14.4 | < 1 |
| Low power with non-AAS (Conducted power = 24 dBm) | 8 | 1.2 | < 1 |
| Additional case with 4×4 AAS (TRP = 28.55 dBm) | 36 | 12.1 | < 1 |

Assuming the deployment in Figure 85 (i.e., WBS BS pointing towards the opposite direction of the FSS ES), in the next table are summarized the separation distances for the long-term protection criterion assuming clutter losses on one and on both ends of the propagation path.

Table 51: Separation distances to prevent harmful interference to a FSS ES for the scenario in Fig. 2 (long-term criterion)

| Case | Corresponding maximum EIRP [dBm/5MHz] | Clutter losses on one end [km] | Clutter losses on both ends [km] |
| --- | --- | --- | --- |
| Medium power with 8×8 AAS (TRP = 47 dBm) | 60.52 | 3.9 | < 1 |
| Medium power with 4×4 AAS (TRP = 47 dBm) | 54.48 | 3.7 | < 1 |
| Medium power with non-AAS (Conducted power = 38 dBm) | 40 | < 1 | < 1 |
| Low power with 8×8 AAS (TRP = 33 dBm) | 46.52 | < 1 | < 1 |
| Low power with 4×4 AAS (TRP = 33 dBm) | 40.48 | < 1 | < 1 |
| Low power with non-AAS (Conducted power = 24 dBm) | 8 | < 1 | < 1 |
| Additional case with 4×4 AAS (TRP = 28.55 dBm) | 36 | < 1 | < 1 |

Sensitivity analysis (short-term criterion)

We provide the separation distances for the short-term criterion for WBS with AAS and with clutter losses only on one end of the propagation path. However, it is noted that this criterion has not been completed in the expert group ITU-R WP4A as indicated in the Document 5D/734, thus these results are not representative in sharing studies and should not determine overall resolutions.

Additionally, note that these probabilities are not directly comparable with percentages of the FSS protection criteria since those are percentages of time while the probabilities calculated in this study are not entirely in the time domain.

Table 52 contains the summary of results.

Table 52: Separation distances between WBS and FSS ES (short-term criterion)

| Case | Corresponding maximum EIRP [dBm/5MHz] | Scenario Fig. 1 [km] | Scenario Fig. 2 [km] |
| --- | --- | --- | --- |
| Medium power with 8×8 AAS (TRP = 47 dBm) | 60.52 | 92 | 15 |
| Medium power with 4×4 AAS (TRP = 47 dBm) | 54.48 | 84 | 12 |
| Low power with 8×8 AAS (TRP = 33 dBm) | 46.52 | 39 | < 4 |
| Low power with 4×4 AAS (TRP = 33 dBm) | 40.48 | 32 | < 2.5 |
| Additional case with 4×4 AAS (TRP = 28.55 dBm) | 36 | 23 | < 1.5 |

Concluding remarks

The results provided in this contribution are derived with a set of assumptions that attempts to reflect realistic scenarios. In general, separations distances ranging from less than 1 kilometre to few tens of kilometres may be needed to prevent harmful interference to FSS earth stations.

For the pessimistic situation where the WBS base station and the FSS earth station are pointing exactly towards each other (Figure 84), results indicate that the longest separation distance is 23 km for medium power WBS base stations with AASs (corresponding to a maximum EIRP = 60.52 dBm/5MHz). For this case, if natural or artificial clutter is present at both ends of the propagation path, separation distances are reduced to 3.3 km.

Conversely, for the situation where the WBS base station is pointing opposite to the FSS earth station (Figure 85), results indicate that the longest separation distance is 3.9 km for medium power WBS base stations with AASs (corresponding to a maximum EIRP = 60.52 dBm/5MHz). If natural or artificial clutter is present at both ends of the propagation path, separation distances are below 1 km for all cases.

For the cases presented in this study, note that the size of the AAS antenna changes very little the long-term criterion results.

By means of coordination on a case-by-case basis, e.g., boresight pointing direction of the WBS base station, separation distances can be decreased to just few kilometres to prevent harmful interference to FSS earth stations for a wide range of WBS base station antenna configurations and EIRP levels.

Coexistence study of WBB LMP with MFCN and FSS ES ECC PT1(23)039_Nokia)

Introduction

In this contribution we present a coexistence analysis of WBB LMP BS in 3.8-4.2 GHz with MFCN services in the adjacent services below 3.8 GHz as well as with FSS ES in the same band within 3.8-4.2 GHz. Taking into account the scope of the EC Mandate as well as the guidance from ECC, we examine coexistence based on the existing UK and Norwegian frameworks, applying an incremental approach with the aim to develop a better understanding of the coexistence feasibility.

Parameters of WBB LMP

The ECC guidance based on the scope of the EC Mandate recommended:

- To consider studies based on the UK and Norwegian frameworks as a starting point.

- To adopt an increment approach based on UK and Norwegian frameworks for WBB LMP.

- To cover a range of coexistence conditions covering both AAS and non-AAS for medium power WBB BS and only non-AAS for low power WBB BS

Taking into account the above, in Table 1 we present the deployment characteristics of the WBB LMP BS that have been agreed in the CG discussions based also on the guidance given at WP5D.

Table 1: Deployment characteristics of WBB LMP BS

| WBB BS parameter | Low power BS | Medium power BS |
| --- | --- | --- |
| Carrier frequency | 3850 MHz | 3850 MHz |
| Bandwidth | 100 MHz | 100 MHz |
| Antenna height | 10m | 20m |
| Downtilt | - | 10 degrees |
| BSS TDD activity factor | 75% | 75% |
| Non-AAS pattern | F.1336 omni | F.1336 sectorial |
| AAS pattern | - | M.2101 |
| Out of band emissions | Specified in Table 6 | Specified in Table 6 |

According to the existing regulatory frameworks in the UK and Norway, both AAS and non-AAS antennas are permitted for WBB services. However, these frameworks only specify a single EIRP PSD level, which covers both the use of AAS and non-AAS antennas. In this analysis, the TRP levels for the AAS antennas as well as the output power levels for the increment analysis of both non-AAS and AAS antennas, as also suggested in ECC guidance, are based on values from ETSI TS 138 104.

ETSI TS 138 104 as also described in contributions ECC PT1_CG4G(22)016 from Lithuania and ECC PT1_CG4G(22)029 from Norway, specifies that the maximum output power levels for the different local and medium range base station classes are as shown in Table 2:

Table 2: BS classes and corresponding output power levels from ETSI 138 104

|  | Local Area BS | Medium Range BS |
| --- | --- | --- |
| BS Type 1-C output power at individual antenna connector | ≤ 24 dBm (or 0.25 W) | ≤ 38 dBm (or 6.3 W) |
| BS Type 1-H output power at individual TAB connector | ≤ 24 dBm | ≤ 38 dBm |
| BS Type 1-H output power at the RIB | ≤ 24 dBm + 10log(NTXU,counted) | ≤ 38 dBm + 10log(NTXU,counted) |
| BS Type 1-O TRP output power at the RIB | ≤ 33 dBm (or 2 W) | ≤ 47 dBm (or 50.1 W) |

Considering the above, a number of different non-AAS and AAS configurations have been identified and examined in this contribution. These configurations are shown in Table 3 below, based on the existing regulatory frameworks, as well as considering the incremental approach.

Table 3: Baseline and incremental power levels for WBB LMP BS

| WBB BS parameter | Low power BS | Medium power BS | Note for the Medium power BS |
| --- | --- | --- | --- |
| NON-AAS |  |  |  |
| Non-AAS BS EIRP (UK/Nor framework) | 31 dBm (with 0dBi antenna gain) | 49 dBm | 39 dBm sum of conducted power from all connectors + 10dBi antenna gain |
| Non-AAS BS EIRP (incremental) |  | 53 dBm | 43 dBm sum of conducted power from all connectors + 10dBi antenna gain |
| AAS |  | TRP + antenna gain |  |
| AAS incremental approach |  | 37 dBm + 21.5 dBi (4x8) | EIRP = 58.5 dBm |
|  |  | 37 dBm + 24.5 dBi (8x8) | EIRP = 61.5 dBm |
|  |  | 43 dBm + 21.5 dBi (4x8) | EIRP = 64.5 dBm |
|  |  | 43 dBm + 24.5 dBi (8x8) | EIRP = 67.5 dBm |

It is noted, as also highlighted in contribution ECC PT1_CG4G(22)022 that the output power levels specified in ETSI TS 138 104 are irrespective of the channel bandwidth. As a result, the comparison of the above output power levels for medium power BS levels with those of macro BS below 3.8 GHz, indicates that they are at least 13-17 dB lower than the non-AAS macro BS levels of ECC Report 331 and at least 17-23 dB lower than the AAS macro BS levels of ECC Report 281.

Coexistence of WBB lmp with MFCN services below 3.8 GHz

Coexistence parameters for MFCN

Since WBB LMP and MFCN services operate in adjacent bands, it is necessary to take into account the effect of emission masks and receiver filters when examining their coexistence.

The MFCN Rx filter considered in this study is taken from ETSI 138 104 and is shown in the Table 4 below:

Table 4: MFCN Rx characteristics

| Rx characteristics | Frequency offset from upper edge of the channel BW | Level |
| --- | --- | --- |
| ACS | 0 MHz ≤ Δf < 20 MHz | 37.6 dB |
| In band blocking | 20 MHz ≤ Δf < 60 MHz | 46.6 dB |
| Out of band blocking | 60 MHz ≤ Δf | 74 dB |

The operational characteristics of the MFCN macro cell are taken from the guidance given at WP5D and are shown in the Table 5 below:

Table 5: MFCN deployment characteristics

| MFCN BS Parameter | Value |
| --- | --- |
| Antenna pattern | Extended AAS model |
| Channel BW | 100 MHz |
| Element gain (dBi) | 6.4 dBi |
| Antenna array configuration (Row x Column) | 4x8 elements |
| Number of element rows in sub-array | 3 |
| Horizontal coverage range | ±60 degrees |
| Vertical coverage range | 90-100 degrees |
| Antenna height | 20 |
| I/N protection threshold | -6 dB |

When considering the deployment of LMP WBB in the 3.8-4.2 GHz in relation to the operation of MFCN services below 3.8 GHz, it is possible to assume different modes of operation; synchronised, semi-synchronised and unsynchronised. In the UK regulatory framework, irrespective of whether synchronisation is considered or not, the EIRP emanating from the transmissions outside the permitted frequency blocks shall not exceed the limits shown in Table 6. The same BEM mask is also applicable in the Norwegian regulatory framework. In addition, as per ECC Decision 11(06) and as suggested in contribution ECC PT1_CG4G(22)019 the out of band emission levels when using AAS antennas for LMP WBB BS, are also shown in Table 6.

Table 6: LMP WBB out of band characteristics

| Rx characteristics | Non-AAS e.i.r.p. limit dBm/5MHz per antenna | AAS TRP limit dBm/5MHz per cell |
| --- | --- | --- |
| 3800-3795 MHz | Min(Pmax-40, 21) | Min(Pmax’-40, 16) |
| 3795-3790 MHz | Min(Pmax-43,15) | Min(Pmax’-43, 12) |
| 3790-3760 MHz | Min(Pmax-43, 13) | Min(Pmax’-43, 1) |
| Below 3760 MHz | -2 | -14 |

Where Pmax is the maximum mean power in dBm measured as EIRP and Pmax’ is the maximum mean power in dBm measured as TRP.

The propagation models used in the analysis are shown in the Table 7 below:

Table 7: Propagation models used in the analysis

| Model | Value |
| --- | --- |
| Propagation model | P.452 with random % of time |
| Clutter model | P.2108 assuming clutter only at one side of the propagation path |

Results of the coexistence analysis

In Figure 1 and Figure 2 below we present the separation distances between different WBB LMP BS configurations and MFCN BS in relation to the I/N protection criterion for MFCN services.

Figure 1: Non-AAS WBB LMP BS as an interferer into adjacent channel MFCN

Figure 2: AAS medium power WBB BS as an interferer into adjacent channel MFCN

From the above figures we can see that for all low and medium power WBB BS, the separation distances required to satisfy the I/N protection criterion for MFCN services are less than 500m. More specifically, regarding AAS antennas, we can see from Figure 2 that different EIRP levels have different effects in separation distances, depending on the underlying TRP and AAS configuration.

For example, considering as a reference point the case of 58.5 dBm (red line), we can see that if we maintain the same AAS configuration and we increase the TRP by 6 dB (pink line) then the required  separation distance increases. However, the joint impact of the same increase of TRP together with an increase in the AAS configuration (cyan line), results to less significant effects in the size of the required separation distances. Furthermore, by maintaining the initial TRP levels and increasing the AAS configuration (blue line), it can be seen that the resulting separation distance decreases.

The above observations in this analysis, are based on the fact that for AAS antennas, the higher the configuration (number of elements) the narrower the beams and thus, the likelihood of the AAS WBB BS pointing with peak gain towards the victim MFCN Rx, decreases.

Coexistence of WBB LMP with FSS ES in 3.8-4.2 GHz

Coexistence parameters for FSS ES

The FSS ES operate co-channel with the WBB LMP services. The selected operational parameters for FSS ES were sourced from the guidance given from WP4A (document 5D/734) and are shown in Table 8.

Table 8: FSS ES deployment characteristics

| Parameter | Value |
| --- | --- |
| Range of operating frequencies | 3800 – 4200 MHz |
| Antenna diameter | 3m |
| Antenna gain | 39.5 dBi |
| Antenna reference pattern | Recommendation ITU-R S.465-6 |
| Antenna elevation angles | 10, 15, 48 |
| Antenna height | 5m |
| I/N long-term protection criterion | -10.5 dB for 20% of time |

The propagation models used to simulate the interfering path are shown in the Table 9 below.

Table 9: Propagation models used in the analysis

| Model | Value |
| --- | --- |
| Propagation model | ITU-R P.452 with random % of time |
| Clutter model | ITU-R P.2108 assuming clutter only at one side of the propagation path |

Results of the coexistence analysis with FSS es

Separation distances for Low Power WBB BS

Figure 3: Separation distances for low power WBB BS (31dBm)

Separation distances for Medium Power non-AAS WBB BS

Figure 4: Separation distances for non-AAS medium power WBB BS (49dBm)

Figure 5: Separation distances for non-AAS medium power WBB BS (53dBm)

Separation distances for Medium Power AAS WBB BS

Figure 6: Separation distances for AAS medium power WBB BS (58.5 dBm, 4x8)

Figure 7: Separation distances for AAS medium power WBB BS (61.5 dBm, 8x8)

Figure 8: Separation distances for AAS medium power WBB BS (64.5 dBm, 4x8)

Figure 9: Separation distances for AAS medium power WBB BS (67.5 dBm, 8x8)

SUMMARY OF RESULTS FROM THE COEXISTENCE ANALYSIS

Summary of separation distances for MFCN and FSS ES

The results of the analysis presented in the above sections is summarised in Table 10 below.

Table 10: Summary of separation distances based on the analysis

| WBB BS parameter | Low power BS | Medium power BS | Note | Separation distances with MFCN | Max Separation distances with FSS ES (lowest elevation) |
| --- | --- | --- | --- | --- | --- |
| NON-AAS |  |  |  |  |  |
| Non-AAS BS EIRP (UK/Nor framework) | 31 dBm (with 0dBi antenna gain) | 49 dBm | 39 dBm output power from all connectors + 10dBi antenna gain | < 500m | < 5km for Low power < 15km for Medium power |
| Non-AAS BS EIRP (incremental) |  | 53 dBm | 43 dBm output power from all connectors + 10dBi antenna gain | < 500m | < 18km |
| AAS |  |  |  |  |  |
| AAS incremental approach |  | 37 dBm + 21.5 dBi (4x8) | EIRP = 58.5 dBm | < 500m | < 15km |
|  |  | 37 dBm + 24.5 dBi (8x8) | EIRP = 61.5 dBm | < 500m | < 13km |
|  |  | 43 dBm + 21.5 dBi (4x8) | EIRP = 64.5 dBm | < 500m | < 18km |
|  |  | 43 dBm + 24.5 dBi (8x8) | EIRP = 67.5 dBm | < 500m | < 17km |

Observations - Conclusions

As it can be seen, regarding the coexistence with adjacent band MFCN services, the separation distances for all the examined WBB LMP configurations are less than 500m.

Regarding the coexistence with in-band, co-channel FSS ES, the separation distances for the low power WBB BS are less than 5km assuming a 0 dBi omni-directional antenna for the WBB BS.

Furthermore, regarding the coexistence of medium power WBB AAS with in-band, co-channel FSS ES, the separation distances range from 13km to 18km depending on the TRP and the AAS antenna configuration. From the results, we can observe that assuming the same TRP level, an increase of the AAS configuration doesn’t overall result in higher separation distances. This is because the higher the AAS configuration, the narrower the antenna beams become and thus, the likelihood of an AAS WBB BS pointing with the peak gain of the main antenna lobe into the FSS ES receiver decreases.

Moreover, looking in total the coexistence results with FSS ES, it can be observed that when comparing the separation distances resulting from the use of similar power levels for AAS (TRP) and non-AAS (output power levels from all connectors) they are of a very similar range. In fact they are also very similar to the ones resulting from the analysis of the power levels of the existing regulatory frameworks.

Finally, the above also leads us to conclude that the use of EIRP as the parameter for defining the permitted AAS power levels, is probably not the most suitable choice of parameter, since for AAS antennas with the same TRP values, higher EIRP values don’t necessarily result to higher separation distances.

- List of References

Communication from the Commission to the European Parliament, the Council, the European Economic and Social Committee and the Committee of the Regions ‘Connectivity for a Competitive Digital Single Market - Towards a European Gigabit Society’ COM(2016) 587 final.

COM(2021) 118 final.

Communication from the Commission to the Council, the European Parliament, the European Economic and Social Committee and the Committee of the Regions ‘5G for Europe: An Action Plan’, COM(2016) 588 final.

Communication from the Commission to the European Parliament, the European Council, the Council, the European Economic and Social Committee and the Committee of the Regions ‘A New Industrial Strategy for Europe’, COM(2020) 102 final.

Document RSPG16-032 final of 9 November 2016, Strategic roadmap towards 5G for Europe: RSPG opinion on spectrum-related aspects for next-generation wireless systems (5G) (RSPG 1st opinion on 5G).

Document RSPG18-005 final of 30 January 2018, Strategic spectrum roadmap towards 5G for Europe: RSPG opinion on 5G networks (RSPG 2nd opinion on 5G).

Document RSPG19-007 final of 30 January 2019, Strategic spectrum roadmap towards 5G for Europe: RSPG opinion on 5G implementation challenges (RSPG 3rd opinion on 5G).

Document RSPG21-033 final of 16 June 2021, RSPG Opinion on a Radio Spectrum Policy Programme  (RSPP).

Document RSPG21-024 final of 16 June 2021, RSPG opinion on additional spectrum needs and guidance on the fast rollout of future wireless broadband networks.

Document RSPG21-022 final of 16 June 2021, RSPG opinion on spectrum sharing – pioneer initiatives and bands.

European Telecommunications Standardisation Institute

https://5g-ppp.eu/wp-content/uploads/2020/09/5GPPP-VerticalsWhitePaper-2020-Final.pdf

Wireless local-area connectivity could serve both private (e.g. enterprise) and public (e.g. community-type) networks, which could be subject to an authorisation decision at Member State level.

Decision 676/2002/EC of the European Parliament and of the Council of 7 March 2002 on a regulatory framework for radio spectrum policy in the European Community, OJL 108 of 24.4.2002.

Such as on Licensed Shared Access.

In particular as a primary (pioneer) 5G frequency band in the European Union.

Subject to subsequent public consultation

RRH : Remote Radio Head

Page 2, Section 2 of EC Mandate

The OFCOM framework proposed antenna heights up to 10m. However, in this study, heights above 10m were considered to assess the impact of the antenna height on the co-channel protection distances

Document 5A/395, available at https://www.itu.int/md/meetingdoc.asp?lang=en&parent=R19-WP5A-C-0395https://www.itu.int/md/meetingdoc.asp?lang=en&parent=R19-WP5A-C-0395

WBS: Wireless Broadband Systems

TS 138 104 - V15.14.0 - 5G; NR; Base Station (BS) radio transmission and reception (3GPP TS 38.104 version 15.14.0 Release 15) (etsi.org)

https://docbox.etsi.org/msg/tfes/70-Draft/TFES15-24/MSG-TFES-15-24v1511_0032.docx

ETSI, TS 138 104 V17.7.0: Base Station (BS) radio transmission and reception, Table 9.3.1-1.

Ofcom, Shared Access Licence, 2022

ETSI, TS 138 104 V17.7.0: Base Station (BS) radio transmission and reception, Table 6.2.1-1.

ECC PT1, ECC PT1_CG4G(22)011_Annex 1:	Analysis of the Norwegian assumptions

Ofcom, UK Spectrum Co-ordination, 2008

ITU-R WP5D, document 5D/716

By non-AAS output power we imply the sum of the conducted power from all antenna connectors and by AAS output power we imply the TRP

ETSI TS 138 104 - V15.14.0 - 5G; NR; Base Station (BS) radio transmission and reception (3GPP TS 38.104 version 15.14.0 Release 15) (etsi.org)

ECC Decision 11(06)