Guidance on Cross border coordination between MFCN and ATS in the 1429-1518 MHz band

ECC Report <No>

approved DD Month YYYY (Arial 9)

[last updated: DD Month YYYY] (Arial 9)

# Executive summary

At WRC-15, the frequency bands 1427-1452 MHz and 1492-1518 MHz were identified globally for International Mobile Telecommunications (IMT) in accordance with Resolution 223 (Rev.WRC-15) and ECC Decision (17)06 harmonises the use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL). The ECC Decision (17)06, together with the amendments to ECC Decision (13)03 for the frequency band 1452-1492 MHz, opens the possibility for a combined use of the whole 1427-1518 MHz frequency band for MFCN SDL. Both ECC Decisions harmonising technical conditions for the use of the frequency bands 1427-1452 MHz, 1452-1492 MHz and 1492-1518 MHz leave flexibility for administrations with regards to the use of incumbent services and applications to adapt to specific national circumstances.

In some CEPT countries, the band 1429-1518 MHz is also allocated on a co-primary basis to the aeronautical mobile service (AMS) for the purpose of aeronautical telemetry within the national territory according to No 5.342 RR. At present, the MIFR contains 1653 assignments to aeronautical telemetry systems (ATS) in the some CEPT countries within the given band. This essential part of ATS stations is used in border areas and needs protection from MFCN base stations (BSs). Both mentioned ECC Decisions refer to the need for cross-border coordination between MFCN and ATS and ECC/DEC(17)06 refer to the need for study in order to provide further guidance, where needed, to administrations for bi/multi-lateral negotiations.

All previous studies on this matter have already defined the criteria for protection of the ATS and the trigger distances to initiate coordination. But according to the results of these studies, the protection criteria and coordination distances had such values that prevented to initiate coordination negotiations. In addition, previous studies did not describe the procedure for coordination calculations taking into account the aggregate interference affect from the BSs of MFCN. The purpose of these studies is to determine a protection criterion acceptable for practical purposes, and to propose an effective mechanism for calculating and monitoring compliance with the requirements established by the coordination agreement. The studies should be completed by developing a coordination agreement template.

This report investigates two protection criteria methods - I/N and C/(I+N). In general, when the information concerning the system sensitivity is available, I/N criteria could be used as a baseline. In cases where information on the link budgets for telemetry system is available, appropriate way to define permitted levels of interference would be the C/(I+N) method.

In order to determine the practical values of the protection criterion for ATS, a comparative analysis of the characteristics of ground and on-board equipment of ATS receivers was carried out. The results of the analysis made it possible to compare the protection level of these receivers. Taking into account the requirements of footnote 5.432 RR regarding the restriction of the service area outside the national territory, the assumption was made to limit the reception of the ATS in the direction of the border only by the side and back lobes of the ATS antenna pattern.

The assumptions mentioned above requires carrying out more accurate calculations of the aggregate interference from the MFCN BSs. Due to random nature of the MFCN interference and due to the rotation of the antenna of the ATS station the measurements conducting at the locations of ATS stations could be complicated or even impossible. In order to reduce the uncertainty in measured results the proposal to transfer the calculations and measurements places from ATS positions to the test points at the borderline was considered.

Analysis was also carried also including the reception by the main beam of the ATS antenna pattern, i. e. taking into account full antenna pattern characteristics.

In some other cases the threshold level can be specified on the location of the ATS station using I/N or C/(I+N) method.

For the case of interference scenario when the ATS on-board transmitter is placed above the borderline at the height of 10000 meters and the ATS ground receiver antenna is always pointing to the aircraft the results also indicate that C/(I+N) ratio of ATS station increase when the ATS station is located closer to the borderline.

Calculations were performed for few types of MFCN topologies: single interferer, single MFNC cluster, several MFCN clusters, MFCN surrounding ATS station, MFCN linearly placed along borderline and modelled based on the real mobile Lithuanian and Poland networks.

Based on a practical interference case between POL-UKR for UMTS-BWA the methodology for aggregated interference calculation/ measurements was presented.

Possible technical measures to eliminate harmful interference from MFCN to ATS stations were considered.

The example of draft template for coordination Agreement is provided in the Annex 3.

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	6

2	Operational characteristics of the systems	7

2.1	Technical characteristics of ATS station	7

2.1.1	Radar-type antenna system	7

2.1.2	Tracking-type antenna system	7

2.2	Technical characteristics of mobile system	9

3	ATS system operation	10

3.1	Protection criteria	10

3.2	Coordination trigger	10

3.3	Receiving system block diagram	10

4	Derivation of the maximum permissible interfering field strength to ATS by method of I/N	12

4.1	Method 1. Recalculation of the power flux density to the field strength level	12

4.2	Method 2. Obtaining the permissible field strength from the typical technical characteristics of the receiver for the L-band	12

5	Principles of frequency usage in the band 1427-1518 MHz in border areas	13

6	Provisions of cross-border coordination between the MFCN and ATS stations	14

7	Methodology for field strength calculations for stations located in the border areas	15

7.1	Interference analysis for different protection methods	15

7.1.1	Aggregate interference simulations	15

7.1.1.1	Network cluster case	15

7.1.1.2	Surrounding network (ring) case	21

7.2	Example of ATS in Kaliningrad region	24

8	Methodology for field strength calculations for on-board receiver stations	31

8.1	Obtaining the permissible field strength level	31

8.2	Interference analysis for different protection methods	31

8.2.1	Aggregate interference simulations	31

8.2.1.1	Network cluster case	31

9	General procedure for field strength calculation on the borderline using I/N or C/(I+N) methods in order to eliminate harmful interference	37

10	Field strength measurement procedure	38

10.1	Common provisions	38

10.2	Requirements to the monitoring equipment	38

10.3	Information to be provided by the affected Administration in case of harmful interference	38

11	Assessment of interference from MFCN at the borderline	40

12	Conclusions	41

ANNEX 1: ATS ground station antenna patterns	42

ANNEX 2: averaged value of iRSS for surrounding network interference calculations	46

ANNEX 3: The approach to calculate the interfering power flux density	48

ANNEX 4: Additional factors for consideration in SEAMCAT simulations	50

ANNEX 5: Methodology for field strength calculations based on practical case	55

ANNEX 6: The example of draft template for coordination Agreement	60

ANNEX 7: List of Reference	61

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| ATS | Aeronautical Telemetry System |
| BS | Base Station |
| BWA | Broadband Wireless Access |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| ECC | Electronic Communications Committee |
| EIRP | Equivalent Isotropically Radiated Power |
| FDR(∆f) | Frequency-Dependent Rejection |
| FSL | Field Strength Level |
| IMT | International Mobile Telecommunications |
| iRSS | Interfering Received Signal Strength |
| MFCN | Mobile/Fixed Communications Network |
| MIFR | Master International Frequency Register |
| SDL | Supplemental Downlink |
| SRTM3 | Shuttle Radar Topography Mission |

# Introduction

The documents [2]-[5] provide examples of various ATS systems. Despite the different characteristics of the ground and on-board equipment, the destination and the main functions of the system remain the same and mainly provide the flight safety of the aircraft by obtaining current information to maintain the state of the performance of systems and crew during the test and during the operation of the aircraft. One such system is the aeronautical telemetry system in the aeronautical mobile service that has been in service for a long time and uses the band 1429-1535 MHz in accordance with RR No. 5.342. The system is intended for the current display and registration, at the ground level, of generic indicators of the state of on-board equipment and crew located in the airplane and for the transfer of these data to the ground station on request from the ground. Currently, the MIFR contains 1653 assignments to aeronautical telemetry systems (ATS) in some CEPT countries (Belarus, the Russian Federation and Ukraine) within the given band.

According to RR (2016 edition) the frequency band 1 429-1 535 MHz in Region 1 is allocated to mobile service, except aeronautical mobile, on a primary basis. According to the footnote No. 5.342 the frequency band 1 429-1 535 MHz is additionally allocated to the aeronautical mobile service on a primary basis in Armenia, Azerbaijan, Belarus, the Russian Federation, Uzbekistan, Kyrgyzstan and Ukraine, exclusively for the purposes of aeronautical telemetry within the national territory. As of 1 April 2007, the use of the frequency band 1 452-1 492 MHz is subject to agreement between the administrations concerned. According to the footnote No. 5.341A in Region 1, the frequency bands 1 427-1 452 MHz and 1 492-1 518 MHz are identified for use by administrations wishing to implement International Mobile Telecommunications (IMT) in accordance with Resolution 223 (Rev.WRC-15). This identification does not preclude the use of these frequency bands by any other application of the services to which it is allocated and does not establish priority in the Radio Regulations. The use of IMT stations is subject to agreement obtained under No. 9.21 with respect to the aeronautical mobile service used for aeronautical telemetry in accordance with No. 5.342.

A more detailed analysis of the operational and technical characteristics of the ATS station, obtained theoretically and practically, will allow to clarify protection criteria, introduce certain restrictions on the protection of ATS and suggest a possible way for efficient use of the band 1427-1518 MHz, along with provision of protection of ATS stations from the MFCN stations, located in the border area.

# Operational characteristics of the systems

## Technical characteristics of ATS station

### Radar-type antenna system

A distinctive feature of the system is the use of pulse-code signals, which allows in some cases to partially combine the functionality of radar and ATS. Therefore, ATS stations can have as a separate functional implementation and be an additional component of radar equipment. In particular, the reflector-type radar antenna and separate components of the radar feeder system are used both by the radar and by the ATS station. The main operating mode of such system is receiving the data from the tested aircraft by synchronised rotation with radar. In this case antenna pattern is according to Recommendation ITU-R M.1851-0 [1]. Further details are provided in Annex 1.

### Tracking-type antenna system

Another technical implementation of the ground station ATS works only on receiving data from the tested aircraft by slow tracking the movement of the aircraft. In this case antenna pattern is according to the Recommendation ITU-R M.1459-0 [2], see Annex 1 for further details.

Therefore, when determining the protection criterion, this could be taken into account.

According to the RR footnote No. 5.342, the service area of ATS stations is limited to the national territory. Since the service area limit is determined by the maximum radius of the system, which in turn is limited by the power budget of the link from the aircraft to the ground level, the possibility of data receiving and transmitting on the aircraft is determined by the flight path of the aircraft within the national territory. Thus, in the course of research, the scenario of the interference from the IMT BS should take into account the effect of interference towards both the main and side lobes of the antenna pattern of the ATS station located in the border area.

Table 1: Telemetry system characteristics

| Parameter | Value |
| --- | --- |
| Station of aeronautical telemetry system |  |
| Application | display and registration, at ground level, of parameters of the status of on-board equipment and crew, located in the airplane, by transmitting them to the ground station of the aeronautical telemetry system of aeronautical mobile service |
| Characteristics of transmitter |  |
| Station name | ATS on-board transmitter |
| Place of deployment | aircraft |
| Maximum EIRP, dBW | 38 |
| Average power, dBW | 5 |
| Pulse duration, µS | 0.5 |
| Occupied bandwidth of emission, MHz | 1/3/5 |
| Class of emission | VХD |
| Operation frequencies, MHz | 1434, 1439, 1444, 1449, 1454, 1459, 1464, 1469, 1474, 1479, 1484, 1489, 1494, 1499, 1504, 1509, 1514, 1519, 1524, 1529, 1533 |
| Maximum antenna height, m | 10000 |
| Maximum antenna gain, dB | 6 |
| Antenna pattern on -3 dB level, º | Nondirectional or low directional |
| Main lobe direction | Low hemisphere |
| Transmission path length, km | up to 320 |
| Characteristics of receiver |  |
| Station name | ATS ground (terrestrial) receiver |
| Station class in accordance with Chapter IV Section 6 of Preface to BR IFIC | FA |
| Place of deployment | land |
| Antenna height, m | 10 |
| Polarization | linear |
| Thermal noise floor (kTB), dBW/5 MHz | −136.98 |
| Noise figure, dB | 7 |
| Maximum antenna gain, dB | 30 |
| Feeder losses, dB | 3 |
| Antenna pattern on -3 dB level (average), º | Vertical pattern: 10º; Horizon pattern: 4⁰ |
| Main lobe direction | Azimuth: 0-360⁰ |
| C/N ratio, dB | 13 |

Protection criterion for the ground ATS station proposed by ATS users based on measurements is provided in Table 2 for information.

Table 2: Protection criterion for the ground ATS station proposed by ATS users

| Parameter | Value |
| --- | --- |
| Field strength for protection, (dBµV/m) | 14 |
| Percentage of time, (%) | 10 |
| Frequency-Dependent Rejection FDR(∆f) | Table 3 |
| Reference bandwidth, MHz | 5 |

Table 3:  Frequency-Dependent Rejection FDR(∆f) value of the ground ATS station

| Parameter | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ∆f–IMT-ATS central frequency difference, (МHz) | -15 | -10 | -8 | -4 | -2 | 0 | 2 | 4 | 8 | 10 | 15 |
| FDR(∆f), (dB) | -62 | -47 | -20 | -3 | -1 | 0 | -1 | -3 | -20 | -47 | -62 |

## Technical characteristics of mobile system

The characteristics of mobile service systems are according to Report ITU-R M.2292-0 and are extracted in Table 4. For the analysis in this document the channel bandwidth of 5 MHz will be considered, however other channel bandwidth values also may be under consideration.

Table 4: Parameters of macro base station

| Base station characteristics |  |
| --- | --- |
| Cell radius | 4.75 km |
| Antenna height | 30 m |
| Sectorization | 3 sectors |
| Downtilt | 3 degrees |
| Frequency reuse | 1 |
| Antenna pattern | Recommendation ITU-R F.1336 (recommends 3.1) ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336 |
| Antenna polarization | Linear / ±45 degrees |
| Feeder loss | 3 dB |
| Channel bandwidth | 5 MHz |
| Maximum base station output power (5 MHz) | 43 dBm |
| Maximum base station antenna gain | 18 dBi |
| Maximum base station output power/sector (EIRP) | 58 dBm |

# ATS system operation

## Protection criteria

The maximum permitted interference power level at the receiver input may be specified according to one of the two criteria:

“I/N” criterion – where the maximum permitted interference is defined in relation to the thermal noise level (I is interfering signal and N is system thermal noise power);

“C/(I+N)” criterion – where the value of the maximum permitted interference is defined in relation to a target reduction in the receiver’s signal-to-interference-plus-noise ratio (C is wanted signal, I is interfering signal, N is thermal noise). This criterion might be appropriate if the receiver operates at some margin above its minimum sensitivity and in cases where information on the link budgets is available.

## Coordination trigger

The main goal of international cross-border coordination is to allow each of the countries a mutual and optimal use of the radio spectrum. Different administrations may wish to adopt different approaches to cross-border coordination. In order to calculate the effect of harmful interference a parameter for permissible harmful interference impact on the receiving station needs to be determined. Coordination triggers could be defined as a) maximum permitted field strength levels (land mobile and broadcasting services), b) coordination distances (broadcasting and fixed services, radars), c) coordination area (broadcasting and space services), triggering power flux density (fixed service). This document will analyse the field strength level method.

The required maximum permitted field strength level for the protection of ATS stations from interference could be calculated in one of two following ways:

at the location of ATS station. This should be seen as the primary method since the main objective is to protect the station in the place where it is located. The information on the location of these stations can be obtained from the Master International Frequency Register (MIFR) or from the administrations concerned;

at the borderline. This should be agreed between neighbouring countries concerned. In such situation the requirements for the protection of ATS ground stations should not impose higher levels comparing to the case of known location. This provides implementation flexibility for the country using ATS without creating additional constraints to the mobile service in the neighbouring countries.

## Receiving system block diagram

The receiving system of the ATS is characterized by a narrow beam directional antenna, therefore a concept of isotropic received signal level (IRL) at the antenna input is not applicable for path analysis in the case of multiple and multidirectional radiation sources. Instead, C/N is calculated at the input of the first active stage of receiving system, usually this is the low-noise amplifier (LNA). Figure 1 presents a simplified functional block diagram of such a receiving system. The received signal level (RSL) or received field strength level (FSL) and the effective noise temperature (noise power N) are calculated at the same reference point at the input of the LNA.

Figure 1: Simplified block diagram of ATS ground receiver

If the characteristics of antenna of the station seeking protection is known then the protection requirement could be calculated at the reference point taking into account antenna pattern.

# Derivation of the maximum permissible interfering field strength to ATS by method of I/N

The protection criterion for protecting the ground component of the ATS from the broadcasting-satellite service (BSS) and mobile satellite service (MSS) provided in the document [2] was automatically transposed to study the protection from the land mobile service (IMT) [4], while the theoretical and practical results for obtaining a protection criterion for the last case were not obtained separately. Comparison of the technical characteristics of on-board and ground ATS receivers indicates their similarity and therefore the basis for obtaining a protective criterion may be taken for the power flux density (pfd) for the on-board receiver (S = −140 dB(W/m2) in the reference bandwidth of 4 kHz). Then the protection criterion can be obtained in two ways.

## Method 1. Recalculation of the power flux density to the field strength level

Field strength level at a given isotropic-radiative power (Recommendation ITU-R P.525-2):

E [dBμV / m] = S + 145.8 = −140 + 145.8 = 5.8 dB (μV / m) / 4 kHz = 36.8 dB (μV / m) / 5 MHz

The ATS antenna gain depends on the antenna pattern used. Taking into account that the reception of interference for the ground receiver (g) occurs on the side lobes of the antenna pattern, for example, at the average side lobes level -20 dB from the gain in the main lobe, then Eg = 26.8 dB (μV / m) / 5 MHz.

In case of reception of interference for the on-board receiver (b) we apply an antenna gain of 6 dB. Then, Eb = 30.8 dB (μV / m) / 5 MHz.

## Method 2. Obtaining the permissible field strength from the typical technical characteristics of the receiver for the L-band

Taking into account the total estimated effect of interference from other services, the receiver's sensitivity threshold may be adopted to determine the permissible field strength of interference at the ATS reference point. The receiver's sensitivity threshold for typical parameters in this frequency range is:

Pr [dBW] = 10 log10 (kTBF× (I / N)) = −228.59 dB (k) + 24.62 dB (T) + 7 dB (F) + 66.99 dB (B) − 6 dB (I / N) = −135.98 dBW = −105.98 dBm,

where k = 1.381x10-23 J / deg - Boltzmann constant, T = 290 Kelvin degrees - ambient temperature, B - noise bandwidth of 5 MHz, F - noise factor = 5, I / N = −6 dB is the interference to noise ratio (interference criterion) at 1 dB sensitivity degradation, which takes into account the estimated effect of total noise from other services.

When recalculating the sensitivity threshold value to the field strength at the reference point by the formula from Recommendation ITU-R P.525-2 for the average frequency of 1482 MHz and for isotropically received power involving no feeder loss, we have the following:

E [dBμV / m] = Pr [dBm] + 20 log F [MHz] + 77.2 = −105.98 + 63.42 + 77.2 = 34.6 dBμV / m / 5 MHz

Taking into account the average losses in the feeder (3 dB), the antenna gain for receiving on the ground (10 dB), the antenna gain for receiving on aircraft (6 dB) and PR (Δf = 0), we obtain the following values of the maximum permissible field strength for the co-channel interference respectively:

Eg = 27.6 dBμV / m / 5 MHz

Eb = 31.6 dBμV / m / 5 MHz

Thus, the calculations for the field strength levels by these two methods, for on-board and ground ATS receivers described above, convey to rather similar results.

# Principles of frequency usage in the band 1427-1518 MHz in border areas

Several regulatory documents regulate the use of the band 1427-1518 MHz. Regarding ATS stations, the documents [6] and [7] provide for the development of guidance on cross-border coordination between MFCN and ATS in order to conclude bilateral or multilateral agreements between affected administrations. The defined protection criteria for ATS stations [2]-[5] and the relevant methods of calculations are preventing the conclusion of such agreements. It becomes impossible to plan and deploy MFCN BSs due to the huge separation distances and incomplete techniques for summing up of individual BSs interference.

However from the analysis of the provided operational and technical characteristics of the ATS station, it follows that part of the frequency band 1427-1518 MHz can be used by existing ground ATS receivers with protection, which is determined by the average level of the side lobes of the ATS station antenna pattern.

Such protection of the ATS receivers could be achieved by modifying technical parameters of individual BSs under the already existing MFCN deployment, or initial planning of the MFCN with predetermined limited parameters.

At the same time, the harmonized use of the entire band 1427-1518 MHz provides an opportunity for equitable access to the spectrum in border areas for countries with different frequency bands allocations, as is the case of countries that use the ATS in aeronautical mobile service in accordance with footnote 5.342 of the Radio Regulations.

All affected countries should take all necessary measures to ensure equal access to spectrum on both sides of the border, in the bands under discussion (1429-1452, 1452-1492, 1492-1518 MHz) and to avoid producing harmful interference while providing possibility to use the frequency bands by other countries without unnecessary restrictions.

All concerned parties have to conduct a work to minimize possible harmful interference. Mutual coordination process includes calculation procedures, measurement campaigns, correspondence, etc. This work is carried out with equal commitment by all parties involved.

# Provisions of cross-border coordination between the MFCN and ATS stations

The cross-border coordination process includes a number of theoretical and practical aspects. For the case of MFCN-ATS, the following coordination provisions may be recommended:

Definition of communication systems and stations involved in coordination:

service name and communication system;

status (active, new, planned, notified …) of frequency assignments to the stations;

frequency bands in which coordination is carried out;

frequencies usage conditions (SDL, ground, on-board, duty cycle etc.) and technical specifications, if needed;

current regulatory documents relevant to the situation of systems interactions.

Recommended threshold level for coordination: as follows from Section 4, the permissible level of the field strength (threshold), produced by MFCN BSs is determined by the parameters of the BSs MFCN transmitters and ATS receivers and can be transferred to the borderline (see Section 8) due to the uncertainty of the direction of reception and duty mode of the ATS receiver. As a baseline, when I/N method is used, it was proposed as an example to use the threshold value of 30 dBμV/m / 5 MHz / 10 m which takes into account the average level of side lobes of ATS antenna pattern. If the permissible field strength value is transferred to the borderline this value will be recalculated in accordance with the distance and position of the ATS station from the border. In some other cases the threshold level can be specified with using the location of the ATS station or using C/(I+N) method.

Concerning the new ATS stations to be put into operation, it is proposed to apply the criterion of the secondary cross-border line located on a distance of 15 km from the borderline. All new ATS stations, which are planned to be located closer than 15 km from the border, should be coordinated with the neighbouring administration. Such a distance is recognized as critical in terms of requiring excessive protection for ATS receivers under the conditions of the operating BSs of the MFCN in a neighbouring country.

The main provisions of coordination, to be inserted in a bilateral/multilateral agreement, should include: a description (or reference) of the format of the request for interference; the actions of concerned parties in the case of interference issues, including the procedure for measuring; method of calculations, including calculation of aggregate interference; the parties' responsibility for the results of calculations and measurements.

# Methodology for field strength calculations for stations located in the border areas

## Interference analysis for different protection methods

The following analysis investigates interference from MFCN network to ATS ground station based on protection criterion by methods I/N and C/(I+N) (see Section 3.1).

### Aggregate interference simulations

Due to variety of border shapes two possible borderline cases will be analysed: straight line type (see Figure 2 and Figure 10) and a ring type (see Figure 15). For the first case (straight line) the ATS system (ground station and aircraft station) will be placed either perpendicular (scenario 1) (Figure 2) or parallel (scenario 2) (Figure 10) to the borderline. These two scenarios will try to assess the interference from MFCN into main lobe and side lobes of the ATS antenna pattern. Both radar-type and tracking-type antenna systems will be considered (i.e. antenna pattern according to Recommendation ITU-R M.1459-0 and Recommendation ITU-R M.1851-0).

#### Network cluster case

Scenario 1

In this case the situation when all systems (mobile network base stations, ATS receiver and ATS transmitter) are in the same line is considered.

Simulations for 1 cell or BS (3 sectors) as interferer and 19 cells or BSs (57 sectors) as interferers (the maximum number of cells or BSs in SEAMCAT cluster, version 5.3.0) are performed. Since the cell radius is 4.75 km then the radius of such network will be 38 km, consequently, this would cover an area of 4536 km2. A system layout is provided in Figure 2.

Figure 2: Aggregate interference from MFCN cluster (scenario 1)

The ATS on-board transmitter is placed above the borderline at the height of 10000 m and 2000 m. Antenna is omnidirectional with 6 dBi antenna gain and is always pointing to the ATS ground receiver. It is possible that aircraft manoeuvres can result in fading of received signal, but this could be solved by employing multiple antennas hereby providing antenna diversity (see paragraph 1.4 of Annex 2 of Recommendation ITU-R M.1459-0). The ATS ground receiver antenna is always pointing to the aircraft. The ATS ground receiver antenna height is 10 m. The nearest base station (reference cell) is 9.5 km from the borderline (i.e. two cell radii) and one of its three sectors is looking directly to ATS which corresponds to the worst case. The BS antenna height is 30 m, antenna pattern is in accordance with Recommendation ITU-R F.1336 and the antenna downtilt angle is set to 3 degrees. A flat-earth terrain profile with land path is considered.

Firstly, we analyse the ATS ground receiver antenna gain of Recommendation ITU-R M.1459 (maximum antenna gain of 29 dBi) towards base station dependence on the distance.

Figure 3: ATS ground receiver antenna elevation angle towards aircraft and gain towards BS versus the distance to the borderline

From Figure 3 when aircraft altitude is 10000 m, it can be noted that for the distances up to 100 km from the borderline the interference at the ATS ground receiver is received mainly by the side lobe of antenna pattern (with corresponding antenna gain). At the distance of approximately 120 km from the border antenna gain reaches 22 dBi and only from 240 km antenna gain exceeds 27 dBi.

It is also worth to note the difference between line-of-sight (LOS) wave propagation versus radio horizon (RH) for different aircraft altitudes. Assuming a perfect sphere with no terrain irregularity, the distance to the horizon from an on-board transmitter (i.e., line of sight) to the ground receiver can be calculated as follow

d≈3.57 h1+h2	(1)

where h is antenna height above ground level in meters and distance d is in kilometres.

Considering the effect of atmosphere on the propagation path of radio waves the maximum service range is increased and is known as radio horizon which is calculated as

d≈4.12 h1+h2	(2)

The Table 5 provides the comparison of LOS and RH for different antenna heights.

Table 5: LOS and RH distance of radio wave propagation

| h1_agl, m | h2_agl, m | LOS, km | RH, km |
| --- | --- | --- | --- |
| 10 000 | 10 | 357.18 | 412.21 |
|  | 30 | 357.54 | 412.62 |
|  | 50 | 357.89 | 413.03 |
| 2000 | 10 | 160.05 | 184.71 |
|  | 30 | 160.85 | 185.63 |
|  | 50 | 161.64 | 186.54 |

The results of interference from clustered network into ATS ground station using antenna pattern in accordance with Recommendation ITU-R M.1459 for I/N and C/(I+N) calculation methods are provided in Figure 4, Figure 5 and Figure 6. The propagation model for the interfering Received Signal Strength (iRSS) path from MFCN BSs to ATS ground station is in accordance with Recommendation ITU-R P.1546-5 and Recommendation ITU-R P.525-2 is used for wanted signal path from ATS on-board transmitter. The use of other propagation models (e.g. in accordance with Recommendation ITU-R P.526-14 [8] or Recommendation ITU-R P.528-3 [9]) could be studied separately. Some analysis is provided in Annex 4.

Figure 4: iRSS dependence on distance between ATS and MFCN (ATS antenna acc. to ITU-R M.1459)

Figure 5: C/(I+N) dependence on distance between ATS and MFCN (ATS antenna acc. to ITU-R M.1459)

According to parameters provided, the transmission path length of the ATS system is up to 320 kilometres and the required C/N = 13 dB. From Figure 5, we can see that for the aircraft flying at a distance of 320 km from the receiver and operating with the maximum EIRP, the C/(I+N) is over 45 dB for the interferer of 1 base station and over 40 dB for the interfering network of 19 base stations. It means that there is a margin of 27-32 dB accordingly.

The results for the analysis based on I/N method are provided in the Figure 6.

Figure 6: I/N dependence on distance between ATS and MFCN (ATS antenna acc. to ITU-R M.1459)

The results of interference from clustered network into ATS ground station using antenna pattern in accordance with Recommendation ITU-R M.1851 for the same calculation methods are provided in Figure 7, Figure 8 and Figure 9. The horizontal antenna pattern could be two types (more information on antenna pattern is provided in Annex 2), but the results are presented only for sin antenna distribution pattern type for horizontal antenna pattern as the difference using cos antenna distribution pattern type in the direction of main lobe is very small. This is because almost all interference is received by the main lobe of ATS ground station.

Figure 7: iRSS dependence on distance between ATS and MFCN (ATS antenna acc. to ITU-R M.1851)

Figure 8: C/(I+N) dependence on distance between ATS and MFCN (ATS antenna acc. to ITU-R M.1851)

Figure 9: I/N dependence on distance between ATS and MFCN (ATS antenna acc. to ITU-R M.1851)

The results indicate that protection requirement I/N = −6 dB is not exceeded already at the distance around 15 km from the borderline when ATS ground receiver antenna height is 10 m.

Scenario 2

In this case the situation when ATS aircraft is flying alongside borderline is investigated. Additional five network clusters, as in scenario 1, are placed along the borderline. The interference from each of the network is calculated at ATS ground station location. ATS ground station distance from the borderline is 10 km, antenna height is 10 m, ATS aircraft station is 320 km from ATS ground station alongside the borderline, aircraft altitude is 10000 m.

Figure 10: Aggregate interference from MFCN clusters (scenario 2)

The results from each clustered network and aggregated from all networks are provided in Table 6 for both type of ATS systems (radar-type antenna and tracking-type antenna).

Table 6: Interference levels for scenario 2

| iRSS, dBm | Antenna pattern |  |  |
| --- | --- | --- | --- |
|  | M.1459 | M.1851_cos | M.1851_sinc |
| iRSS1 | −105.64 dBm | −118.13 dBm | −98.13 dBm |
| iRSS2 | −114.71 dBm | −138.18 dBm | −118.17 dBm |
| iRSS3 | −117.08 dBm | −154.31 dBm | −135.02 dBm |
| iRSS4 | −120.79 dBm | −158.74 dBm | −142.79 dBm |
| iRSS5 | −123.23 dBm | −161.17 dBm | −148.34 dBm |
| iRSS6 | −126.22 dBm | −161.42 dBm | −153.10 dBm |
| iRSS total | −104.66 dBm | −118.09 dBm | −98.09 dBm |

It could be noted that for this type of scenario the highest interference is from the first network which is placed closest to the ATS. The results also indicate that protection requirement of I/N = −6 dB for ATS with antenna ITU-R M.1459 is exceeded only by 0.36 dB and for the ITU-R M.1851_sinc antenna type by 7.87 dB. But for the ITU-R M.1851_cos antenna type iRSS is below the protection requirement by 12.3 dB.

#### Surrounding network (ring) case

In addition to the above analysis there could be some cases when mobile networks could possibly surround ATS ground station therefore the ATS receiver could experience the interference not only from main lobe and side lobes but also from back lobes.

The previous calculations have been based on 2-tiers tri-sector 3GPP layout with the cell radius of a 4.75 km. The geographical separation of the victim receiver and interferer LTE network was modelled by specifying the modelled cellular cell as centred “infinite” network. This is illustrated in the Figure 11, where DE is ATS Rx distance from the network edge, DC is distance from hexagonal cluster centre, RC is cluster maximum range, and α is central (network visibility) angle. The ATS aircraft is considered to be above the edge of the network, i.e. at DE, this corresponds to the borderline.

ATS Rx

LTE-C

DE

DC

RC

α

a

Figure 11: Layout used in SEAMCAT simulation

It is obvious that in case of any surrounding network, interfering Received Signal Strength (iRSS) will depend on the BS arrangement inside the LTE cluster (especially in near distances, DE, from the cluster), therefore, and for getting the more even distribution of interfering BS over the cluster area, the LTE-C cluster was rotated around its centre and iRSS was calculated every 5 degrees. It is equivalent to the ATS antenna rotation around the cluster keeping the unchanging distance and antenna pointing direction to the cluster centre. A set of simulations were done using ATS Rx antenna (in accordance with Recommendations ITU-R M.1459 and M.1851) and aggregated iRSS were calculated. Then the averaged value of iRSS were evaluated for each antenna. It depends on the distance DE and aircraft altitude. The results of such simulations are presented in Annex 2.

Knowing the iRSS average from the LTE cluster we can estimate the interference of any surrounding network. This can be achieved by introducing the interfering angular power flux density of interfering network which directly follows from the calculated by means of SEAMCAT average value of the iRSS (see Annex 3).

ATS Rx

LTE-C

DE

DC

RC

αeff

reff

Seff

ATS Rx

eff/2

DC

2

reff

S()

1

d ()

rariri

Thus, having defined the power flux density value , the whole interfering signal strength iRSS produced by a ring-shaped network of the width of 2RC in the range of  varying from 0 to 360 degrees with one degree angle increment (Figure 12) can be calculated using equation

(3)

where G(φi,θi) denotes the linear antenna gain factor, and φi and θi are antenna pattern azimuth and elevation angles, respectively.

ATS Rx

DC

2RC

φ

LTE

DE

Figure 12: Effective ring-shaped LTE network

The results of interference from surrounding network when ATS ground station uses antenna patterns according to Recommendation ITU-R M.1459 (tracking-type antenna system) and Recommendation ITU-R M.1851 (radar-type antenna system) are provided in Table 7 when ATS ground receiver antenna height is 10 m and MFCN BS antenna height is 30 m. Three radii DE (10, 40 and 80 km) of surrounding networks are studied. This corresponds to the area of 314 km2, 5026 km2, 20106 km2 and could be compared to the territory of Riga city, twice as country of Luxembourg and Slovenia respectively, surrounded by MFCN.

Table 7: Interference from surrounding network

| Antenna pattern | h, m | DE, km | ±φfront, deg | iRSSfront, dBm | iRSSback, dBm | Diff1, dB | iRSSring, dBm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| tracking-type antenna system (M.1459) | 2000 | 10 | 46 | −95.69 | −101.21 | 5.51 | −94.62 |
|  |  | 40 | 47 | −90.95 | −108.62 | 17.67 | −90.88 |
|  |  | 80 | 47 | −93.81 | −113.76 | 19.95 | −93.77 |
|  | 10000 | 10 | 19 | −109.25 | −100.57 | −8.67 | −100.02 |
|  |  | 40 | 46 | −104.90 | −108.93 | 4.02 | −103.46 |
|  |  | 80 | 47 | −105.82 | −113.78 | 7.96 | −105.18 |
| radar-type antenna system (M.1851_cos) | − | 10 | 22 | −108.00 | −112.22 | 4.22 | −106.60 |
|  |  | 40 | 22 | −117.84 | −121.88 | 4.05 | −116.39 |
|  |  | 80 | 22 | −123.05 | −127.00 | 3.95 | −121.58 |
| radar-type antenna system (M.1851_sinc) | − | 10 | 29 | −107.33 | −101.16 | −6.17 | −100.22 |
|  |  | 40 | 29 | −115.57 | −109.35 | −6.22 | −108.42 |
|  |  | 80 | 29 | −119.18 | −112.93 | −6.24 | −112.01 |
| radar-type antenna system (M.1851_cos), ant_downtilt 2 deg | − | 10 | 23 | −91.20 | −110.68 | 19.48 | −91.15 |
|  |  | 40 | 23 | −100.66 | −120.10 | 19.45 | −100.61 |
|  |  | 80 | 23 | −105.91 | −125.34 | 19.43 | −105.87 |
| radar-type antenna system (M.1851_sinc) ant_downtilt 2 deg | − | 10 | 29 | −90.94 | −92.59 | 1.66 | −88.68 |
|  |  | 40 | 29 | −100.04 | −101.68 | 1.63 | −97.77 |
|  |  | 80 | 29 | −105.01 | −106.63 | 1.62 | −102.74 |

Where h is ATS aircraft altitude, φfront is antenna pattern angle covering main lobe and side lobes, iRSSfront is received interference level for φfront, iRSSback is received interference level for the rest of antenna pattern, Diff1 is the difference between interference level of iRSSfront and iRSSback, iRSSring is overall interference level from ring type network.

From the Table 7 it is observed that for tracking-type antenna ATS system (i.e. ITU-R M.1459 antenna pattern) when surrounding network radius, DE, is 10 km and altitude of aircraft is 10000 m and also for radar-type antenna system with sinc antenna distribution pattern type for horizontal antenna pattern the main interference is received by back lobes of ATS antenna. For all other cases in the Table 7 the main interference is received by main lobe and side lobes of ATS antenna and the interference from back lobes is lower from 4 dB up to 20 dB. For the radar-type antenna system antenna if antenna is downtilted by 2 degrees the interference will increase.

The comparison of ring-type network results with the clustered network case (scenario 1) is provided in Table 8.

Since for the clustered network case simulations were performed when the nearest base station (reference cell) one of three sectors was looking directly to ATS, therefore to compare the results with ring type network we need to look at the averaged value of iRSS for clustered network at these three radii DE under investigation. The values for iRSSring, iRSSfront, iRSSback and Diff1 from Table 7 are also included in Table 8.

Table 8: Comparison of clustered network and ring network interference

| ATS type | h, m | DE, km | iRSSclust_max, dBm | iRSSclust_av, dBm | iRSSring, dBm | Diff2, dB | iRSSfront, dBm | iRSSback, dBm | Diff1, dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tracking-type antenna system (M.1459) | 2000 | 10 | −91.98 | −96.13 | −94.62 | 1.51 | −95.69 | −101.21 | 5.52 |
|  |  | 40 | −86.29 | −91.05 | −90.88 | 0.17 | −90.95 | −108.62 | 17.67 |
|  |  | 80 | −91.13 | −93.94 | −93.77 | 0.17 | −93.81 | −113.76 | 19.95 |
|  | 10000 | 10 | −105.07 | −107.47 | −100.02 | 7.45 | −109.25 | −100.58 | −8.67 |
|  |  | 40 | −104.39 | −106.13 | −103.46 | 2.67 | −104.91 | −108.93 | 4.02 |
|  |  | 80 | −105.29 | −107.04 | −105.18 | 1.86 | −105.82 | −113.78 | 7.96 |
| radar-type antenna system (M.1851_cos) | − | 10 | −97.75 | −107.95 | −106.60 | 1.35 | −108.00 | −112.22 | 4.22 |
|  |  | 40 | −112.11 | −117.95 | −116.39 | 1.56 | −117.84 | −121.88 | 4.04 |
|  |  | 80 | −119.38 | −123.36 | −121.58 | 1.78 | −123.05 | −127.00 | 3.95 |
| radar-type antenna system (M.1851_sinc) | − | 10 | −97.72 | −107.29 | −100.22 | 7.07 | −108.37 | −100.94 | −7.43 |
|  |  | 40 | −111.97 | −117.44 | −108.42 | 9.02 | −116.62 | −109.13 | −7.49 |
|  |  | 80 | −119.16 | −122.97 | −112.01 | 10.96 | −120.24 | −112.72 | −7.52 |
| radar-type antenna system (M.1851_cos), ant_downtilt_2 deg | − | 10 | −81.14 | −91.21 | −91.15 | 0.06 | −91.20 | −110.68 | 19.48 |
|  |  | 40 | −94.83 | −100.68 | −100.61 | 0.07 | −100.66 | −120.10 | 19.45 |
|  |  | 80 | −101.95 | −105.96 | −105.87 | 0.09 | −105.91 | −125.34 | 19.43 |
| radar-type antenna system (M.1851_sinc), ant_downtilt_2 deg | − | 10 | −81.14 | −90.97 | −88.68 | 2.29 | −90.94 | −92.59 | 1.66 |
|  |  | 40 | −94.77 | −100.40 | −97.77 | 2.63 | −100.04 | −101.68 | 1.63 |
|  |  | 80 | −101.81 | −105.69 | −102.74 | 2.95 | −105.01 | −106.63 | 1.62 |

Where iRSSclust_max is maximum interference level from clustered network, iRSSclust_av is average interference level from clustered network, Diff2 is the difference between iRSSclust_av and iRSSring.

From the Table 8 we can see that the interference from surrounding network is higher by 0.17-10.96 dB. It should be highlighted that at the same time the protection requirement of I/N = −6 dB for radar-type antenna systems (both antenna distribution pattern types for horizontal antenna pattern) is not exceeded.

## Example of ATS in Kaliningrad region

For the example presented below, an assumption was made that SDL in the frequency band 1427-1518 MHz will be deployed in the same sites of the base stations in the 800 MHz frequency band. This deterministic analysis is based on real mobile network deployment in the 800 MHz frequency band in the Republic of Lithuania. 3959 sectors have been used with all parameters currently in operation, only the frequency was changed to 1482 MHz to represent the L-band situation. The nearest ATS station (located 70 km and away from the border with Lithuania), registered in MIFR is in the Kaliningrad enclave. The antenna height of 50 m is provided for this station (site name Kaliningrad, location 020°24'00" E 54°46'00" N). We will calculate the electric field strength level both on the location of the ATS ground station and on the borderline. The propagation model is according to Recommendation ITU-R P.1546-5 with the probability of 10% of time and 50% of location. SRTM3 terrain data was used.

For the aggregate interference analysis, an area with a radius of 250 km around the ATS station was investigated. The picture below shows the situation of the locations of the ATS ground station, the mobile network base stations and the points on the borderline.

Figure 13: ATS ground receiver and mobile network base stations geographical arrangement

Since the ATS ground station is rotating in the horizontal plane, the azimuths from 10 degrees to 105 degrees (with a step of 5 degrees) were studied. The ATS on-board transmitter is placed on the borderline at the height of 10000 m and 2000 m. For tracking-type ATS system the ATS ground receiver antenna is always pointing to the on-board transmitter and antenna pattern is in accordance with Recommendation ITU-R M.1459. For radar-type ATS system antenna pattern is in accordance with Recommendation ITU-R M.1851.

Taking into account the average feeder losses (3 dB) the values of the received signal level at the reference point together with the ATS noise floor and protection level are presented in  and the results of I/N and C/(I+N) are provided in Figure 14 and Figure 15.

Figure 14: I/N of ATS ground receiver’s reference point with the Lithuanian mobile network as interferer at ATS location dependences on receiver antenna azimuth

Figure 15: C/(I+N) of ATS ground receiver’s reference point with the Lithuanian mobile network as interferer at ATS location dependences on receiver antenna azimuth

According to analysis by method I/N (Figure 14) we can see that interfering signal should be reduced by more than 20 dB in order satisfy the protection requirement I/N = −6 dB.

According to analysis by method C/(I+N) (Figure 15) we can see that protection criteria is satisfied with excess from 15 dB up to 40 dB.

The electric field strength level on the borderline for an isotropic antenna of 10 m height was also calculated for each azimuth under consideration and is provided in Figure 16 where flat Earth option was also included for comparison.

Figure 16: Interfering received signal level at the ATS station for different receiver directional antenna azimuth values and at the borderline

Additionally, the received signal level on the borderline from the Lithuanian mobile network as provided in the Figure 16 and interference levels at ATS ground station from  are converted to field strength level and are provided in the Figure 17.

Figure 17: Field strength level at ATS location from the Lithuanian mobile network

The interfering field strength level from mobile network using ATS ground station antenna according to Recommendation ITU-R M.1851 is lower by 15-20 dB than using antenna pattern according to Recommendation ITU-R M.1459. The aggregate field strength level produced by the mobile network is on average 7 dB higher comparing to 1 base station.

Moreover, in order to compare the results for the case of Lithuanian mobile network as interferer and ATS station with 50 m antenna height as a victim with the simulations in Section 7.1 the cases with different distances (from 90 km to 160 km) between systems were also investigated for tracking-type ATS system. The results are provided in Figure 19.

Figure 18: Comparison of network cluster case (scenario 1) and Lithuanian network

Analysis showed that similar tendency of C/(I+N), as for the case of multiple base stations scenario presented in Figure 2, is observed (for ATS ground station antenna height of 50 m). These results for this case, under the assumptions provided in this section, indicate that C/(I+N) ratio of ATS station increase when station is located closer to the borderline which shows that in this case there is no need to change any LTU mobile network parameters in order to protect the ATS station in Kaliningrad.

It is also possible that the neighbouring country mobile networks could have aggregate interference effect into ATS radar, therefore additionally POL mobile network was included into this analysis. POL network was also based on real 800 MHz network sites and BS parameters. For the aggregate interference analysis, an area with a radius of 200 km around the ATS station was investigated, this accounts for around 7000 sectors. The Figure below shows the overall situation. The calculation parameters in the analysis were the same as for LTU network case.

Figure 19: ATS ground receiver and multiple mobile network base stations geographical arrangement

The results are provided in Figures below.

Figure 20: Interference level from LTU and POL MFCNs at ATS location

Figure 21: Interference level from LTU and POL MFCNs at ATS location

This clearly shows that interference from multiple networks in Kaliningrad case for ground ATS station could be considered separately.

# Methodology for field strength calculations for on-board receiver stations

## Obtaining the permissible field strength level

The derivation of maximum permissible interference field strength level for on-board receiver is provided in Section 4 based on the condition that ATS radar and on-board receiver sensitivity is similar and that required I/N is equal to −6 dB. This led to the permissible field strength of around 30 dB(μV/m) / 5 MHz.

In the absence of the characteristics for the on-board receiver of the telemetry system comparative analysis with other frequency bands for radiodetermination/radionavigation systems were carried out. For example, ITU-R Recommendation M.1830 provides permissible interference field strength for aircraft responder of ground radar in the 645-862 MHz frequency band with the permissible interference field strength of 73 dB(μV/m) / 3 MHz.

From given permissible interference field strength level it is difficult to say whether system noise figure or signal to noise ratio is different from the assumptions in Section 4.

In case receiver's noise figure in 800 MHz frequency band should not be worse than in the 1.5 GHz frequency band it is thus obvious that interference to noise level is not the same. From given permissible interference field strength level and system thermal noise power we can get the I/N. Calculations show that I/N would be around 46 dB. Thus, applying I/N of −6 dB to on-board receiver in the 1427-1518 MHz frequency band would create a difference of more than 50 dB.

In case I/N is equal to −6 dB then it is possible to acquire the system noise figure.

Next, we will simulate interference from MFCN network to ATS on-board station to investigate the situation in L-band with the above-mentioned assumption for known system noise figure.

## Interference analysis for different protection methods

The following analysis investigates interference from MFCN network to ATS on-board station based on protection criterion by methods I/N and C/(I+N) (see Section 3.1).

### Aggregate interference simulations

#### Network cluster case

  - Scenario 1

In this case the situation when all systems (mobile network base stations, ATS receiver and ATS transmitter) are in the same line is considered.

Simulations with SEAMCAT for 19 cells or BSs (57 sectors) as interferer and 519 cells or BSs (285 sectors) are performed. Such networks would cover an area of 4536 km2 and 22680 km2 accordingly. A system layout is provided in Figure 2. Only in this case the ATS ground station is fixed at three different distances from the border (10 km, 40 km and 80 km) and aircraft is flying from the borderline towards ATS radar and onwards.

Figure 22: I/N at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 10 km)

Figure 23: C/(I+N) at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 10 km)

Figure 24: dRSS at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 10 km)

Figure 25: I/N at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 40 km)

Figure 26: C/(I+N) at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 40 km)

Figure 27: dRSS at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 40 km)

Figure 28: I/N at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 80 km)

Figure 29: C/(I+N) at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 80 km)

Figure 30: dRSS at on-board receiver (ATS radar antenna acc. to ITU-R M.1459, 80 km)

Results show that for tracking-type antenna system the I/N is not exceeded for aircraft altitude of 10000 m. For the aircraft altitude of 2000 m, the I/N is exceeded not more than 2 dB in the direction of borderline.

# General procedure for field strength calculation on the borderline using I/N or C/(I+N) methods in order to eliminate harmful interference

This methodology describes multistep approach how to evaluate and eliminate real interference. These steps include calculations, measurement and adjustment of mobile network parameters, if needed. An example of UKR-POL case on BWA-MFCN is provided in Annex 2.

Protection criterion by method I/N or C/(I+N) at ATS ground station location from the BSs MFCN could be recalculated at the borderline if agreed by concerned parties.

Due to random nature of MFCN network interference and rotation of ATS stations antenna the following steps could be used:

Points of calculation / monitoring of field strength level (FSL) are replaced from the location of ATS station location to the borderline. Figure 20 below shows the need to transfer the test point for the calculation / monitoring to borderline.

Figure 31: Example of MFCN interference to the ATS receiver close to the borderline

a test points are determined on the borderline according following the item A4.1;

all sectors and all frequency channels of MFCN BSs, which are covered by the characteristics of the ATS receiver and radiate in the direction of the test points are involved in calculations. To determine the MFCN BSs the methodology given in item A4.2 could be used for calculations;

aggregated FSL is calculated at each test point and compared with the permissible (threshold) values of the interference FSL at the reference point of the ATS;

based on the results of calculations, the adjustment measures to parameters for the individual MFCN BSs are determined, if needed. Examples of such measures which could be applied are listed:

switching-off the sector;

changing of the antenna tilt;

changing of the direction of maximum radiation of the sector antenna;

reduction of radiation power;

decreasing of the height of the sector antenna deployment;

changing the frequency channel.

After adjustment measures are applied the party using ATS could perform measurements.

# Field strength measurement procedure

## Common provisions

Field strength measurements can be carried out on a regular basis or in the event of harmful interference. Administrations (Regulatory Authorities) shall perform measurements in order to verify the agreed level of permissible interferences. The measurements should be performed in accordance with the procedure of measurements given in CEPT/ERC Recommendation 74-02 E (Bucharest 1999).

In case aggregated FSL of MFCN BSs exceeds the threshold level, an analysis of the highest interfering BSs sectors with their identification should be conducted.

Due to the threshold level of the MFCN BSs signal to be measured, the general measurement methodology consists of such stages:

  - Search for the maximum aggregated MFCN signal (channel power) with the directional antenna and spectrum analyser / certified measurement equipment;

Measure the aggregated level of channel power in already identified directions;

Identify the MFCN BSs sectors and measure power of the individual signal level (channel power).

Measurements of interference level are carried out at the test points on the borderline or, if measurements at the test points cannot be carried out due to practical reasons (e.g. the test point is not accessible), at a point as close as possible to the test point on the borderline.

## Requirements to the monitoring equipment

Recommended set of equipment:

hardware (HD): spectrum analyser + scanner (digital receiver + processor);

software (SW).

Recommended requirements for monitoring equipment:

sensitivity – not worse than ATS receiver sensitivity for the reference bandwidth + protection criteria;

nonconformity of measured level (with S/N ratio more than 16 dB) – not higher than 1 dB;

reference frequency stability 1×10–6 per 1 year;

possibility to identify MFCN network, individual BS and sectors of the BS.

Antenna equipment parameters:

antenna mast up to 10 m height;

calibrated cable with losses not higher than 3 dB/10 m;

directional antenna with a determined antenna factor at MFCN BSs frequencies is recommended.

Calculation of an average field strength value (Emeas av, dBµV/m), for n measurements of Еmeas і:

dBµV/m	(4).

## Information to be provided by the affected Administration in case of harmful interference

- Name of the point of measurements

Coordinates of the test point (DDNMM'SS" DDEMM'SS")

Height of the point of measurements above sea level (m)

Date of measurements

Azimuth of measurement antenna (degrees)

Polarization (degrees)

Antenna factor (dB/m)

Total attenuation (dB)

Channel width (MHz)

Central frequency (carrier) of MFCN BS sector channel (MHz)

Measured channel power (dBm)

Field strength (dBµV/m)

Information which can be useful for affecting and affected sides for investigation of the harmful interference cease:

identifier (MCC, MNC, CID, PN): MCC - country code; MNC - operator code; CID -base station identifier; PN - sector code.

# Assessment of interference from MFCN at the borderline

To estimate the possibility to recalculate the field strength level (FSL) at the borderline the data from the MIFR for frequency assignments to ATS stations of Ukraine were extracted as an example. Under the assumption that agreed between concerned parties permissible FSL is 30 dBμV/m at ATS location the FSL at the nearest point at the borderline was recalculated using Recommendation ITU-R P.1546-5 (10% time, 50% locations) and results are provided in Table 9.

Table 9: Example of some ATS stations in Ukraine and distance from them to neighbouring countries

| Site name | Location | Administration | Minimum distance to borderline, km | Maximum distance to borderline*, km | FSL recalculated to nearest point at borderline, dBμV/m / 5 MHz / 10 m |
| --- | --- | --- | --- | --- | --- |
| BALTA | 029°36'00" E - 47°56'00" N | MDA | 24 | 293 | 40.5 |
| BOLHRAD | 028°40'00" E - 45°40'00" N | MDA | 9 | 338 | 34.7 |
|  |  | ROU | 37 | 509 | 44.1 |
| CHERNIVTSI | 025°58'00" E - 48°15'00" N | ROU | 27 | 230 | 41.4 |
|  |  | MDA | 48 | 439 | 46.5 |
| LYMANSKE | 030°01'00" E - 46°40'00" N | MDA | 4 | 310 | 32.2 |
| NOVOVOLYNSK | 024°11'00" E - 50°41'00" N | POL | 7 | 212 | 34.7 |
| RAKHIV | 024°11'00" E - 48°03'00" N | ROU | 14 | 604 | 36.8 |
| STARYI SAMBIR | 022°59'00" E - 49°26'00" N | POL | 18 | 235 | 38.4 |
|  |  | SVK | 48 | 130 | 46.5 |
| UZHHOROD | 022°15'00" E - 48°38'00" N | POL | 54 | 337 | 47.6 |
|  |  | SVK | 1 | 57 | 30.4 |
|  |  | HNG | 23 | 89 | 43.1 |
| VYNOHRADIV | 023°02'00" E - 48°10'00" N | HNG | 16 | 70 | 38 |
|  |  | ROU | 11 | 604 | 35 |
| ZABOLOTNE | 024°15'00" E - 51°38'00" N | POL | 42 | 307 | 45.2 |

* Maximum distance to the last point of common borderline

# Conclusions

The refined operational and technical characteristics of ATS stations indicate the similarity of ground and on-board equipment. This allows accept the reference criterion for the protection of ground equipment, as defined in Recommendation ITU-R M.1459 for on-board receiver.

According to the RR No. 5.342 the ATS service area is restricted by the national territory only. This allows to consider the impact of MFCN BSs to the ATS station on the side and back lobes of the ATS station antenna as the main scenario. Consequently, there is an opportunity to relax the requirements for the maximum permissible interference field strength level at the antenna input of the ATS station.

Due to random nature of the MFCN interference and due to the rotation of the antenna of the ATS station the measurements conducting at the locations of ATS stations could be complicated or even impossible. In order to reduce the uncertainty in results it was proposed to transfer the calculations and measurements places from ATS positions to the test points at the borderline.

Based on a practical interference case between POL-UKR for UMTS-BWA the methodology for aggregated interference calculation/measurements was presented.

In order to provide the protection to ATS station the field strength level in some cases could be calculated at the location of ATS station.

Methods I/N or C/(I+N) could be considered for the evaluation of interference from MFCN BSs to ATS stations. In case of C/(I+N) method a proper attention should be drawn to margin of wanted signal from aircraft in order to account signal losses related with aircraft maneuverer, propagation conditions, etc.

In case of possible aggregate interference from more than one neighbouring country all concerned parties have to conduct a work to minimize possible harmful interference. In this case it is recommended that all affected countries take all necessary measures to avoid harmful interference while providing possibility to use the frequency band for other countries with less possible restrictions.

Calculations shows, that for some cases, surrounding MFCN could increase interference level to ATS station up to 4 dB and for worst case topology of the MFCN located along the borderline (5 clustered networks) would increase the interference level up to 1.2 dB.

The example for Kaliningrad using both methods was investigated together with general simulations using SEAMCAT. From the results using C/(I+N) method and input conditions provided in this report it could be concluded that ATS ground receiver in Kaliningrad is sufficiently protected from the interference of mobile network in the Republic of Lithuania.

Possible technical measures to eliminate harmful interference from MFCN to ATS stations were considered.

The example of draft template for coordination Agreement is provided in the Annex 5.

ATS ground station antenna patterns

Antenna pattern of Recommendation ITU-R M.1459

For the case when ATS ground station works only on receiving data from the tested aircraft by slow tracking the movement of the aircraft as mentioned in Section 2.1 it was proposed to use antenna pattern according to Recommendation ITU-R M.1459-0.

The ATS ground receiver antenna pattern of Recommendation ITU-R M.1459 (Annex 1, paragraph 1, antenna gain 29 dBi, equations 1a-1f) is provided in Figure 32, Figure 33 and Figure 34.

Figure 32: ATS ground receiver vertical antenna pattern

Figure 33: ATS ground receiver horizontal antenna pattern

Figure 34: ATS ground receiver 3D antenna pattern

Antenna pattern of Recommendation ITU-R M.1851

For the case when ATS stations is an additional component of radar equipment as mentioned in Section 2.1 antenna pattern according to Recommendation ITU-R M.1851-0 is used. This is antenna with constant height pattern or also called cosecant squared pattern. Such antenna patter permits an adapted distribution of the radiation in the beam and cause a more ideal space scanning. With that a more uniform signal strength at the input of the receiver is achieved as a target moves with a constant height within the beam. The ideal pattern is a nearly rectangular shape with rounded corners. In reality, the diagram is superposed with a number of side lobes mainly on the back at higher elevation angles. The shape of all these back-side lobes is combined into a nearly parabolic slope.

In the analyses in this report for the vertical plane the half power antenna beamwidth of the main lobe (θ3) is 10 degrees, the maximum angle of the cosecant squared part is 35 degrees, cosecant floor level is −50 dB and electrical elevation antenna angle 0 degrees. For the horizontal plane the average level for side lobes is −20 dB with cos antenna distribution pattern type. It was also proposed to use sinc antenna distribution pattern type and it was also included in the analysis.

Figure 35: ATS ground receiver vertical antenna pattern

Figure 36: ATS ground receiver horizontal antenna pattern (cos, average envelope)

Figure 37: ATS ground receiver horizontal antenna pattern (sinc, average envelope)

Figure 38: ATS ground receiver 3D antenna pattern (cos)

Figure 39: ATS ground receiver 3D antenna pattern (sinc)

averaged value of iRSS for surrounding network interference calculations

When ATS antenna rotates, the interfering Received Signal Strength (iRSS) depends on the BS arrangement inside the LTE cluster. To take into account rotation of ATS antenna for any surrounding network, the LTE cluster was rotated around its centre and iRSS was calculated every 5 degrees. This is equivalent to the ATS antenna rotation around the cluster keeping the unchanging distance and antenna pointing direction to the cluster centre. Then the averaged value of iRSS was evaluated. The results of the simulations are presented in Figures below for different ATS antenna under consideration.

Figure 40: iRSS dependence on LTE cluster rotation angle and its average value for DE = 10 km

Figure 41: iRSS dependence on LTE cluster rotation angle and its average value for DE = 40 km

Figure 42: iRSS dependence on LTE cluster rotation angle and its average value for DE = 80 km

The approach to calculate the interfering power flux density

The hexagonal LTE cluster, to simplify power flux density evaluation, is substituted by a circular LTE cluster of the same effective area, Seff, and the same other BS characteristics, and the distance from the ATS, DC.

Figure 43: Effective area of circular LTE-C cluster to generate averaged interfering signal at ATS receiver

The area of a hexagonal LTE-C cluster consisting of 19 cells is calculated using equation

(1)

where a is cell radius (Cell range = 2a). Then, the circular cluster effective radius, reff, equals to

(2)

and effective central angle, eff, equals to

(3)

The interfering received signal power element PiRSS entering the ATS receiver is proportional to the BS transmitters number passing the antenna aperture solid angle element and in case of a flat circular LTE cluster (vertical PiRSS component variation is neglected, i.e. is constant) is proportional to the conical beam and circular cluster intersection area (), which is expressed as

(4)

Angular power flux density [mW/deg] at the place of ATS can be evaluated for known antenna pattern (AP) and the calculated iRSS power, PiRSS, using equation

(5)

where G(,θ) is the linear antenna gain factor,  and θ are antenna pattern azimuth and elevation angles, respectively,  is central angle and K are weighted coefficients, which are proportional to the intersection area given by the equation (4).

Figure 44: Conical beam and circular cluster intersection area and chord midpoint position

In case of rotational symmetry antenna (spherical antenna pattern), gain factor depends only on off-axis angle , and in case of discrete G(i), (5) simplifiers to

(6)

where i is defined as

(7)

and ra is radius-vector pointing from the ATS ground receiver to the ATS on-board transmitter, ri is radius-vector pointing from the ATS receiver to the corresponding point located at the same as the BS antenna height, the distance d(i) = DCcos(i), which corresponds to a chord midpoint distance from the ATS Rx in the ri direction, and the angle i, taken from the central line. N/2 is defined as the integer part of eff/2, when taking angular increment of one degree. Weighted coefficients Ki are expressed as the normalized intersection area S(i)/S(0), and are evaluated using equation

(8)

When the two different antenna patterns (horizontal and vertical) are used for no spherical antenna gain G(,θ) calculation, the horizontal and vertical planes are combined to get a pseudo 3-D antenna gain based on the derivations presented in Annex 5 and Annex 7 of Recommendation ITU-R F.1336-4.

Additional factors for consideration in SEAMCAT simulations

MFCN system

The analysis with SEAMCAT was done with the parameters from Table 4, i.e. antenna height of 30 m, downtilt of 3 degrees, which were used simulating one tri-sector 3GPP 2-tiers (without wrap-around) cluster (57 sectors for tri-sector deployment). It is thus of great interest how the results would change then these parameters vary. Therefore, some additional investigations were carried out regarding:

System layout - 5 clusters (99 cells / 285 sectors) or 12 clusters (228 cells / 684 sectors);

Base station parameters - antenna height of 60 m and antenna downtilt of 6 degrees;

Propagation model - in accordance with ITU-R Recommendation P.528 with 10 percent time probability.

The results for ATS ground station and on-board stations are provided in the following sections. [The on-board antenna is directed towards the ATS ground transmitter.]

ATS ground receiver case

Figure 45: iRSS at ATS location for 1 and 12 MFCN clusters (ATS AP acc. to ITU-R M.1459, aircraft altitude 2000 m)

Figure 46: iRSS at ATS location for 1 and 12 MFCN clusters (ATS AP acc. to ITU-R M.1459, aircraft altitude 10000 m)

Figure 47: iRSS at ATS location for 1 MFCN cluster with antenna downtilt of 3 and 6 degrees and antenna height of 30 m and 60 m (ATS AP acc. to ITU-R M.1459, aircraft altitude 2000 m)

Figure 48: iRSS at ATS location for 1 MFCN cluster with antenna downtilt of 3 and 6 degrees and antenna height of 30 m and 60 m (ATS AP acc. to ITU-R M.1459, aircraft altitude 10000 m)

Figure 49: C/(I+N) at ATS location for 1 MFCN cluster with antenna height of 30 m and downtilt of 3 degrees (ATS AP acc. to ITU-R M.1459)

Figure 50: C/(I+N) at ATS location for 1 MFCN cluster with antenna height of 30 m and downtilt of 3 degrees (ATS AP acc. to ITU-R M.1851)

ATS on-board receiver case

Figure 51: iRSS at on-board receiver (ATS radar antenna acc. to ITU-R M.1459); 5 and 12 clusters

Figure 52: iRSS at on-board receiver (ATS radar antenna acc. to ITU-R M.1459); 5 clusters, downtilt 3 and 6 degrees, antenna height 30 m

Figure 53: dRSS at on-board receiver (ATS radar antenna acc. to ITU-R M.1459)

Methodology for field strength calculations based on practical case

The methodologies outlined in items A2.1 and A2.2 are based on the practical experience gained by Ukraine during the calculation exercises conducted in 2014 with the aim of preparing for the conclusion of a bilateral co-ordination agreement between Ukraine and Poland (UKR-POL) in the E-GSM band. To protect the BSs of new MFCN/UMTS networks of Poland from the mobile BWA BSs of Ukraine it was agreed to use a protection criterion of 22 dBμV/m in the 1.25 MHz band at a height of 10 m at the borderline (the interference power at the MFCN/ UMTS receiver input is of −109 dBm/ 5 MHz).

The methodology to determine test points

Requirements for test points:

test points should ensure that the interference is taken into account in all possible directions of it may occur from;

the distance from the borderline is determined as closely as possible, taking into account the availability of the access to the place of measurement on both sides of the border and the availability of a convenient site for the placement of measuring equipment;

desired placement near the operating BS or one that is planned to be installed;

the distance between the test points is determined by the mean value of the distance of the MFCN BSs, which create interference, to the borderline and the width of the directional diagram. With accounting the case when no restrictions apply to the BS, it is acceptable to set a distance of 30 km, and a typical width of the diagram at a level of 0.5 P takes 90 degrees. Then the distance between the test points will be 42 km (see Figure 32) and for the length of the UKR-POL border at 542 km, 13 points can be obtained (15 points were actually identified).

Figure 55: Test point selection

The selection of MFCN BSs to be involved in calculations

The selection of the MFCN/UMTS BSs was carried out in three stages.

First stage

In the first stage, all operating and planned sectors of the Ukraine BWA BSs, located at a maximum distance (about 150 km) from the borderline, were involved in the calculations. The calculations were carried out for several (3-4) nearest reference MFCN/ UMTS stations. Such calculations allowed to take into account the remote BWA BSs located on high altitude positions. The results of the calculations were compared with the protection criterion for the critical case. Those BWA BSs, which field strength level (FSL) was below the protection criterion, were excluded from further consideration.

From the Ukraine experience with the calculation done in the first stage of the calculations exercises, 2506 sectors of the BWA BSs were involved. The calculations were carried out in relation to 4 deployed MFCN/ UMTS BSs, located close to the borderline. As a result, 2071 sectors remained to be taken into account at the following stages of calculation.

Second stage

In the second stage of practical case, the FSL calculations from the remaining sectors of BWA BSs were carried out relative to the test points at the borderline and to the defined MFCN/ UMTS protection criterion 22 dBμV/m/ 1.25 MHz/ 10 m at the borderline (the interference power at the MFCN/ UMTS receiver input is of −109 dBm/ 5 MHz).

Those sectors of BWA BSs, which created FSL lower than the criterion of 5-6 dB excluded from the further calculations. This margin based on the experience of calculations is considered as sufficient to not be taken into account when summing up signals.

Figure 56: Example of transformation of points of calculation from ATS station location to borderline

As a result of the calculations of the 2nd stage of practical case, it was left to account for about 600 sectors. An example of the calculation results for point 13 is presented in Table 10. Only 7 sectors individually provided an excess of the threshold (–109 dBm at UMTS receiver input), but in order to sum up the individual interference, so many other sectors should have been involved that, as a result of addition, it will give a total level that was lower than protection criterion. For these selected stations the individual measures to reduce the level of emissions are applied including switching-off the interfering sectors.

Table 10: An example of calculation results of total FSL from individual sectors of BWA BSs at test point no. 13

|  | Name of point | Rx frequency, MHz | KTBF, dBm | Name of interfering BS | Tx frequency, MHz | Level of interference at the input of receiver, dBm | TD dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Point 13 (106) | 882.3 | −109 | Sambir 108484 (472) | 882.3 | −86.17 | 22.85 |
| 2 | Point 13 (106) | 882.3 | −109 | Yavorivsk (426) | 882.3 | −93.57 | 15.55 |
| 3 | Point 13 (106) | 882.3 | −109 | Н14008−46 (359) | 882.3 | −99.92 | 9.59 |
| 4 | Point 13 (106) | 882.3 | −109 | Sambir 108484 (470) | 882.3 | −101.86 | 7.91 |
| 5 | Point 13 (106) | 882.3 | −109 | UMTS3074−46 (392) | 882.3 | −102.98 | 6.99 |
| 6 | Point 13 (106) | 882.3 | −109 | Yavorivsk (427) | 882.3 | −106.25 | 4.6 |
| 7 | Point 13 (106) | 882.3 | −109 | Yavorivsk (425) | 882.3 | −107.81 | 3.64 |
| 8 | Point 13 (106) | 882.3 | −109 | 461762 (406) | 882.3 | −109.05 | 2.99 |
| 9 | Point 13 (106) | 882.3 | −109 | 461762 (407) | 882.3 | −109.55 | 2.74 |
| 10 | Point 13 (106) | 882.3 | −109 | Lviv 59866 (373) | 882.3 | −112.33 | 1.66 |
| 11 | Point 13 (106) | 882.3 | −109 | Novyi Rozdil 109340 (346) | 882.3 | −114.32 | 1.12 |
| 12 | Point 13 (106) | 882.3 | −109 | Sambir 108484 (471) | 882.3 | −115.33 | 0.91 |
| 13 | Point 13 (106) | 882.3 | −109 | Н14004−46 (350) | 882.3 | −116.11 | 0.77 |
| 14 | Point 13 (106) | 882.3 | −109 | Sokilnyky 109414 (442) | 882.3 | −116.22 | 0.75 |
| 15 | Point 13 (106) | 882.3 | −109 | LV0013−46 (389) | 882.3 | −117.19 | 0.61 |
| 16 | Point 13 (106) | 882.3 | −109 | Lviv 109325 (205) | 882.3 | −118.35 | 0.48 |
| 17 | Point 13 (106) | 882.3 | −109 | Pustomyty 106123 (481) | 882.3 | −119.54 | 0.37 |
| 18 | Point 13 (106) | 882.3 | −109 | Lviv 90799 (395) | 882.3 | −120.19 | 0.32 |
| 19 | Point 13 (106) | 882.3 | −109 | 461808 (525) | 882.3 | −120.37 | 0.31 |
| 20 | Point 13 (106) | 882.3 | −109 | Lviv 59866 (374) | 882.3 | −120.74 | 0.28 |
| 21 | Point 13 (106) | 882.3 | −109 | Lviv 25968 (416) | 882.3 | −120.94 | 0.27 |

Third stage

In the third stage of practical case a list of those BWA BSs sectors, which relate to individual test points, was created. For them, the summation of individual levels (in power) was carried out. The summation of individual powers is carried out in an arithmetic form of numbers with further transfer of the summation result into a logarithmic form. Those sectors, which did not lead to exceeding the threshold when involved in summing the results of individual calculations, were removed from the analysis. Table 11 shows the final results of the calculations for the 15-th test points based on Ukraine practical experience.

Table 11: final results of calculations for the 15-th test points, based on Ukraine experience

Adjustment of MFCN BS parameters based on practical case

As an example, in Table 12, data are presented for taking into account the change in the gain of the real typical and simulated (according to HCM method) antenna, depending on the angle of elevation.

Table 12: Example of antenna gain dependence on antenna tilt

| Tilt angle of antenna pattern, degree | Losses La, dB |  | Accounted antenna gain Ga*, dB |
| --- | --- | --- | --- |
|  | Antenna pattern of Kathrein 739 650 in vertical plane | Calculated antenna pattern of 004LA type in vertical plane (Appendix 1 to Annex 6 of HCM) |  |
| 0 | 0 | 0.0 | 12 |
| 1 | 0.2 | 0.0 | 12 |
| 2 | 0.9 | 0.2 | 11 |
| 3 | 1.9 | 1.0 | 10 |
| 4 | 3.5 | 3.0 | 9 |
| 5 | 5.7 | 8.1 | 5 |

* Ga = 15 dB – L – La – antenna gain, where L = 3 dB – an average feeder loss; La = an average attenuation due to antenna tilt (γ = 0°, 4°, 5°).

In addition, Table 13 presents numerical indicators of sector shutdowns efficiency and adjustment of parameters (example of practical case) during the optimization of the existing BWA network of Ukraine in order to minimize the impact on the UMTS network of Poland. In the beginning 2506 sectors were involved in the calculations. Actually, the change affected the 464 sectors belonging to 111 BWA BSs located in the 80 km wide border area. The number of switched off sectors was 9% of the total number of sectors affected by practical changes. The number of sectors requiring technical adjustments is 28% of the total number of sectors affected by practical changes. The rest of the adjustments (63%) came from the frequency parameters of the BSs of the BWA network.

Table 13: Adjustment of Ukraine BWA network

|  | Number of BWA BSs in 80 km border area (number of sectors) | Number of sectors |  | Number of channels |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Requiring correction of technical characteristics | Requiring switching off | Requiring correction of technical characteristics | Requiring switching off |
| Total | 111 (464) | 132 | 43 | 619 | 125 |

Based on these results, the most effective means of changing the parameters of the operating network in the practical case of re-setting the BWA network of Ukraine was the re-setting of frequency plan of the network.

The example of draft template for coordination Agreement

Draft

TECHNICAL AND PROCEDURAL ARRANGEMENT for usage of the 1427-1518 MHz band by terrestrial systems in the border areas between Telecommunication Administration XX and YY

Date

Introduction

- Principles

Technical conditions for coordination of the MFCN stations with the ATS stations

Technical conditions for coordination of ATS stations with MFCN stations

General provisions

Harmful interference

Procedure of coordination

Revision and cancellation

Coming into force

On behalf of the Telecommunication Administration of XX

On behalf of the Telecommunication Administration of YY

Appendix

Field strength measurement procedure

- Common provisions

Requirements to the monitoring equipment

Information to be provided by the affected Administration in case of harmful interferences

Examples of test points

List of Reference

ITU-R Recommendation M.1851-0 "Mathematical models for radiodetermination radar systems antenna patterns for use in interference analyses"

ITU-R Recommendation M.1459-0 "Protection criteria for telemetry systems in the aeronautical mobile service and mitigation techniques to facilitate sharing with geostationary broadcasting-satellite and mobile-satellite services in the frequency bands 1 452-1 525 MHz and 2 310-2 360 MHz"

ECC Report 227 "Compatibility Studies for Mobile/Fixed Communication Network (MFCN) Supplemental Downlink (SDL) operating in the 1452-1492 MHz band"

ITU-R Report M.2324-0 “Sharing studies between potential International Mobile Telecommunication systems and aeronautical mobile telemetry systems in the frequency band 1 429-1 535 MHz”

Final acts of the CEPT multi-lateral meeting for the frequency band 1452-1479.5 MHz, Constanţa, 2007 (MA02revCO07)

ECC Decision (17)06 “The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)“

ECC Decision (13)03 “The harmonised use of the frequency band 1452-1492 MHz for MFCN SDL”

ITU-R Recommendation P.526-14 "Propagation by diffraction"

ITU-R Recommendation P.528-3 "Propagation curves for aeronautical mobile and radionavigation services using the VHF, UHF and SHF bands"

ITU-R Recommendation P.1546-5 "Method for point-to-area predictions for terrestrial services in the frequency range 30 MHz to 3 000 MHz"

ITU-R Recommendation P.525-2 "Calculation of free-space attenuation"